# Appendix B. Specifications

| Nr. | Aspect | Requirement RWS | Your suggestion |
| --- | --- | --- | --- |
| 1 | Area of coverage. | See fig. 5 |  |
| 2 | Number of required radar locations. Indicate the locations that you would prefer, see figure 6. | Max. 3 |  |
| 3 | Occupied space per radar station | <150 m long <10 m wide <5 m high |  |
| 4 | Spatial resolution of the radar measurements (averaged over the entire area) | <250 m in range <7,5 degrees azimuth |  |
| 4 | Presentation as two-dimensional array of current vectors | Grid with 250 x 250m cells |  |
| 5 | Accuracy of current vectors (averaged over het entire area) | <10 cm/s |  |
| 6 | Measuring time window. | < 5 minutes |  |
| 7 | Update frequency | > 1 x per minute |  |
| 8 | Transmit frequency | 44-47 MHz |  |
| 9 | Band width | <600 kHz |  |
| 10 | Transmit power (ERP) | <20 W |  |
| 11 | Availability | Failure rate < 3 x per year, failure time < 12 hours |  |
| 12 | Technical lifetime | >10 years |  |
| 13 | Guarantee | >1 year |  |