# Call Setup Time and Call Setup Success Rate Class I: Railway Emergency MOC with dispatcher

Purpose:	This test will measure the call setup time and call setup success rate of a Mobile Originating Railway Emergency Call (MOC) – this is a VGCS Call (TS91)

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via Netact Counters (only CSSR) and/or QATS Railway and/or QATS UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test together with a fixed station that is authorized to make Group Calls.

Optionally a Mobile Station can be used to check actual cell broadcast channel activation.  This will, however, have no impact on call setup time and can be left out.

For the sake of efficiency, this test will also evaluate the Call Setup Success Rate by using the same calls to calculate the call setup success rate.

A minimum of 200 calls per BSC will be done.

All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Call Setup Time and Call Setup Success Rate will be calculated during post-processing.

- For continuous network monitoring purposes three sources will be used, namely:

  - Netact counters via direct integration into NPM (only CSSR).

  - QATS Railway data of all traffic, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Railway data, specific for UAP, reported via TSV file principle to the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

Both systems, as specified under 1 and 2, monitor all traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to point 3, specific UAP, the data structure of QATS is available

in document [EXPDM1322.CC.COM.Drive test TSV table description_v1.5].

Remark: The Call Setup Time derived from UAP (point 3) includes the Um (air) interface and is thus different (usually longer) than the Call Setup Time derived from QATS Railway data of all traffic (point 2).

The Call Setup Time and Call Setup Success Rate will be calculated and reported within NPM.

Additional information:

- For validation purposes:

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM;

- Reports based on all operational traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational traffic per BSC

- Reports based purely on UAP.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System

- Mobile IM3 filter activated

- For monitoring purposes:

- Netact

- QATS Railway for reporting on all operational traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

Call Setup Time: 95% of all REC call setups within the reporting period must be established in less than 2 seconds

Call Setup Success Rate: 99% of all REC call setups within the reporting period must be established within 3 seconds

Note: no statistical relevance is obtained if less than 200 calls (Validation only).

Call Setup Time and/or Call Setup Success Rate:

- For validation purpose:

Call Setup Time: The time difference between the Layer 3 messages “CONNECT” and “Channel Request”, measured at the Igsmr Interface.

Call Setup Success: A call is considered successfully set up if the call setup time does not exceed 3 seconds.

- For monitoring purpose:

Call Setup Time: The time difference between “Connect” and “CReq CL3 info”, measured at A interface. (reported value duration in the Xdr_duration_Railway_table) of Mobile Originating Railway Emergency Call (MOC) calls.

Call Setup Success: A call is considered successfully set up if the call setup time does not exceed 3 seconds.

Call setup time

- Monitoring purpose (UAP):

Specific UAP the description and data structure for this KPI is not yet available (roadmap Q4 2018).

Will be added to document [EXPDM1322.CC.COM.Drive test TSV table description_v1.5 ] when design is ready.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Call Setup Time REC (Romes) | Call Setup Time REC (QATS) |
| Target | 95% < 2 seconds | 95% < 2 seconds |
| Post-processing | Required | Required |
| Minimum sample size | 200 calls per BSC | N/A |
| KPI number | N/A | KPI1 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | BSC |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_ETSI_8W_REC_199 | N/A |
| Remark | N/A | N/A |

| Name | Call Setup Success Rate REC (Romes) | Call Setup Success Rate REC (Netact/QATS) |
| --- | --- | --- |
| Target | 99% ≤ 3 seconds | 99% < 3 seconds 99% Success rate (sub-system counters) |
| Post-processing | Required | Required |
| Minimum sample size | 200 calls per BSC | N/A |
| KPI number | N/A | KPI6 |  |
| Reporting period | N/A | Monthly |  |  |
| Reporting level | N/A | BSC |  |  |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_ETSI_8W_REC_199 | N/A |
| Remark | N/A | N/A |

Deliverables:	Validation: RSCMD file containing all relevant data and post-processed data.

Monitoring: NPM reports

Note: the Romes measurement data and QATS data from the 15 min Reporting server, both post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |