# C/I Downlink

Purpose:	This test will measure the carrier to interference ratio in the downlink.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via R&S Autonomous Scanner system.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

Only the scanner part of the ROMES Drive-Test System will be used.  No calls will be made.

The entire GSM-R band will be scanned by the scanner; it will report RxLev & C/I downlink for each GSM-R channel, meaning that each measurement sample will contain up to 19 C/I values and their corresponding RxLev (“up to” is quoted because in practice the scanner can never decode the C/I for all 19 channels at a single point in time, mostly due to interference or too low received levels).

The ROMES software stores timestamps & GPS coordinates as well as relevant data in log files.

The C/I corresponding to the channel having the highest RxLev on each measurement point will be taken into account as being the actual C/I of that location.  Only C/I values of BCCH channels are taken into account.

The Average C/I over 100m will be calculated during post-processing in the Automatic Post Processing System (APPS), this will be an arithmetic average.

In other words, the average C/I is calculated by taking for each measurement sample the C/I of the channel having the highest RxLev at that point.  The channel can thus change at each measurement sample.  That means that the averaging is not channel-aware.

For this reason, the most frequent cell ID will also be reported over each 100m section.  The most frequent cell ID will be calculated after the Average C/I has been calculated; the Average C/I will be an input to calculate the most frequent cell ID of the section: for each measurement sample taken into account, the cell ID having the highest RxLev (which corresponds to the C/I taken into account for that measurement sample) will be taken into account to calculate the most frequent cell ID of the section.

The C/I is captured by the TSMW scanner in time triggered mode.  The number of samples per 100 meters will depend on the train speed and on the (possible) other measurements done by the TSMW at the same time.

The TSMW is capable of scanning 350 channels per second (this is the maximum value with optimum settings, in practice, it will be about 10-15% lower).

This means about 10 samples per second with the configuration used for the NITs.  That is 12 samples over 100 m at 300 km/h.

The ROMES software stores timestamps, GPS coordinates and the C/I related data in RSCMD log files.

The post-processed data per 100-meter section will be uploaded into NPM.

- For continuous network monitoring purposes, the R&S Autonomous Scanner system is used.

The same process as described above in section a is applicable, except the TSMW being the TSME. The TSME is the successor of the TSMW and is the incorporated scanner within the R&S Autonomous Scanner system.

The post-processed data per 100-meter section will be uploaded into NPM.

Additional information:

- For validation purposes:

This test will be executed once in both driving directions on a given track.

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

All carrier to interference ratio in the downlink measurements, based on above described method, are taken into account.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System

- Kaelus filter

- For monitoring purposes:

- R&S Autonomous Scanner system

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

- For monitoring purposes:

The BTS, BSC, MSC network elements are operational and tested.

KPI Definition:

C/I Downlink: the C/I shall be greater than or equal to 17 dB on average over 100 meters.

C/I Downlink:

- For validation purpose:

The carrier-to-interference ratio (C/I) is the quotient between the average per 100-meter section received modulated carrier power C and the average per 100-meter section received co-channel interference power I, from other transmitters than the useful signal.

- For monitoring purpose:

The carrier-to-interference ratio (C/I) is the quotient between the average per 100-meter section received modulated carrier power C and the average per 100-meter section received co-channel interference power I, from other transmitters than the useful signal.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | C/I Downlink (Romes) | C/I Downlink (Autonomous Scanner) |
| Target | C/I ≥ 17 dB on average over 100 meters | N/A |
| Post-processing | APPS | APPS |
| Minimum sample size | 100 meters (1 section) | N/A |
| KPI number | N/A | KPI38 |
| Reporting period | N/A | Depending on Autonomous Scanner |
| Reporting level | N/A | Trace |
| IM3 filter in MS | N/A | N/A |
| mpJules antenna constraint | The antennas next to the antenna used for this test can only be used for passive tests (no uplink transmission). | At least an isolation of 40 dB should be granted with other transmitting antennas |
| Band-Pass filter on antenna | Yes | N/A |
| Template | Any (the settings are included in all available templates) | N/A |
| Remark | N/A | N/A |

Deliverables:	Romes and Autonomous Scanner: RSCMD file containing all relevant data

Note: the Romes and Autonomous Scanner post-processed 100-meter section data will be uploaded to NPM via the Multi-Vendor Interface Measurement Train.

The following observations / remarks have been made during the Acceptance Test Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |