# Coverage

Purpose:	This test will verify the signal levels (coverage) on all tracks evaluated in 100m sections.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via R&S Autonomous Scanner system.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

An odometer (wheel trigger or ground speed sensor) will provide the ROMES Drive-Test System with a pulse every 10 cm, instructing the ROMES to collect data at that point in time, using the TSML-CW scanner.  This way, the measurement rate is independent of the train speed.

An isotropic antenna with a gain of 0 dBi is assumed, at 4 meters high (train roof).  Compensations for possible antenna gain and feeder cable losses will be input in the ROMES software (this will not compensate for a non-omnidirectional pattern of the roof’s antenna).

The ROMES software stores timestamps, GPS coordinates and the Coverage related data in RSCMD log files.

Post-processing takes place in the Automatic Post Processing System (APPS) by using the AEP95 calculation method which is agreed upon with ProRail.

The post-processed data per 100-meter section will be uploaded into NPM

- For continuous network monitoring purposes, the R&S Autonomous Scanner system is used.

The R&S Autonomous Scanner system will be configured to collect coverage measurements, based on time trigger. The Time Trigger will be configured to allow at least 1 sample every 10 cm, based on maximum allowed train speed of the train on which the R&S Autonomous Scanner system is installed. During most measurements, more than 1 sample per 10 cm is collected.

An isotropic antenna with a gain of 0 dBi is assumed, at 4 meters high (train roof).  Compensations for possible antenna gain and feeder cable losses will be configured in the R&S Autonomous Scanner system software (this will not compensate for a non-omnidirectional pattern of the roof’s antenna).

The R&S Autonomous Scanner system software stores timestamps, GPS coordinates and the Coverage related data in RSCMD log files.

Post-processing takes place in the Automatic Postprocessing System (APS) by using the AEP95 calculation method which is agreed upon with ProRail.

The post-processed data per 100-meter section will be uploaded into NPM.

Additional information:

- For validation purposes:

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

Schematic of the antennas positions on the mpJules train:

In case this test is carried out with the mpJules train, the following antennas are recommended:

- 3 or 4 in case CAB2 is driving

- 5 or 6 in case CAB1 is driving

Assuming the rear pantograph is used.

The distance triggered CW raw data is sampled (detector set on RMS) until enough data for covering a section (100 m) is available.

- For monitoring purposes:

All coverage related measurements following the AEP95 method are taken into account.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System

- For monitoring purposes:

- R&S Autonomous Scanner system

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

- For monitoring purposes:

The BTS, BSC, MSC network elements are operational and tested.

KPI Definition:

- For validation purpose:

According to the AEP95 method, the calculated AEP95 is rounded to 1 decimal before determining if the 100m section complies with the coverage requirement (>= -85 dBm)

- For monitoring purpose:

According to the AEP95 method, the calculated AEP95 is rounded to 1 decimal before determining if the 100m section complies with the coverage requirement (>= -85 dBm)

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Gebiedsindeling (Romes) | Gebiedsindeling (Autonomous Scanner) |
| Target | -85 dBm | -85 dBm |
| Post-processing | APPS | APPS |
| Minimum sample size | N/A | N/A |
| KPI number | N/A | KPI13 |
| Reporting period | N/A | Depending on autonomous scanner usage |
| Reporting level | N/A | Trace |
| IM3 filter in MS | N/A | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_8W_Answer_NQA | N/A (train specific) |
| Remark | N/A | N/A |

Deliverables:	Romes and Autonomous Scanner: RSCMD file containing all relevant data

Note: the Romes and Autonomous Scanner post-processed 100-meter section data will be uploaded to NPM via the Multi-Vendor Interface Measurement Train.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |