# Dropped Call Rate for Circuit Switched Voice Calls

Purpose:	This test will measure the Dropped Call Rate for Mobile Originating Normal Operations Voice Calls (MOC).

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via NetAct Counters

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

During the test short voice calls of normal priority (Priority 4) to the responder will be made.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Dropped Call Rate will be calculated during post-processing.

- For continuous network monitoring purposes three sources will be used, namely:

  - NetAct counters via direct integration into NPM.

The Dropped Call Rate will be calculated and reported within NPM.

Additional information:

- For validation purposes:

- Call window: 30 seconds
- Call duration: 10 seconds 
- Minimum pause: 10 seconds

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM;

- Reports based on all operational traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational traffic per Cell

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- Netact

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both Validation and Monitoring:

Dropped Call Rate for Circuit Switched Voice Calls: [Number of connections released unintentionally / Total number of successfully established connections] within the reporting period.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Dropped Call Rate of Circuit Switched Calls (Romes) | Dropped Call Rate of Circuit Switched Calls (NetAct) |
| Target | ≤ 2% | ≤ 2% |
| Post-processing | Required | Required |
| Minimum sample size | T.b.d. | N/A |
| KPI number | N/A | KPI157 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | Cell |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_MR_8W_Voice_CSSR-CST_02.027-R | N/A |
| Remark | N/A | Both 2W and 8W calls are taken into account (cumulated) |

Deliverables:	Validation: RSCMD file containing all relevant data and post-processed data.

Monitoring: NPM reports

Note: the Romes measurement data and QATS data from the 15 min Reporting server, both post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |