# Interruption Time at Handover

Purpose:	This test will measure the Interruption Time at Handover

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via QATS Railway and/or QATS UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

For the whole duration of the test, the mobile station will establish a dedicated voice call to 	the responder.

The postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

Interruption Time at Handover will be calculated during post-processing.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all live traffic reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS UAP Data reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

The QATS Railway systems, as specified under a and b, monitor all traffic, of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

The Interruption Time at Handover Rate will be calculated and reported within NPM.

- for all live users together

- specific for the UAP’s

Additional information:

- For validation purposes:

- Call window: long (endless) voice call

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

All HO’s are taken into account, without split over different user groups.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

HO Time: 95% of all Interruption Time at Handover measurements within the reporting period must be established in less than 500 milliseconds

Note: no statistical relevance is obtained if less than 600 calls.

Interruption Time at Handover:

- For validation purpose:

HO Time: The time difference between the Layer 3 messages “HANDOVER COMMAND” and “HANDOVER COMPLETE” or “HANDOVER FAILURE”, whichever comes first, measured at the Igsmr Interface.

- For monitoring purpose:

HO Time: The time difference between “HOCommand” and “HOComplete”, measured at A interface (reported value duration in the Xdr_duration_Railway_table) of relevant voice calls.

HO Time

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Interruption Time at Handover (Romes) | Interruption Time at Handover (QATS) |
| Target | 95% < 500 milliseconds | 95% < 500 milliseconds |
| Post-processing | Required | Required |
| Minimum sample size | 600 handovers per BSC | N/A |
| KPI number | N/A | KPI104 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | BTS |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_8W_Long_Voice | N/A |
| Remark | N/A | N/A |

Deliverables:	Validation: RSCMD file containing all relevant data and post-processed data.

Monitoring: NPM reports

Note: the Romes measurement data and QATS data from the 15 min Reporting server will be

uploaded to NPM via the Multi-Vendor Interfaces.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |