# Received Level in Idle mode

Purpose:	This test will measure the received level in Idle Mode.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via QATS Railway and/or QATS UAP

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

During the test no calls will be made. The test will evaluate the downlink received level in Idle mode.

The postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Received Level in Idle mode will be calculated during post-processing, where the values per 100m section are reported.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all traffic, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Railway data, specific for UAP, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

Both systems, as specified under 1 and 2 monitor all traffic, of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to point 2, specific UAP, the data structure of QATS is not yet available (roadmap Q4 2018).

NPM reports, per 100m section, the Received Level in Idle mode measurements in the 100m section.

Additional information:

- For validation purposes:

Idle mode means that the MS is camping on cells, performing cell reselections only.

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Two reports are available;

- One based on all operational traffic.

- One based purely on UAP.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both Validation and Monitoring:

Received Level in Idle mode values are reported per 100m section, where the following information is reported for each 100m section:

- Driving direction (up- or downdriving); reporting is done per driving direction, so there will be two reports per 100m section

- Most frequent Cell-ID

- Received level in Idle Mode Minimum (minimum of all minimums)

- Received level in Idle Mode Average (average of all minimums)

The output of the reported received level in Idle Mode is used by ProRail for defining the Gebiedsindeling (GI).

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Received Level in Idle mode (Romes) | Received Level in Idle mode (QATS) |
| Target | N/A | N/A |
| Post-processing | Required | Required |
| Minimum sample size | N/A | N/A |
| KPI number | N/A | KPI102 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | Landelijk |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_8W_Answer_NQA | N/A |
| Remark | N/A | N/A |

Deliverables:	Validation: RSCMD file containing all relevant data and post-processed data per 100m section.

Monitoring: NPM reports

Note: the Romes measurement data and QATS data from the 15 min Reporting server will be

uploaded to NPM via the Multi-Vendor Interfaces.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |