# Amount of Blocked PDCH requests and PDCH request failure rate

Purpose:	This test will measure the Amount of Blocked PDCH requests and calculate the PDCH request failure rate.

A distinction is made between:

- Dedicated measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via NetAct Counters

Test configuration:

Description:

- For dedicated measurements purposes, the ROMES Drive-Test System is used for this test.

The test will measure the Amount of Blocked PDCH requests and calculate the PDCH request failure rate.

For the sake of efficiency, this test will evaluate the PDCH request failure rate by using the same data as those used to evaluate the Amount of Blocked PDCH requests.

All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Amount of Blocked PDCH requests and PDCH request failure rate will be calculated during post-processing.

- For continuous network monitoring purposes one source will be used, namely:

  - NetAct counters via direct integration into NPM.

The Amount of Blocked PDCH requests and Blocked PDCH request failure rate will be calculated and reported within NPM.

Additional information:

- For dedicated measurements purposes:

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM;

- Reports based on all operational traffic

- Reports based on all operational traffic per Cell

Required hardware and Software:

- For dedicated measurements purposes:

- ROMES Drive-Test System

- Mobile IM3 filter activated

- For monitoring purposes:

- NetAct

Prerequisites:

- For dedicated measurements purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both dedicated measurements and Monitoring:

Amount of Blocked PDCH requests: Number of PDCH requests that could not be served, observed in the reporting period.

PDCH request failure rate: [[Amount of Blocked PDCH requests] / [Total Amount of PDCH requests] *100] as observed in the reporting period

Summary:

|  | Dedicated measurements (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Amount of Blocked PDCH requests (Romes) | Amount of Blocked PDCH requests (Netact) |
| Target | T.b.d. | N/A |
| Post-processing | Required | Required |
| Minimum sample size | T.b.d. | N/A |
| KPI number | N/A | KPI65 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | Cell |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | T.b.d. NIT-PDCH | N/A |
| Remark | N/A | N/A |

| Name | PDCH request failure rate (Romes) | Blocked PDCH request failure rate (Netact) |
| --- | --- | --- |
| Target | T.b.d. | N/A |
| Post-processing | Required | Required |
| Minimum sample size | T.b.d. | N/A |
| KPI number | N/A | KPI67 |  |
| Reporting period | N/A | Monthly |  |  |
| Reporting level | N/A | Cell |  |  |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | NIT-PDCH | N/A |
| Remark | N/A | N/A |

Deliverables:

- Dedicated measurements: RSCMD file containing all relevant data and post-processed data.

- Monitoring: NPM reports

Note: the Romes measurement data, post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Dedicated measurements only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |