# Received Quality in Dedicated mode

Purpose:	This test will measure the Received Quality in Dedicated Mode.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via Netact Counters and/or QATS Railway and/or QATS UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

For the whole duration of the test a dedicated voice call to the landside station (acting as a responder) will be made. The test will evaluate the downlink received quality.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Received Quality in Dedicated mode will be calculated during post-processing: an average of the reported values per Cell-ID will be done, as well the 95th and 99th percentile per Cell-ID.

Both the averaged Cell-ID and the measured raw data (1 value each 480 ms) will be reported.

The post-processed averaged data per Cell-ID, including the 95th percentile values and the 99th percentile values, will be uploaded into NPM via the Multi-Vendor Interface Measurement Train.

- For continuous network monitoring purposes three sources will be used, namely:

  - Netact counters via direct integration into NPM

  - QATS Railway data of all traffic, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Railway data, specific for UAP, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

Both systems, as specified under a and b monitor all traffic, of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to point c, specific UAP, the data structure of QATS is not yet available (roadmap Q4 2018).

NPM reports, per Cell-ID, the Average Quality, the 95th percentile value and the 99th percentile value of all calls in the cell (dedicated mode).

Additional information:

- For validation purposes:

Dedicated mode means that the MS is in a call.

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Two reports are available;

- One based on all operational traffic.

- One based purely on UAP.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- Netact

- QATS Railway for reporting on all operational traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both Validation and Monitoring:

- Averaged RXQual per Cell-ID over the reporting period.

- 95th percentile value per Cell-ID over the reporting period.

- 99th percentile value per Cell-ID over the reporting period.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Received Quality in Dedicated mode (Romes) | Received Quality in Dedicated mode (Netact/QATS) |
| Target | N/A | N/A |
| Post-processing | Required | Required |
| Minimum sample size | All values per cell | All values per cell |
| KPI number | N/A | KPI123 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | Trace |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | The antennas next to the antenna used for this test can only be used for passive tests (no uplink transmission). | N/A |
| Band-Pass filter on antenna | Yes | N/A |
| Template | ProRail_8W_ Long_Voice | N/A |
| Remark | N/A | N/A |

Deliverables:	Validation: RSCMD file containing all relevant data and post-processed data per Cell-ID.

Monitoring: NPM reports

Note: the Romes measurement data and QATS data from the 15 min Reporting server will be

uploaded to NPM via the Multi-Vendor Interfaces.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |