# SUBSET-093: Connection Establishment Delay & Connection Establishment Error Ratio

Purpose:	This test will measure the Connection Establishment Delay (CS Data Call Setup Time) and the Connection Establishment Error Ratio

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system.

- Monitoring measurements obtained via QATS Railway and/or QATS UAP.

Test configuration :

Description:

- For validation purposes, short CS Data Calls to the responder will be made using a normal priority.

A waiting period of 30 seconds shall be observed between the end of a call and the next call attempt.

For the sake of efficiency, this test will also evaluate the Connection Establishment Error Ratio by using the same calls to calculate the Connection Establishment Delay. Only successful connections will be evaluated.

All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Call Establishment delay and the Call Establishment Error Ratio will be calculated during post-processing.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all traffic, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Railway data, specific for UAP, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

The QATS Railway system, as specified under a, monitors all traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to point b, specific UAP, only calls generated by UAP are monitored.

The Call Establishment delay and the Call Establishment Error Ratio will be calculated and reported within NPM.

Additional information:

- For validation purposes:

- Call window: 30 seconds
- Call duration: 0 seconds 
- Minimum pause: 20 seconds

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM;

- Reports based on all operational traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational traffic per BSC

- Reports based purely on UAP.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

Connection Establishment Delay: 95% of all CS Data call setups within the reporting period must be established in less than 8.5 seconds and all CED are lower or equal to 10 seconds.

Connection Establishment Error Ratio: The amount of Connection Establishment Errors within the reporting period must be less or equal to 16 (out of 1078 samples)

Note: no statistical relevance is obtained if less than 100 calls for CED and 1078 calls for CEER.

Connection Establishment Delay and/or Connection Establishment Error Ratio:

- For validation purpose:

Connection Establishment Delay: The time difference between the delivery of the command ATD xxx to the mobile and the later of the two following events, measured at the Igsmr Interface:

- Indication of successful connection establishment by CONNECT yyy

- Transition of circuit 109 (DCD) to ON

Connection establishment delay [ms] = ( t CS Connection Completed [ms] | t Circuit 109 ON [ms]) – t CS Connection Request [ms]

Connection Establishment Error Ratio: This is the ratio of the number of unsuccessful attempts for connection establishment to the total number of connection establishment attempts.

Unsuccessful cases are:

- All responses except CONNECT yyy, if the connection establishment error is not caused by the service user.

- If successful connection establishment is not signalized within the period specified as maximum delay.

- If neither a connection establishment indication nor an error indication is received up to the maximum time of connection establishment, the attempt is counted as unsuccessful.

Successful connection:

A connection is considered as successful when it is established, and actual data is being sent on both uplink and downlink.

- For monitoring purpose (QATS Railway):

Connection Establishment Delay: The time difference between “Alerting” and “CM Service Request”, measured at A interface (reported value duration in the Xdr_duration_Railway_table) of CS Data calls.

Connection Establishment Delay

Connection Establishment Error Ratio: This is the ratio of the number of unsuccessful attempts for connection establishment to the total number of connection establishment attempts.

Unsuccessful cases are:

- All responses except Alerting, if the connection establishment error is not caused by the service user.

- If successful connection establishment is not signalized within the period specified as maximum delay.

- If neither a connection establishment indication nor an error indication is received up to the maximum time of connection establishment, the attempt is counted as unsuccessful.

Successful connection:

A connection is considered as successful when it is established, and actual data is being sent on both uplink and downlink.

- Monitoring purpose (UAP):

T.b.d.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Connection Establishment Delay (Romes) | Connection Establishment Delay (QATS) |
| Target | 95% < 8.5 s 100% ≤ 10 s | 95% < 8.5 s 100% ≤ 10 s |
| Post-processing | Required | Required |
| Minimum sample size | 100 calls per BSC | N/A |
| KPI number | N/A | KPI125 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | N/A |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_O2475_Data_CSSR-CST_02.024-R | N/A |
| Remark | N/A | N/A |

| Name | Connection Establishment Error Ratio (Romes) | Connection Establishment Error Ratio (QATS) |
| --- | --- | --- |
| Target | Connection Establishment Errors ≤ 16 | Connection Establishment Errors ≤ 16 |
| Post-processing | Required | Required |
| Minimum sample size | 1078 calls per BSC | N/A |
| KPI number | N/A | KPI126 |  |
| Reporting period | N/A | Monthly |  |  |
| Reporting level | N/A | N/A |  |  |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_O2475_Data_CSSR-CST_02.024-R | N/A |
| Remark | N/A | N/A |

Deliverables:	Validation: RSCMD file containing all relevant data and post-processed data.

Monitoring: NPM reports

Note: the Romes measurement data and QATS data from the 15 min Reporting server, both post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |