# SUBSET 093: Circuit-Switched Data Transfer Delay

Purpose:	This test will measure the Circuit-Switched Data Transfer Delay of MO calls.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via QATS Railway and/or QATS UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

During the testing period automatic repeating frames are sent by the ROMES system.  The frames are echoed back by the landside station.

Delay times, sent and received frames are stored in the ROMES mobile station log files.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Circuit-Switched Data Transfer Delay periods will be calculated during post-processing.

During this test 100 samples per BSC area are measured.  The 100 samples in each BSC area 	are taken from at least 3 different cells.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all ETCS-2 CS traffic, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Railway data, specific for UAP, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

The QATS Railway system, as specified under a, monitors all ETCS-2 CS traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to point b, specific UAP, only calls generated by UAP are monitored.

The Circuit-Switched Data Transfer Delay periods will be calculated and reported within NPM.

Additional information:

- For validation purposes:

- The frame size is 30 bytes

- The CSD connection speed is 9600 bps

- The protocol is V110 (transparent mode)

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

As per O2475v3 (section 2.2.1):

Measuring for a higher data transfer rate also qualifies the test area for the lower rates (e.g. measuring at 9.6 kbps also qualifies 4.8 and 2.4)

This explains why the test is only executed at 9600 bps.

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM:

- Reports based on all operational ETCS-2 CS traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational ETCS-2 CS traffic per BSC (if present on BSC)

- Reports based purely on UAP

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational ETCS-2 CS traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

Transfer Delay: The test can be considered as successful if it can be stated with a probability for right acceptance of 95% that at least 99% of all data transmission delay measurements meet to the requirement of a one-way transfer delay of less than or equal to 500 milliseconds.

Note: no statistical relevance is obtained if less than 100 samples.

Circuit-Switched Data Transfer Delay:

- For validation purpose:

Transfer Delay: The Transfer Delay is defined as the elapsed time between the request for transfer of a user data unit and the indication of successfully transferred end-to-end user data unit.

- For monitoring purpose:

Transfer Delay: The Transfer Delay is defined as the elapsed time between the request for transfer of a user data unit and the indication of successfully transferred end-to-end user data unit.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Data Transmission Delay (Romes) | Data Transmission Delay (QATS) |
| Target | 99% ≤ 500 ms | 99% ≤ 500 ms |
| Post-processing | Required | Required |
| Minimum sample size | 100 samples | N/A |
| KPI number | N/A | KPI127 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | N/A |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ETCS_Transfer_Delay | N/A |
| Remark |  | N/A |

Deliverables:	Validation: RSCMD file containing all relevant data and post-processed data.

Monitoring: NPM reports

Note: the Romes measurement data and QATS data from the 15 min Reporting server, both post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |