# SUBSET 093: Data Transmission Interference and Error-Free periods

Purpose:	This test will measure the Data Transmission Interference and the Error-Free periods against the requirements of the Subset-093.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via QATS Railway and/or QATS UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

This test will be executed as per the O2475 specifications.

During the test a continuous CS Data call is established to the landside station.

At least two tests will occur: one at 4800 bps, one at 9600 bps in order to have results for both speeds for the area under test.

At the mobile side, at access point Igsmr, the ROMES mobile station is used.

At the fixed end the landside station is used.

The software (on both ends) sends data without any safety layer over the link: the protocol does not send any error correction and activates no retransmission. The equipment shall be set-up to make a continuous call. The bits are sent with 4800/9600 bps using the V.110 transmission protocol.

Both ends will transmit data simultaneously. The data from the Mobile side is received at the landside station, and the data transmitted from the landside station will be received by the Mobile side.  This allows to test the uplink and the downlink simultaneously, as the data being sent in both directions is different.

Randomized packets of 30 bytes are sent.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Data Transmission Interference and the Error-Free periods will be calculated during post-processing.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all ETCS-2 CS traffic, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Railway data, specific for UAP, reported from the 15 min Reporting server and imported to NPM via the QATS-NPM interface.

The QATS Railway system, as specified under a, monitors all ETCS-2 CS traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to point b, specific UAP, only calls generated by UAP are monitored.

The Data Transmission Interference and the Error-Free periods will be calculated and reported within NPM.

Additional information:

- For validation purposes:

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM:

- Reports based on all operational ETCS-2 CS traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational ETCS-2 CS traffic per BSC (if present on BSC)

- Reports based purely on UAP

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational ETCS-2 CS traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

TI and EFP: The test is successful if it can be stated with a confidence interval of 95% that the Transmission interference rate criterion (95% < 0.8 seconds and 99% < 1 second) and error-free period criterion (95% > 20 seconds and 99% > 7 seconds) can be fulfilled.

Data Transmission Interference and the Error-Free periods:

- For validation purpose:

Transmission interference period TTI:

- TTI starts with the first erroneous data frame. An erroneous data frame is a 30 bytes frame bearing at least one bit in the received data stream that does not comply with the associated bits of the transmitted data frame.

- TTI ends before the first error-free data frame. An error-free data frame is a 30 bytes data frame bearing no bit in the received data frame that does not comply with the associated bits of the transmitted data frame.

TTI ends before the first error-free data frame. An error-free data frame is a 30 bytes data frame bearing no bit in the received data frame that does not comply with the associated bits of the transmitted data frame.

Transmission recovery period TREC:

- TREC starts with the first error-free data frame.

- TREC ends before the first erroneous data frame.

In case of a TREC period equal or less than 4 consecutive error-free data frames, this short error free period shall be counted as one error period and shall be merged with the adjacent Transmission interference period TTI. A TREC period always starts with 5 or more consecutive error-free data frames.

- An interference, 2 frames from first to last erroneous frame.

- An error-free series of frames longer than or equal to 5 frames, TTI = 2 frames and TREC = 6 frames.

- An interference, 2 frames from first to last erroneous frame.

- An error-free series of frames shorter than 5 frames, consider D as an interfered period and merged it with C and E.

- An isolated interference of 1 frame.

- Back to a series of error-free frames longer than or equal to 5 frames, TTI is incremented with the sum of C+D+E, considered as a single interfered period. TTI = 6 frames. TREC is incremented until the next erroneous frame.

- For monitoring purpose:

Transmission interference period TTI:

- TTI starts with the first erroneous data frame. An erroneous data frame is a 30 bytes frame bearing at least one bit in the received data stream that does not comply with the associated bits of the transmitted data frame.

- TTI ends before the first error-free data frame. An error-free data frame is a 30 bytes data frame bearing no bit in the received data frame that does not comply with the associated bits of the transmitted data frame.

Transmission recovery period TREC:

- TREC starts with the first error-free data frame.

- TREC ends before the first erroneous data frame.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | TI+EF (DL & UL) (Romes) | TI+EF (DL & UL) (QATS) |
| Target | IF: 95% < 800 ms IF: 99% < 1000 ms EF: 95% > 20 s EF: 99% > 7 s | IF: 95% < 800 ms IF: 99% < 1000 ms EF: 95% > 20 s EF: 99% > 7 s |
| Post-processing | Required | Required |
| Minimum sample size | 1500 samples (30 bytes) per track kilometer per driving direction (deviations are possible on a project basis, to be agreed in the test plan). | N/A |
| KPI number | N/A | TI: KPI129 EFP: KPI130 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | N/A |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | The antennas next to the antenna used for this test can only be used for passive tests (no uplink transmission). | N/A |
| Band-Pass filter on antenna | Yes | N/A |
| Template | TIR_Data_Call | N/A |
| Remark | The required sample size mean that 2 drives per direction are needed as long as the driving speed does not exceed 153 km/h. | N/A |

Deliverables:	Validation: RSCMD file containing all relevant data and post-processed data.

Monitoring: NPM reports

Note: the Romes measurement data and QATS data from the 15 min Reporting server, both post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |