# Amount of TBF Rejections

Purpose:	This test will measure the Amount of TBF Rejections.

A distinction is made between:

- Dedicated measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via NetAct Counters

Test configuration:

Description:

- For dedicated measurements purposes, the ROMES Drive-Test System is used for this test.

The test will evaluate the Amount of TBF Rejections.

All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Amount of TBF Rejections will be calculated during post-processing.

- For continuous network monitoring purposes one source will be used, namely:

  - Netact counters via direct integration into NPM.

The Amount of TBF Rejections (per cause) will be calculated and reported within NPM.

Note: the amount of TBF Rejections per cause can only be evaluated by Netact counters coming from the FlexiBSS platform. The ROMES Drive-Test System is not able to evaluate the amount of TBF Rejections per cause, only the total amount of TBF Rejections can be evaluated.

Additional information:

- For dedicated measurements purposes:

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM;

- Reports based on all operational traffic

- Reports based on all operational traffic per Cell

Required hardware and Software:

- For dedicated measurements purposes:

- ROMES Drive-Test System

- Mobile IM3 filter activated

- For monitoring purposes:

- NetAct

Prerequisites:

- For dedicated measurements purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both Dedicated measurements and Monitoring:

Amount of TBF Rejections: T.b.d.

Summary:

|  | Dedicated measurements (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Amount of TBF Rejections (Romes) | Amount of TBF Rejections per cause (Netact) |
| Target | T.b.d. | N/A |
| Post-processing | Required | Required |
| Minimum sample size | T.b.d. | N/A |
| KPI number | N/A | KPI69 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | Cell |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | T.b.d. | N/A |
| Remark | N/A | N/A |

Deliverables:	Dedicated measurements: RSCMD file containing all relevant data and post-processed data.

Monitoring: NPM reports

Note: the Romes measurement data, post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Dedicated measurements only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |