# Call Setup Time and Call Setup Success Rate for Voice MOC PtP

Purpose:	This test will measure the call setup time and call setup success rate for Mobile Originating Normal Operations Voice Calls (MOC).

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained

- Netact Counters (only CSSR)

- QATS Railway

- QATS Drive Test UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

During the test short voice calls of normal priority (Priority 4) to the responder will be made.

A minimum of 200 calls per BSC will be done.

For the sake of efficiency, this test will also evaluate the Call Setup Success Rate by using the same calls to calculate the call setup success rate. All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Call Setup Time and Call Setup Success Rate will be calculated during post-processing.

- For continuous network monitoring purposes three sources will be used, namely:

  - Netact counters via direct integration into NPM (only CSSR).

  - QATS Railway data of all traffic, reported from the Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Drive Test data, generated by the QATS UAP’s, reported via TSV file principle to the Reporting server and imported to NPM via theTSV-XML converter and MVI Measurement Systems.

Both systems, as specified under 1 and 2, monitor all traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 3 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. The operator defines the duration of the scenario. The operator can schedule the scenario to generate the KPI data based on UAP operation in trains.

In the TSV file dt_voice_call_setup_time_moftc_railway the Call Setup Time is stored with time stamps and position information
In the TSV file dt_xdr_railway the call information to determine the call setup success rate is stored with time stamps and position information

Additional information:

- For validation purposes:

- Call window: 30 seconds
- Call duration: 10 seconds
- Minimum pause: 10 seconds

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM;

- Reports based on all operational traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational traffic per BSC

- Reports based purely on UAP.

- QATS Drive Test UAP

During the measurement a cycle of 3 call types are combined; Fixed Originated Mobile Terminated (FOMTC), Mobile Originated Fixed Terminated (MOFTC), Mobile Originated Mobile Terminated (MOMTC).
If a call fails (i.e. unsuccessful) the same call type will be retried.

- Call duration:		5 seconds
- Interval between calls:	10 seconds

Note: During this scenario 3 types of calls are measured. The call rate of MOFTC calls is 1/3. So every minute approximately 1 MOFTC calls is made during this scenario.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- Netact

- QATS Railway for reporting on all operational traffic.

- QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both Validation and Monitoring:

Call Setup Time: 95% of all MOC call setups within the reporting period must be established in less than 5 seconds

Call Setup Success Rate: 99% of all MOC call setups within the reporting period must be established within 7.5 seconds

Notes: no statistical relevance is obtained if less than 200 calls per BSC (Validation only).

Call Setup Time and/or Call Setup Success Rate:

- For validation purpose:

Call Setup Time: The time difference between the Layer 3 messages “Alerting” and “Channel Request”, measured at the Igsmr Interface.

Call Setup Success: A call is considered successfully set up if the call setup time does not exceed 7.5 seconds.

- Monitoring purpose

- QATS Railway:

Call Setup Time: The time difference between “Alerting” and “CM Service Request”, measured at A interface (see table below) of priority 4 voice calls.

Remark: The reported value of QATS Railway does not include the Um interface.

Call Setup Success: N/a

Remark: QATS Railway only considers ‘end cause’ and ‘end event’ for CSSR.

Call setup time

- QATS Drive Test UAP:

Call Setup Time: The time difference between “ATD” and “CONNECT” AT commands, measured at Igsmr_UAP interface.

Datafile:	dt_voice_call_setup_time_moftc_railway
Parameter: 	setup_time

Call Setup Success: A call is considered successfully set up if the call setup time does not exceed 7.5 seconds.

# Failed calls:

Datafile:	dt_xdr_railway
Filter:		xdr_type=1 AND xdr_subtype=1 AND xdr-direction=1 AND
		um_failure=2 AND call established=0

SUM

Datafile:	dt_voice_call_setup_time_moftc_railway
Parameter: 	setup_time
Filter:		setup_time > 7.5 sec

# Call attempts

Datafile:	xdr_railway
Filter:		xdr_type=1 AND xdr_subtype=1 AND xdr-direction=1

Call Setup Success Rate = ((#Call attempts- #Failed calls) / # Call Attempts) * 100%

Note: NPM uses different parameter names ten QATS/UAP. In the tables the criteria to be used to calculate the KPI are described.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Call Setup Time MOC (Romes) | Call Setup Time MOC (QATS UAP) |
| Target | 95% < 5seconds | 95% < 5 seconds |
| Post-processing | Required | Required |
| Minimum sample size | 200 calls per BSC | N/A |
| KPI number | N/A | KPI2 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | BSC |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_MR_8W_Voice_CSSR-CST_02.027-R | E-CST |
| Remark | N/A | N/A |

| Name | Call Setup Success Rate MOC (Romes) | Call Setup Success Rate MOC (Netact/QATS UAP) |
| --- | --- | --- |
| Target | 99% ≤ 7.5 seconds | 99% < 7.5 seconds 99% Success rate (sub-system counters) |
| Post-processing | Required | Required |
| Minimum sample size | 200 calls per BSC | N/A |
| KPI number | N/A | KPI7 |  |
| Reporting period | N/A | Monthly |  |  |
| Reporting level | N/A | BSC |  |  |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_MR_8W_Voice_CSSR-CST_02.027-R | E-CST 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and post-processed data.

Monitoring:

- NPM reports

- QATS 15’database; table xdr_railway

- QATS DT TSV-files; dt_voice_call_setup_time_moftc_railway; dt_xdr_railway

Note: the Romes measurement data, QATS data from the 15 min Reporting server and QDT UAP TSV-files, all post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |