# Call Setup Time and Call Setup Success Rate

# Class II: Normal Operations VGCS MOC with dispatcher

Purpose:	This test will measure the call setup time and call setup success rate of a Mobile Originating Normal Operations VGCS Call (MOC) with dispatcher.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via

- Netact Counters (only CSSR)

- QATS Railway

- QATS Drive Test UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test together with a fixed station is used that is authorized to make Group Calls.

Optionally a Mobile Station can be used to check actual cell broadcast channel activation.  This will, however, have no impact on call setup time and can be left out.

For the sake of efficiency, this test will also evaluate the Call Setup Success Rate by using the same calls to calculate the call setup success rate.

A minimum of 200 calls per BSC will be done.

All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Call Setup Time and Call Setup Success Rate will be calculated during post-processing.

- For continuous network monitoring purposes three sources will be used, namely:

  - Netact counters via direct integration into NPM (only CSSR).

  - QATS Railway data of all traffic, reported from the Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Drive Test data, generated by the QATS UAP’s, reported via TSV file principle to the Reporting server and imported to NPM via the TSV-XML converter and MVI Measurement systems.

Both systems, as specified under 1 and 2, monitor all traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 3 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

The Call Setup Time measured by the UAP complies to the measurement method End-to-End in the 0-2875 specification.

In the TSV file dt_vgcs_rec_setup_time_railway the Call Setup Time is stored with time stamps and position information
In the TSV file dt_xdr_railway the call information to determine the call setup success rate is stored with time stamps and position information

Additional information:

- For validation purposes:

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM;

- Reports based on all operational traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational traffic per BSC

- Reports based purely on UAP.

- QATS Drive Test UAP

During the measurement a cycle of 2 call types are combined; Mobile Originated VGCS and Fixed Originated VGCS.

- Group Id:			185

- Call duration:		15 seconds
- Interval between calls:	15 seconds

If a call fails to setup correctly , a treshold timer stops the call after 5 minutes.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System

- Mobile IM3 filter activated

- For monitoring purposes:

- Netact

- QATS Railway for reporting on all operational traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

Call Setup Time: 95% of all VGCS call setups within the reporting period must be established in less than 5 seconds

Call Setup Success Rate: 99% of all VGCS call setups within the reporting period must be established within 7.5 seconds

Note: no statistical relevance is obtained if less than 200 calls (Validation only).

Call Setup Time and/or Call Setup Success Rate:

- For validation purpose:

Call Setup Time: The time difference between the Layer 3 messages “CONNECT” and “Channel Request”, measured at the Igsmr Interface.

Call Setup Success: A call is considered successfully set up if the call setup time does not exceed 7.5 seconds.

- For monitoring purpose:

- QATS Railway

Call Setup Time: The time difference between “DT1 GCC Connect” and “CReq CL3 info”, measured at A interface of Mobile Originating Voice Group Services Calls.

Call Setup Success: A call is considered successful if the call has been established. Call Setup Success Rate is the percentage of established calls of all call attempts. In the NPM system the 7.5 sec criteria is used as well to determine the Call setup success rate.

Call setup time

- QATS Drive Test UAP

Call Setup Time: The time difference between “SETUP” and “CONNECT” AT commands, measured at Igsm interface.

Datafile:	dt_vgcs_rec_setup_time_railway
Parameter: 	setup_time
Filter:

Call Setup Success: A call is considered successfully set up if the call setup time does not exceed 7,5 seconds.

# Failed calls:

Datafile:	dt_xdr_railway
Parameter:	
Filter:		xdr_type=1 AND xdr_subtype=1 AND xdr_direction=1 AND
		adr_application_type=30
		um_failure=2 AND call established=0 

SUM

Datafile:	dt_vgcs_rec_setup_time_railway
Parameter: 	setup_time
Filter:		setup_time > 7,5 sec

# Call attempts

Datafile:	xdr_railway
Parameter:	
Filter:		xdr_type=1 AND xdr_subtype=1 AND xdr_direction=1 AND
		xdr_application_type=30

Call Setup Success Rate = ((#Call attempts- #Failed calls) / # Call Attempts) x 100%

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Call Setup Time VGCS (Romes) | Call Setup Time VGCS (QATS) |
| Target | 95% < 5 seconds | 95% < 5 seconds |
| Post-processing | Required | Required |
| Minimum sample size | 200 calls per BSC | N/A |
| KPI number | N/A | KPI5 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | BSC |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_ETSI_8W_VGCS_200 | F-VGCS |
| Remark | N/A | N/A |

| Name | Call Setup Success Rate VGCS (Romes) | Call Setup Success Rate VGCS (Netact/QATS) |
| --- | --- | --- |
| Target | 99% ≤ 7.5 seconds | 99% < 7.5 seconds 99% Success rate (sub-system counters) |
| Post-processing | Required | Required |
| Minimum sample size | 200 calls per BSC | N/A |
| KPI number | N/A | KPI10 |  |
| Reporting period | N/A | Monthly |  |  |
| Reporting level | N/A | BSC |  |  |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_ETSI_8W_ VGCS_200 | F-VGCS 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and post-processed data.

Monitoring:

- NPM reports

- QATS 15’database; table xdr_railway

- QATS DT TSV-files; dt_vgcs_rec_setup_time_railway; dt_xdr_railway

Note: the Romes measurement data, QATS data from the 15 min Reporting server and QDT UAP TSV-files, , both post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |