# Handover Success Rate

Purpose:	This test will measure the Handover Success Rate

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained

- Netact Counters

- QATS Railway

- QATS Drive Test UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

For the whole duration of the test, the mobile station will establish a dedicated voice call to the responder.

All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

Interruption Time at Handover will be calculated during post-processing.

- For continuous network monitoring purposes three sources will be used, namely:

  - Netact counters via direct integration into NPM.

  - QATS Railway data of all traffic, reported from the Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Drive Test data, generated by the QATS UAP, reported by the TSV-file principle to the Reporting server and imported NPM via to TSV-XML converter and MVI Measurement Systems.

Both systems, as specified under 1 and 2 monitor all traffic, of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 3 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. In the scenario the type and duration of the connections are described to gather the necessary information for the KPI. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

In the TSV file dt_xdr_ho_railway the call information to determine the call setup success rate is stored with time stamps and position information

Additional information:

- For validation purposes:

- Call window: long (endless) voice call

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Based on Netact Counters or QATS Railway all HO’s are taken into account. A separate monitoring takes place via UAP.

Several reports are available in NPM;

- Reports based on all operational traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational traffic per BSC

- Reports based purely on UAP.

- QATS Drive Test UAP

Call: long voice call (3600 sec)

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- Netact

- QATS Railway for reporting on all operational traffic.

- QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

HO Success Rate: 99% of all handovers are successful.

Note: no statistical relevance is obtained if less than 600 handovers.

Handover Success Rate:

- For validation purpose:

HO Success Rate: Based on Layer 3 messages “HANDOVER COMMAND” and “HANDOVER COMPLETE”, measured at the Igsmr Interface the Handover Success Rate is calculated.

In case a message “HANDOVER FAILURE” is reported, or no message at all, the handover is considered as unsuccessful.

Note: A handover failure does not necessarily mean that a call drop occurs: in most cases, the MS returns on the previous channel and another handover is attempted if the conditions for a handover are still fulfilled.

- For monitoring purpose:

- QATS Railway:

HO Success Rate Live traffic: Based on the QATS xdr_ho_railway table the Handover Success Rate will be calculated of all live traffic.

- QATS Drive Test UAP:

HO Success Rate UAP: Based on QATS data the Handover Success Rate of UAP calls will be calculated.

#Failed Handovers

Datafile:	dt_xdr_ho_railway
Parameter: 	
Filter:		ho_is_failure=1

#Handover Attempts

Datafile:	dt_xdr_ho_railway
Parameter: 	
Filter:		All entries in the file

Handover Success Rate = ((#Handover Attempts- #Failed Handovers) / # Handover Attempts) x 100%

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Handover Success Rate (Romes) | Handover Success Rate (Netact/QATS) |
| Target | 99% | 99% |
| Post-processing | Required | Required |
| Minimum sample size | 600 handovers per BSC | N/A |
| KPI number | N/A | KPI103 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | BTS |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_8W_Long_Voice | E-MOS 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

  - RSCMD file containing all relevant data and post-processed data.

Monitoring:

  - NPM reports

  - QATS 15’database; table xdr_ho_railway

  - QATS DT TSV-files; dt_xdr_ho_railway

Note: the Romes measurement data, QATS data from the 15 min Reporting server and QATS Drive Test UAP TSV files, all post-processed and handled via MVI’s, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |