# Interruption Time at Handover

Purpose:	This test will measure the Interruption Time at Handover

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via

- QATS Railway

- QATS Drive Test UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

For the whole duration of the test, the mobile station will establish a dedicated voice call to the responder.

The postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

Interruption Time at Handover will be calculated during post-processing.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all live traffic reported from the Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Drive Testdata, generated by the QATS UAP’s reported  via the TSV principle to the Reporting server and imported to NPM via the TSV-XML converter and MVI Measurement Systems.

The QATS Railway systems, as specified under 1, monitors all traffic, of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 2 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

In the TSV file dt_xdr_ho_railway the Handover Duration is stored with time stamps and position information

Additional information:

- For validation purposes:

- Call window: long (endless) voice call

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

All HO’s are taken into account, without split over different user groups.

Several reports are available in NPM;

- Reports based on all operational traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational traffic per BSC

- Reports based purely on UAP.

- QATS Drive Test UAP

Call: long  voice call (3600 sec)

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational traffic.

- QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

HO Time: 95% of all Interruption Time at Handover measurements within the reporting period must be established in less than 500 milliseconds

Note: no statistical relevance is obtained if less than 600 handover.

Interruption Time at Handover:

- For validation purpose:

HO Time: The time difference between the Layer 3 messages “HANDOVER COMMAND” and “HANDOVER COMPLETE” or “HANDOVER FAILURE”, whichever comes first, measured at the Igsmr Interface.

- For monitoring purpose:

- QATS Railway:

HO Time: The time difference between “HOCommand” and “HOComplete”, measured at A interface (reported value duration in the xdr_ho_railway table) of relevant voice calls.

HO Time

- QATS Drive Test UAP:

HO Time: The time difference between “HOCommand” and “HOComplete”, measured at Um interface (reported value duration in the dt_xdr_ho_railway TSV file) of relevant voice calls.

Datafile:	dt_xdr_ho_railway
Parameter: 	ho_duration
Filter:

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Interruption Time at Handover (Romes) | Interruption Time at Handover (QATS) |
| Target | 95% < 500 milliseconds | 95% < 500 milliseconds |
| Post-processing | Required | Required |
| Minimum sample size | 600 handovers per BSC | N/A |
| KPI number | N/A | KPI104 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | BTS |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_8W_Long_Voice | E-MOS 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and post-processed data.

Monitoring:

- NPM reports

- QATS 15’database; table xdr_railway

- QATS DT TSV-files; dt_xdr_ho_railway

Note: the Romes measurement data, QATS data from the 15 min Reporting server and QATS Drive Test TSV files, all post-processed and handled via MVI’s, will be uploaded to NPM via the Multi-Vendor Interfaces.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |