# Received Level in Idle mode

Purpose:	This test will measure the received level in Idle Mode.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained

- QATS Drive Test UAP

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

During the test no calls will be made. The test will evaluate the downlink received level in Idle mode.

The postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Received Level in Idle mode will be calculated during post-processing, where the values per 100m section are reported.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Drive Test data, generated by the QATS UAP, reported by the TSV principle to the Reporting server and imported to NPM via the QATS-NPM interface.

In relation to 1 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

In the TSV file dt_radio_measurement_railway the RxLev Idle mode is stored with time stamps and position information

Additional information:

- For validation purposes:

Idle mode means that the MS is camping on cells, performing cell reselections only.

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

In NPM onereport is  available;

- One based purely on UAP.

- QATS Drive Test UAP

In Idle mode the GSM-R modem is camping on a cell and performing cell reselections. The RxLev values (L3 level) are made available by the GSM-R modem at a rate of approximately once every second (approximately  3500- 3600 RxLev samples per hour). A reduction of approximately 50% of all the RxLev values measured by the GSM-R modem in the UAP takes place.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both Validation and Monitoring:

Received Level in Idle mode values are reported per 100m section, where the following information is reported for each 100m section:

- Driving direction (up- or downdriving); reporting is done per driving direction, so there will be two reports per 100m section

- Most frequent Cell-ID

- Received level in Idle Mode Minimum (minimum of all minimums)

- Received level in Idle Mode Average (average of all minimums)

The output of the reported received level in Idle Mode is used by ProRail for defining the Gebiedsindeling (GI).

- For monitoring purposes:

- QATS Drive Test UAP

Datafile:	dt_radio_measurement_railway
Parameter: 	dl_rx_level_idle
Filter:		dl_measure_idle_count != NULL

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Received Level in Idle mode (Romes) | Received Level in Idle mode (QATS) |
| Target | N/A | N/A |
| Post-processing | Required | Required |
| Minimum sample size | N/A | N/A |
| KPI number | N/A | KPI102 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | Landelijk |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_8W_Answer_NQA | H-IDLE 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and post-processed data per 100m section.

Monitoring:

- NPM reports

- QATS DT TSV-files; dt_radio_measurement_railway

Note: the Romes measurement data and QDT UAP TSV-files, all  post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |