# PDP Context Activation Failure Rate and Success Rate

Purpose:	This test will measure the Amount of failed/Successfull PDP Context Activations.

A distinction is made between:

- Dedicated measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via

- NetAct Counters

- QATS Railway

- QATS Drive Test UAP

Test configuration:

Description:

- For dedicated measurements purposes, the ROMES Drive-Test System is used for this test.

The test will evaluate the Amount of failed/Successfull PDP Context Activations.

All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Amount of failed/Successfull PDP Context Activations will be calculated during post-processing.

- For continuous network monitoring purposes one source will be used, namely:

- Netact counters via direct integration into NPM.

- QATS Railway data of all traffic, reported from the Reporting server and imported to NPM via the QATS-NPM interface.

- QATS Drive Test data, generated by the QATS UAP’s, reported via TSV file principle to the Reporting server and imported to NPM via the TSV-XML converter and MVI Measurement Systemens.

Both systems, as specified under 1 and 2, monitor all traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 3 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

In the TSV file dt_gprs_failure_rate_railway the PDP failures are stored with time stamps and position information

Additional information:

- For dedicated measurements purposes:

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM;

- Reports based on all operational traffic

- Reports based on all operational traffic per Cell

- QATS Drive Test UAP

The following loop will be excuted during the run of the scenario:
GPRS Attach (avarage 2 s)

Wait 10 ms

PDP Context Qctiviation (avarage 1,5 s)

Wait 10 ms

PDP Context Deactivation (average 2 s)

Wait 10 ms

GPRS Detach (average 8 s)

Wait 10 s

No Traffic is sent between the PDP Context Activation and the PDP Context Deactivation.

In th event that a  procedure fails to complete it times out after 15 seconds, and is retried. If 5 consecutive attempts fail the loop restarts.

Required hardware and Software:

- For dedicated measurements purposes:

- ROMES Drive-Test System

- Mobile IM3 filter activated

- For monitoring purposes:

- NetAct

- QATS Drive Test and UAP for reporting purely on UAP

Prerequisites:

- For dedicated measurements purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both Dedicated measurements and Monitoring:

Failed/Successfull PDP Context Activation rate: Percentage of PDP Context Activation that have not succeeded/Succeeded in the reporting period.

- For monitoring purposes:

- QATS Drive Test UAP

# PDP Failures:

Datafile:	dt_gprs_failure_rate_railway 
Parameter:

Filter:		xdr_type=1280 AND failure=1

# PDP attempts

Datafile:	dt_gprs_failure_rate_railway
Filter:		xdr_type=1280

PDP Failure Rate = (#PDP Failure / #PDP Attempts) x 100%

Summary:

|  | Dedicated measurements (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Amount of failed PDP Context Activations (Romes) | Amount of failed PDP Context Activations (Netact) |
| Target | ≤ 2% | ≤ 2% |
| Post-processing | Required | Required |
| Minimum sample size | T.b.d. | N/A |
| KPI number | N/A | KPI114 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | Cell |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | T.b.d. | C-Context 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Dedicated measurements:

- RSCMD file containing all relevant data and post-processed data.

Monitoring:

- NPM reports

- QATS 15’database; table xdr_railway

- QATS DT TSV-file; dt_gprs_failure_rate_railway

Note: the Romes measurement data, post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Dedicated measurements only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |