# Voice Quality (PESQ)

Purpose:	This test will verify the voice quality (PESQ P862.1) for MOC and MTC.  MTM calls are allowed to be excluded because of practical limitations of the test equipment.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via

- QATS Drive Test UAP

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

For the whole duration of the test a dedicated voice call to the Landside station (SQA server) will be made.  Voice samples will be played in both directions and the transmitted sample will be compared to the original one.

An algorithm (PESQ P.862) will automatically calculate the PESQ P862.1 (Narrow Band) for both the uplink and downlink directions.

The extracted PESQ P862.1 scores distribution will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

- For monitoring purposes:

- QATS Drive Test UAP

During the test one voice call will be made. During the call, an 8 second English voice recording is played in both directions, followed by 2 seconds of silence. There is then a pause of 200ms before the 8 seconds of English voice recording is repeated. A pair of UL & DL measurements are recorded by the probe at the end of each cycle.

In the TSV file dt_mos_measurement_railway the up- and downlink MOS values (PESQ P862.1 scores) are stored with time stamps and position information.

Additional information:

- For validation purposes:

- Call window: long (endless) voice call

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

- Call window: long voice call (3600 sec)

The QDT UAP settings needed for this test are saved in Scenario E. The duration of the measurement can be configured in the scenario.

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Drive Test and UAP

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both validation and monitoring purpose:

Voice Quality(PESQ): 85% of the samples shall have a PESQ greater or equal to 3.

95% of the samples shall have a PESQ greater or equal to 2.

PESQ, Perceptual Evaluation of Speech Quality, is a family of standards comprising a test methodology for automated assessment of the speech quality as experienced by a user of a telephony system.

It is standardized as ITU-T Recommendation P.862.1

- For validation purposes:

No additional information

- For monitoring purposes:

- QATS Drive Test UAP

Datafile:	dt_mos_measurement_railway
Parameter: 	dl_mos; ul_mos
Information:	MOS values are assigned to the position where the speech quality sample is ended

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Voice Quality (Romes) | N/A |
| Target | 85% PESQ ≥ 3 95% PESQ ≥ 2 | N/A |
| Post-processing | Required, combining Romes and Landside files | N/A |
| Minimum sample size | None | N/A |
| KPI number | N/A | KPI118 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | BSC |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | The antennas next to the antenna used for this test can only be used for passive tests (no uplink transmission). | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_SQA_M4_02.004-R | E-MOS 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and PESQ P862.1 scores distribution report

Monitoring:

- QATS DT TSV-file; dt_mos_measurement_railway

Note: the Romes and QATS Drive Test TSV files will be uploaded to NPM via the Multi-Vendor Interfaces.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |