# Received Quality in Dedicated mode

Purpose:	This test will measure the Received Quality in Dedicated Mode.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via

- Netact Counters

- QATS Drive Test UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

For the whole duration of the test a dedicated voice call to the landside station (acting as a responder) will be made. The test will evaluate the downlink received quality.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Received Quality in Dedicated mode will be calculated during post-processing: an average of the reported values per Cell-ID will be done, as well the 95th and 99th percentile per Cell-ID.

Both the averaged Cell-ID and the measured raw data (1 value each 480 ms) will be reported.

The post-processed averaged data per Cell-ID, including the 95th percentile values and the 99th percentile values, will be uploaded into NPM via the Multi-Vendor Interface Measurement Train.

- For continuous network monitoring purposes two sources will be used, namely:

  - Netact counters via direct integration into NPM

  - QATSDrive Test data,  generated by the QATS UAP’s, reported via the TSV principle to  the Reporting server and imported t NPM via the TSV-XML converter and MVI Measurement Systems.

The system as specified under 1 monitor all traffic, of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 3 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. In the scenario the type and duration of the connections are described to gather the necessary information for the KPI. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

In the TSV file dt_radio_measurement_railway the DL_RxQual is stored with time stamps and position information

Additional information:

- For validation purposes:

Dedicated mode means that the MS is in a call.

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Two reports are available;

- One based on all operational traffic.

- One based purely on UAP.

- QATS Drive Test UAP

Call: long voice call (3600 sec)

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- Netact

- QATS QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For both Validation and Monitoring:

- Averaged RXQual per Cell-ID over the reporting period.

- 95th percentile value per Cell-ID over the reporting period.

- 99th percentile value per Cell-ID over the reporting period.

- For monitoring purposes:

- QATS Drive Test UAP (KPI123):

The DL_RxQual value is stored approximately every second in the TSV file (i.e. half of all the layer 3 RxQual samples are stored).During handovers RxQual samples can be missing for a second.

Datafile:	dt_radio_measurement_railway
Parameter:	dl_rx_qual_com_cs
Filter:

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Received Quality in Dedicated mode (Romes) | Received Quality in Dedicated mode (Netact/QATS) |
| Target | N/A | N/A |
| Post-processing | Required | Required |
| Minimum sample size | All values per cell | All values per cell |
| KPI number | N/A | KPI123 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | Trace |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | The antennas next to the antenna used for this test can only be used for passive tests (no uplink transmission). | N/A |
| Band-Pass filter on antenna | Yes | N/A |
| Template | ProRail_8W_ Long_Voice | E-MOS 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and post-processed data per Cell-ID.

Monitoring:

- NPM reports

- QATS Drive Test UAP TSV-file; dt_radio_measurement_railway

Note: the Romes measurement data and QATS Drive Test UAP TSV files, all post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |