# Subset-093: Connection Loss Rate

Purpose:	This test will verify the Connection Loss Rate.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via

  - QATS Railway

  - QATS Drive Test UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

This test shall be executed following the specifications of O-2475.

The mobile station will establish a data connection to the landside station.

As per O-2475, a total of 300 aggregated hours of data connection (9600 baud) have to be collected to validate this KPI (CLR < 10-2/hour) with a confidence level of 95%.

A reduced number of test hours may be used to validate this KPI with a lower confidence level (see further).

The test plan will mention which track is driven for this test and how many test hours are required.

All postprocessed Romes measurement files will be uploaded to NPM via the Multi-Vendor Interface for the measurement train.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Connection Loss Rate will be calculated during post-processing.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all ETCS-2 CS traffic, reported from the Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Drive Test data, generated by the UAP, reported via the TSV principles to the Reporting server and imported to NPM via the QATS-NPM interface.

The QATS Railway system, as specified under 1, monitors all ETCS-2 CS traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 2 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

The Connection Loss Rate complies to the measurement method ERTMS/GSM-R Quality of Service Specification (0-2475 v3). For the measurement a connection speed of 4800 bps is used

In the TSV file dt_connection_loss_rate_failure_detailed_railway the Connection Los Rate  is stored with time stamps and position information

Additional information:

- For validation purposes:

- Call window: long data call (3600 sec)

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM:

- Reports based on all operational ETCS-2 CS traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational ETCS-2 CS traffic per BSC (if present on BSC)

- Reports based purely on UAP

- QATS Drive Test UAP

During the test a long ETCS CDS call will be made. Case of a dropped call a new ETCS CDS call will be made 5 seconds after the call has dropped.

During the ETCS CDS call transfers of standard size packages will be repeated with an interval between the packages of 350ms for a speed of 4800bps.
The speed rate can be determined by the operator when configuring the scenario (9600/4800/2400 bps)

Note: The intervals between sentpackages are dependant on the used speed rate

Speed rate		Interval between packages
9600 bps			200 ms
4800 bps			350 ms
2400 bps			650 ms

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational ETCS-2 CS traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

- For validation purpose:

Connection Loss Rate: The KPI is successfully validated with a certain confidence level, if no connection loss occurs within a specific number of hours:

- 300 hours for 95% confidence level

- 69 hours for 50% confidence level

- 11 hours for 10% confidence level

- For monitoring purpose:

Connection Loss Rate: The KPI is successfully validated, if no connection loss occurs within a specific number of hours:

•	300 hours for 95% confidence level

- QATS Drive Test UAP

# Connection losses

Datafile:		dt_connection_loss_rate_failure_detail_railway
Parameter: 	failure_count
Filter:

Call duration

Datafile:		dt_transfer_delay_railway
Parameter: 	duration
Filter:		For each call the call duration is stop_time – start-time. Total call duration is the sum of 		all individual call durations.

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Connection Loss Rate (Romes) | Connection Loss Rate (QATS) |
| Target | CLR < 10-2 per hour | CLR < 10-2 per hour |
| Post-processing | Required | Required |
| Minimum sample size | To be defined | N/A |
| KPI number | N/A | KPI128 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | N/A |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_EndLess_Data_Call | B S093 TTD |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and post-processed data.

Monitoring:

- NPM reports

- QATS 15’database; table xdr_railway

- QATS DT TSV-files; dt_transfer_delay_railway, dt_connection_loss_rate_failure_detail_railway

Note: the Romes measurement data, QATS data from the 15 min Reporting server and QDT UAP TSV files, all post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |