# SUBSET-093: Network Registration Delay KPI

Purpose:	This test will measure the Network Registration Delay.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via

- QATS Railway

- QATS Drive Test UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

This test will verify the time it takes for the mobile station to register successfully:

- on the home network (ProRail’s network) when camping on a forbidden network (inbound border crossing)

- on a roaming network when camping on a forbidden network (outbound border crossing)

This test can be executed statically (no drive-test required).

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Network Registration Delay will be calculated during post-processing.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all traffic, reported from the Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Drive Test data, generated by the QATS UAP, reported via the TSV principle to the Reporting server and imported to NPM via the QATS-NPM interface..

The QATS Railway system, as specified under 1, monitors all traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 2 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

The Network Registration Delay measured by the UAP complies to Subset093 specifications and the measurement method ERTMS/GSM-R Quality of Service Specification (0-2475 v3).

In the TSV file dt_network_registration_delay_railway the Network Registration Delay is stored with time stamps and position information

Additional information:

- For validation purposes:

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM:

- Reports based on all operational traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational traffic per BSC

- Reports based purely on UAP

- QATS Drive Test UAP

During the test the following loop is executed.

Register to a foreign/forbidden network (approximately 15 seconds)

Register to an authorized network (approximately 10 seconds)

Wait 5 seconds

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational traffic.

- QATS Railway, QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

Network Registration Delay: The KPI is successfully validated if the 95th percentile is less than or equal to 30 seconds, if the 99th percentile is less than or equal to 35 seconds and if all samples are less than or equal to 40 seconds.

Note: no statistical relevance is obtained if less than 100 inbound calls and 100 outbound calls.

Network Registration Delay:

- For validation purpose:

Network Registration Delay: (As per O-2475) The network registration delay is the elapsed time between the registration request (AT command) “+COPS=1,2, network2select” and the indication of successful registration onto GSM-R network “+CREG: 1” or “+CREG: 5”, with “network2select”, measured at the Igsmr Interface, being:

- the Visited PLMN for the outbound border crossing test.

- the Home PLMN for the inbound border crossing test.

- For monitoring purpose (QATS Railway):

- QATS Railway

Network Registration Delay: The time difference between “ChannelRequired” and “LUAcc”, measured at A interface (reported value duration in the Xdr_duration_Railway_table) of relevant Network Location Updates.

Network Registration Delay

- QAT Drive Test

Network Registration Delay

Datafile:		dt_network_registration_delay_railway
Parameter: 	delay [ms]
Filter:

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Network Registration Delay (Romes) | Network Registration Delay (QATS) |
| Target | NRD ≤ 30 s (95%) NRD ≤ 35 s (99%) NRD ≤ 40 s (100%) | NRD ≤ 30 s (95%) NRD ≤ 35 s (99%) NRD ≤ 40 s (100%) |
| Post-processing | Required | Required |
| Minimum sample size | 100 samples outbound and 100 samples inbound (per BSC) | N/A |
| KPI number | N/A | KPI131 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | N/A |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ProRail_O2475_RegistrationDelay_DE_NetReg-R | B S093 NRD 1) |
| Remark | N/A | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and post-processed data.

Monitoring:

- NPM reports

- QATS DT TSV-files; dt_network_registration_delay_railway

Note: the Romes measurement data, QATS data from the 15 min Reporting server and QDT UAP TSV files, all post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |