# SUBSET 093: Circuit-Switched Data Transfer Delay

Purpose:	This test will measure the Circuit-Switched Data Transfer Delay of MO calls.

A distinction is made between:

- Validation measurements obtained via the ROMES drive test system

- Monitoring measurements obtained via

- QATS Railway

- QATS Drive Test UAP.

Test configuration :

Description:

- For validation purposes, the ROMES Drive-Test System is used for this test.

During the testing period automatic repeating frames are sent by the ROMES system.  The frames are echoed back by the landside station.

Delay times, sent and received frames are stored in the ROMES mobile station log files.

The ROMES software stores timestamps, GPS coordinates and the related layer 3 messages in log files.

The Circuit-Switched Data Transfer Delay periods will be calculated during post-processing.

During this test 100 samples per BSC area are measured.  The 100 samples in each BSC area are taken from at least 3 different cells.

- For continuous network monitoring purposes two sources will be used, namely:

  - QATS Railway data of all ETCS-2 CS traffic, reported from the Reporting server and imported to NPM via the QATS-NPM interface.

  - QATS Drive Test data, generated by the QATS UAP, reported via the TSV principle to the  Reporting server and imported to NPM via the QATS-NPM interface.

The QATS Railway system, as specified under 1, monitors all ETCS-2 CS traffic of the live network (operational Railway traffic, as well as traffic generated by test systems, like measurement train or any other test devices).

In relation to 2 (QATS Drive Test/UAP) the settings for the measurement of this KPI are stored in a scenario. The operator defines the duration of the scenario. The operator can schedule the scenario to retrieve the KPI data based on UAP operation in trains.

The Transfer Delay measured by the UAP complies  to the measurement method ERTMS/GSM-R Quality of Service Specification (0-2475 v3). For the measurement a connection speed of 4800 bps is used

In the TSV file dt_transfer_delay_railway the Transfer Delay  counters (also a distribution in 100ms intervals) are stored with time stamps and position information

Additional information:

- For validation purposes:

- The frame size is 30 bytes

- The CSD connection speed is 9600 bps

- The protocol is V110 (transparent mode)

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

As per O2475v3 (section 2.2.1):

Measuring for a higher data transfer rate also qualifies the test area for the lower rates (e.g. measuring at 9.6 kbps also qualifies 4.8 and 2.4)

This explains why the test is only executed at 9600 bps.

The ROMES settings needed for this test are saved in a ROMES template which can be loaded prior to executing the test.

- For monitoring purposes:

Several reports are available in NPM:

- Reports based on all operational ETCS-2 CS traffic, defined by ProRail as “Diensten Rapportage”

- Reports based on all operational ETCS-2 CS traffic per BSC (if present on BSC)

- Reports based purely on UAP

- QATS Drive Test UAP

- Call duration: 					3600 sec

- Time interval after a dropped call: 			5 sec

- Time interval regular end of a call (3600 sec ): 	5 sec

During the ETCS CDS call transfers of standard size packages will be repeated with an interval between the packages of 350ms for a speed of 4800bps.

Note: For other speeds rate different intervals between the packages are used

Speed rate		Interval between packages
9600 bps			200 ms
4800 bps			350 ms
2400 bps			650 ms

Required hardware and Software:

- For validation purposes:

- ROMES Drive-Test System, ROMES Landside station

- Mobile IM3 filter activated

- For monitoring purposes:

- QATS Railway for reporting on all operational ETCS-2 CS traffic.

- QATS Drive Test and QATS UAP’s.

Prerequisites:

- For validation purposes:

The BTS, BSC, MSC network elements are operational and tested.

Sufficient capacity shall be available on the GSM-R network to perform this test; congestion or blocking will invalidate the test.

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

Transfer Delay: The test can be considered as successful if it can be stated with a probability for right acceptance of 95% that at least 99% of all data transmission delay measurements meet to the requirement of a one-way transfer delay of less than or equal to 500 milliseconds.

Note: no statistical relevance is obtained if less than 100 samples.

Circuit-Switched Data Transfer Delay:

- For validation purpose:

Transfer Delay: The Transfer Delay is defined as the elapsed time between the request for transfer of a user data unit and the indication of successfully transferred end-to-end user data unit.

- For monitoring purpose:

Transfer Delay: The Transfer Delay is defined as the elapsed time between the request for transfer of a user data unit and the indication of successfully transferred end-to-end user data unit.

- QATS Drive Test UAP:

# Frame delay < 500ms

Datafile:		dt_transfer_delay_railway
Parameter: 	frame_delay_le_05 [number]
Filter:

# Frames

Datafile:		dt_transfer_delay_railway
Parameter: 	frame_delay_total_count [number]
Filter:

# Frames in 100ms interval (range 0 -999ms)

Datafile:		dt_transfer_delay_railway
Parameter: 	frame_delay_interval_1; frame_delau_interval_2;         ; frame_delay_interval_9
		[number]
Filter:

frame_delay_interval_1:  [0– 100 ms]

frame_delay_interval_i:  [(i-1) 100 – i*100 ms]

frame_delay_Interval_9:  [900 – 1000 ms]

Summary:

|  | Validation (A) | Monitoring (B) |
| --- | --- | --- |
| Name | Data Transmission Delay (Romes) | Data Transmission Delay (QATS) |
| Target | 99% ≤ 500 ms | 99% ≤ 500 ms |
| Post-processing | Required | Required |
| Minimum sample size | 100 samples | N/A |
| KPI number | N/A | KPI127 |
| Reporting period | N/A | Monthly |
| Reporting level | N/A | N/A |
| IM3 filter in MS | Activated | N/A |
| mpJules antenna constraint | None | N/A |
| Band-Pass filter on antenna | No | N/A |
| Template | ETCS_Transfer_Delay | B S093 TTD 1) |
| Remark |  | N/A |

1) See PRJ_172 UP for CPM Design Document v2.5

Deliverables:

Validation:

- RSCMD file containing all relevant data and post-processed data.

Monitoring:

- NPM reports

- QATS 15’database; table hdlc_rtt_railway

- QATS DT TSV-files; dt_transfer_delay_railway,

Note: the Romes measurement data, QATS data from the 15 min Reporting server and QDT UAP TSV files, all post-processed and handled via MVI, will be uploaded to NPM.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |