# NIT-EOG-PDPContext Activation

Purpose:	This test will measure the EoG PS PDP Context Activation delay time.

The test is valid for:

- Monitoring measurements obtained via

- QATS Drive Test UAP.

Test configuration :

Description:

- For network monitoring purposes one source will be used, namely:

  - QATS Drive Test data,  generated by the QATS UAP’s, reported via the TSV principle to  the TSV Reporting server and imported to NPM via the TSV-XML converter and MVI Measurement Systems.

In relation to 1 (QATS Drive Test/UAP) the settings for the measurement of this KPI are defined in a scenario. In the scenario the definition is made that will result in gathering the necessary information for the KPI. The operator defines the duration for repetition of the scenario. The operator can schedule the scenario repetition for a specified time period based on UAP operation in trains.

In the TSV file dt_gprs_delays_railway is stored with time stamps and position information

Additional information:

- For monitoring purposes:

- QATS Drive Test UAP

Report available;

- One based purely on UAP.

The scenario generates a PDP context at the start of the action, and performs a loop containing 1 DNS procedure per loop with 1 second between each DNS procedure. Each loop iteration generates one measurement. A failure threshold timer is set to 3 seconds.

Required hardware and Software:

- For monitoring purposes:

- QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For Monitoring:

PDP Context Activation Delay: The PDP Context Activation Delay is defined as the value of elapsed time from the request for PDP context activation to the indication of successful PDP context activation
The PDP Context Activation Delay should be less or equal to 3 seconds for 99% of PDP context activations
A PDP context activation with a delay greater than 3 seconds should be evaluated as an error.

The measured PDP Context Activation Delay available in the TSV files, will be grouped per 15 minutes period in the TSV/XML convertor software and via XML-files loaded into the NPM system. Within the ProRail Reporting Suite the:

- Averaged PDP Context Activation Delay

- Number of samples

- Number of failed samples

- 90th percentile value

- 95th percentile value

- 99th percentile value

is calculated over the selected reporting area and time period.

- For monitoring purposes:

- QATS Drive Test UAP (KPI134):

The PDP Context Activation delay time reported in milliseconds is found from pdp_context_activation_delay.

Datafile:	dt_gprs_delays_railway
Parameter:	pdp_context_activation_delay

Filter:		pdp_context_activation_delay_count = 1

Summary:

|  | Monitoring (B) |
| --- | --- |
| Name | PDP Context Activation Delay (QATS) |
| Target | ≤3 s (99%) |
| Post-processing | Required |
| Minimum sample size | All values per cell |
| KPI number | KPI134 |
| Reporting period | Monthly |
| Reporting level | Trace |
| IM3 filter in MS | N/A |
| mpJules antenna constraint | N/A |
| Band-Pass filter on antenna | N/A |
| Template | D S093 GPRS Delays |
| Remark | N/A |

Deliverables:

Monitoring:

- QATS Drive Test UAP TSV-file: dt_gprs_delays_railway

Note:

- The QATS Drive Test UAP TSV files, all post-processed and handled via MVI, will be uploaded to NPM

- Current UAP implementation is based on Subset-093 draft release v3.0.B.G. UAP implementation is subject to modification when new Subset-093 is implemented.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |