#### NIT-EOG-Transaction Transfer Delay

Purpose:	This test will measure the EoG Transaction Transfer delay time.

The test is valid for:

- Monitoring measurements obtained via

- QATS Drive Test UAP.

Test configuration :

Description:

- For network monitoring purposes one source will be used, namely:

  - QATS Drive Test data,  generated by the QATS UAP’s, reported via the TSV principle to  the TSV Reporting server and imported to NPM via the TSV-XML converter and MVI Measurement Systems.

In relation to 1 (QATS Drive Test/UAP) the settings for the measurement of this KPI are defined in a scenario. In the scenario the definition is made that will result in gathering the necessary information for the KPI. The operator defines the duration for repetition of the scenario. The operator can schedule the scenario repetition for a specified time period based on UAP operation in trains.

In the TSV file dt_gprs_delays_railway is stored with time stamps and position information

Additional information:

- For monitoring purposes:

- QATS Drive Test UAP

Report available;

- One based purely on UAP.

The scenario is used to generate Subset 093 transaction transfer measurement data for KPIs 136 (Transaction Transfer Delay OBU 100 octet), 137 (Transaction Transfer Delay RBC 320 octet) and 138 (Transaction Transfer Delay RBC 560 octet ). The scenario establishes a single PDP context for the duration specified and repeatedly transfers frames to measure transfer delay. In case of non-acknowledged frames, there is a time-out of 60 seconds.

Required hardware and Software:

- For monitoring purposes:

- QATS Drive Test and UAP for reporting purely on UAP.

Prerequisites:

- For monitoring purposes:

No specific prerequisites, however congestion or blocking may influence the results. Those latest are out of direct control and will be handled via the Capacity Management process.

KPI Definition:

For Monitoring:

Transaction Transfer Delay: The Transaction Transfer Delay is defined as the elapsed time from the request to send an IP packet at service access point of the transmitting wntity to the indication of the acknowledging IP packet at the service access point of the receiving entity
OBU originated transfer: The transaction transfer delay including one 100 octet IP packet and one 45 octet IP packet  shall be ≤1.6s (95%) and ≤ 2.6s (99%).
RBC originated transfer: The transaction transfer delay  including one 320 octet IP packet and on 45 octet IP packet shall be ≤ 2.0s (95%) and ≤ 3.0s (99%).
RBC originated transfer: The transaction transfer delay including one 560 octet IP packet and one 45 octet IP packet shall be ≤ 2.5s (95%) and ≤ 3.5s (99%).
The Transaction Transfer Delays greater than the 99% figure should be evaluated as an error.

The measured transaction transfer delay available in the TSV files, will be grouped per 15 minutes period in the TSV/XML convertor software and via XML-files loaded into the NPM system. Within the ProRail Reporting Suite the:

- Averaged Transaction Transfer Delay

- Number of samples

- 90th percentile value

- 95th percentile value and

- 99th percentile value

is calculated over the selected reporting area and time period.

- For monitoring purposes:

- QATS Drive Test UAP (KPI136):

The Transaction Transfer delay time (100 bytes) reported in milliseconds is found from etcs_dns_lookup_delay.

Datafile:	dt_gprs_delays_railway
Parameter:	ul_transfer_delay_100_bytes

Filter:		ul_transfer_delay_100_bytes_count = 1

- QATS Drive Test UAP (KPI137):

The Transaction Transfer delay time (320 bytes) reported in milliseconds is found from etcs_dns_lookup_delay.

Datafile:	dt_gprs_delays_railway
Parameter:	dl_transfer_delay_320_bytes

Filter:		dl_transfer_delay_320_bytes_count = 1

- QATS Drive Test UAP (KPI138):

The Transaction Transfer delay time (560 bytes) reported in milliseconds is found from etcs_dns_lookup_delay.

Datafile:	dt_gprs_delays_railway
Parameter:	dl_transfer_delay_560_bytes

Filter:		dl_transfer_delay_560_bytes_count = 1

Summary:

|  | Monitoring (B) |
| --- | --- |
| Name | Transaction Transfer Delay (QATS) |
| Target | OBU 100 octet; ≤ 1,6 s (95%); ≤ 2,6 s (99%) RBC 320 octet; ≤ 2,0 s (95%); ≤ 3,0 s (99%) RBC 560 octet; ≤ 2,5 s (95%); ≤ 3,5 s (99%) |
| Post-processing | Required |
| Minimum sample size | All values per cell |
| KPI number | KPI136, KPI137, KPI138 |
| Reporting period | Monthly |
| Reporting level | Trace |
| IM3 filter in MS | N/A |
| mpJules antenna constraint | N/A |
| Band-Pass filter on antenna | N/A |
| Template | D S093 TTD |
| Remark | N/A |

Deliverables:

Monitoring:

- QATS Drive Test UAP TSV-file: dt_gprs_delays_railway

Note:

- The QATS Drive Test UAP TSV files, all post-processed and handled via MVI, will be uploaded to NPM

- Current UAP implementation is based on Subset-093 draft release v3.0.B.G. UAP implementation is subject to modification when new Subset-093 is implemented.

The following observations / remarks have been made during the Acceptance Test (Validation only):

| Witness: |  |  |  |  |
| --- | --- | --- | --- | --- |

| Tester: |  |  |  |  |
| --- | --- | --- | --- | --- |
| Date: |  |  |  |  |

| Result obtained: |  |  |  |
| --- | --- | --- | --- |
|  | PASSED |  | FAILED |