# Sub-System Design Document

Measurement Systems

Table of contents

1	Introduction	3

1.1	Scope	3

2	GSM-R Drive Test System (Romes)	4

2.1	Purpose	4

2.2	General	5

2.2.1	Mobile part	6

2.2.2	Jumper cables	7

2.2.3	Romes Landside Units	9

2.2.4	Network Problem Analyzer (NPA)	13

2.2.5	Updates/upgrades	15

3	Autonomous Scanner (R&S TSMA)	16

3.1	Introduction	16

3.1.1	Measurements	16

3.2	Autonomous Scanner mobile part	17

3.2.1	Network Scanner	19

3.2.2	Battery Pack	21

3.2.3	Software	22

3.2.4	Remote access	23

3.2.5	4G modem	23

3.2.6	AC/DC Power Supply	24

3.2.7	GSM-R antenna including GPS	24

3.3	Landside Server	25

3.3.1	Hardware	26

3.3.2	Measurement data handling	26

4	Unattended Probes (UAP)	28

4.1	Unattended Probe solution	28

4.1.1	Monitored interfaces	30

4.1.2	Solution dimensioning	30

4.1.3	IT configuration	31

4.1.4	Connectivity UAP Landside	32

4.1.5	UAP Data logging	32

4.1.6	Archive rules	33

4.2	QATS Drive Test	34

4.2.1	Scenario management	36

4.2.2	Status and Fault management	36

4.2.3	Tracking and tracing	37

4.2.4	Congestion management	37

4.2.5	Remote Maintenance	38

4.3	Proposed scenario’s	38

4.3.1	Radio coverage measurements	38

4.3.2	Derived KPI fields	39

4.4	Hardware dimensioning	42

4.4.1	On-board Unattended Probe	42

4.4.2	UAP Hardware	42

4.4.3	UAP Software	43

4.4.4	UAP - UPS power supply	43

4.4.5	UAP software upgrade	43

4.4.6	UAP Data Export functionality	44

5	References	45

# Introduction

## Scope

The scope of the document is to provide a detailed overview of the Measurement Systems which are available:

- GSM-R Drive Test System (Romes)

- Autonomous Scanner (R&S TSMA)

- Unattended Probes (UAP)

The environmental requirements imposed by the GSM-R Drive Test System will be described in a separate document called “Specifications measurement vehicle”.

# GSM-R Drive Test System (Romes)

## Purpose

The GSM-R Drive Test System is intended to be used for tuning & optimization and/or validation measurements on the GSM-R network.

The validation of a GSM-R network focuses on the verification of the KPIs of the network and verifies if they correspond to the applicable specifications.  For validation measurements the focus is on the collection of statistical relevant data for KPI evaluation.

For tuning measurements, the focus lies on obtaining relevant measurement data allowing fine tuning/optimization of the network, in terms of antenna azimuths and parameters optimization. For this purpose, besides purely KPI information more detailed loggings are required, which will be used by radio planning specialists.  Coverage measurements are also part of this.

## General

The GSM-R Drive test system is based on the ROMES drive test system by Rohde & Schwarz.

The drive test system is composed of 3 main parts:

- The mobile part for utilization in a drive test vehicle (car or train)

- Landside units connected to the GSM-R core network via an ISDN PRI interface

- Network Problem Analyzer Software for evaluation of the measured KPIs

The system is housed in 2 x 3U rugged flight cases with covers (front, rear) and robust handles. The flight cases have an anti-vibration frame for plug-in components.

Three additional cases house various equipment as described in section 2.3.

A license is required to operate the drive test system. This license is installed on the system by means of hard lock USB dongles & license files.

The system is controlled by means of software, namely:

- ROMES running on a laptop for the mobile part of the system.  The ROMES software can be used for data collection (protected by the ROMES dongle), and for data replay, in which case the dongle is not necessary.

- SQA running on the desktop PCs forming the landside units (protected by the SQA dongles)

- NPA running on a separate computer to analyze the KPIs (protected by the NPA dongle)

The combination of mobile part, including wireless scanners & test mobiles and the landside units allows for scenario driven automated drive testing of the GSM-R network.

### Mobile part

The mobile part of the drive test system is composed of:

- One TSMW Radio Network analyzer allowing decoding of GSM layer 3 system information messages and interference detection (with a license for 1 out of 2 RF inputs)

- One TSML-CW Radio Network analyzer, allowing CW coverage measurements (80 MHz to 6 GHz)

- One TrioRail S75 2W Trace mobile

- 3 TrioRail 8W TRC5-trace mobiles

- One Sound Card for voice quality tests

- One 230V AC to 12V DC Convertor

- One Laptop with ROMES drive test software

- One USB –hard lock dongle with the ROMES license

- One Band-pass filter Kaelus DDF9007F1V1-1 including switch module (SWT9001F1V1-1) and DC injector (DCI9002F1V1-1)

Housings:

- 2 x 3U rugged flight cases, housing all the modules (except the laptop) requiring a 12V DC power supply, allowing utilization as well in a car.  This is the unique source of power required to power the entire system, including the laptop.

- One trolley case housing the 230V to 12V convertor, the laptop and all the system’s cables & small accessories

- One trolley case housing the Kaelus filter and its accessories.

- One flight case housing the Pegasem odometer, used as external odometer in vehicles not equipped with an in-built odometer.

Laptop specifications:

- Intel Core i7-4810MQ (2.8 GHz)

- 8 GB Ram

- Windows 7 64 bits Professional

The mobile part of the system is mainly intended to be used within a measurement train but can also be used within a car or even on the terrain as such. The requirements set towards the utilization in a measurement train or car are described in the document “Specifications measurement vehicle”.

The PC (laptop) with the ROMES drive-test software, controlling the mobile part, needs to be time-synchronized by means of the GPS.  This is automatically done by the ROMES software as long as a GPS is connected to it.

The ROMES system needs to be configured specifically depending on the test to be performed.  Each NIT document provides a reference to a ROMES template (XML file) containing the settings for the test to be performed.

The ROMES mobile part cannot be remote controlled; this feature is currently not available in the license provided.

There is no limitation as per the amount of measurements that can occur simultaneously in the Drive-test system: The four phones and two scanners can be used simultaneously.  There could be other limitations outside of the scope of the Drive-test system (cell capacity …).

It is not recommended to run other software on the same laptop while ROMES is acquiring data.

The ROMES software can be installed on several computers for replay purposes.  However, data collection is only possible with the USB dongle.  In both cases (replay & data collection), it is the same piece of software used.

Figure 2-1: Picture of the mobile ROMES system

### Jumper cables

The ROMES system contains a whole set of 8 jumper cables for GSM-R and 2 jumper cables for GPS.

The GSM-R jumper losses are indicated below.

| Jumper cable | Jumper loss (dB) |
| --- | --- |
| 1 | 1.4 |
| 2 | 1.1 |
| 3 | 1.3 |
| 4 | 1.4 |
| 5 | 1.3 |
| 6 | 1.4 |
| 7 | 1.3 |
| 8 | 1.3 |

Table 2-1: Jumper cable losses

### Romes Landside Units

#### General

Two landside units are available, allowing the termination of mobile originated calls (MOC) from the mobile part of the drive test system, and performing and analyzing end-to-end tests.  These calls will be circuit-switched calls that will run through the different network elements (radio, transmission & core network), from the mobile unit to the landside unit. 
In a standard test configuration, one landside unit will be installed per MSS, as indicated in  Figure 2.

Figure 2-2: ROMES Landside connections

The ROMES Landside units will be continuously powered up and remain connected to the core network. The PRI interfaces of the core network are monitored via the Netact system of core network. Further as all sessions on this interface are terminated on the landside unit, with own logging software (which is part of the ROMES solution), no additional monitoring is required on the PRI lines towards the landside units.

Landside units do host software licenses in the form of a license file together with a USB dongle (SQA license).

The standard Mobirail antivirus software shall be installed on each Landside PC.

#### Hardware

Each landside PC will be running under Windows 7 32 bits Professional and will have the following specifications:

- Core i5

- 4 GB Ram

- 500 GB HDD

- Power requirements: 275 W

The PRI interface in the PC is a Dialogic Diva 8-channel ISDN PRI PCI board.  This PCI board can handle 8 B-channels (E1 timeslots) simultaneously.

Each PC provided is a Dell Optiplex 7010.

#### Installation locations

The landside PCs will be installed in two different locations: Amsterdam & Rotterdam.

#### Connections

For the standard setup each landside unit will be connected via an ISDN PRI interface with the Core network.  An entire E1 is dedicated to each landside unit.  
The ISDN PRI interface of the landside unit can handle 8 lines.

The attributed numbers on the ISDN Lines are the following:

Depending on the tests to be executed the configuration can be remotely changed, corresponding to the test sequence running on the mobile part of the drive test system.

The ROMES Landside Unit can also be used to terminate mobile originated calls from other test equipment, i.e. Kapsch MT2 or future planned so-called unattended probes.

The landside units are connected to the Mobirail LAN via the Ethernet interface for the following reasons:

- Accessing an NTP Server, for time synchronization

- Remote Access for Authorized persons:

  - data collection

  - configuration changes

  - software updates as part of the maintenance contract

Remote access to the landside units is done via RDP (using standard ports) from within the Mobirail LAN, following the standard RAS procedures, as described in the SSDD RAS.

#### Remote setup of a landside station

For End-to-End testing up to an RBC location, a landside unit can be temporarily moved towards this location. The requirements towards such an RBC location, for utilizing the landside unit over there are specified here-below:

- The landside unit must have a link to the MGW (ISDN line), with 8 lines available (see 2.4.2)

- The landside unit must have a link to the Mobirail LAN via the Ethernet interface, for the purposes highlighted in 2.4.4. In case such a link is not available, only local configuration changes will be performed

### Network Problem Analyzer (NPA)

The Network Problem Analyzer (NPA) is a standalone Windows application (completely independent from the ROMES software) provided to automate the task of finding problem spots in a radio telecommunication network. It scans the content of measurement files created with the R&S ROMES software (*.rscmd files).  Such files contain data recorded during drive-tests made with mobiles, network scanners and other devices (i.e. GPS).
When NPA is not used on the same PC/LAN as the ROMES software, the *.rscmd files must be transferred to the location where NPA is used; this can be done by means of a USB flash disk for example.

The size of the *.rscmd files depends on the type of measurement executed.  A typical day of measurements can generate up to 10 GB of *.rscmd files.

In summary the tool simplifies repetitive tasks occurring in the network optimization by supporting in:

- Finding problematic areas in radio communication networks faster

- Avoiding manual analysis of measurement files where no problems exist

- Shortening file-scanning times in R&S ROMES by only showing parts of the measurement file

- Providing an overview over a set of files

- Combining data from various files to build KPIs summaries

Figure 3 shows the interaction between ROMES drive test software and NPA.

Figure 2-3: NPA Workflow

Utilizing the NPA software requires a valid license, which is available on a USB-dongle (NPA license).  The NPA software can be used on any Windows computer (see minimum requirements below) so long as the dongle is connected to it.

NPA system minimum requirements:

- Core i5 (Haswell) or higher with 2 GHz clock or faster

- 8 GB Ram

- Windows 7 64 bits Professional

The NPA software can run on the ROMES laptop, concurrently with ROMES in replay mode.

### Updates/upgrades

An update contract is in place, which includes software updates for the ROMES and NPA software. As part of the update contract regular software updates will be installed on the systems (2015 update schedule, indicates updates each 2 to 4 month). Whenever an update comes available this update will be installed by Mobirail on the drive test system, even if the update would not include updates on the software parts used within the GSM-R project. This is required for the purposes of the technical support provided by Rohde & Schwarz as part of the software update contract.

The contract does not include software upgrades (e.g. to a new version of ROMES for example).

# Autonomous Scanner (R&S TSMA)

## Introduction

The Autonomous Scanner system is intended to be used for performing autonomous Coverage and C/I measurements in the GSM-R network.

The Autonomous Scanner system comprises of 5 mobile sets, which are installed in 5 different (Eurailscout) trains, and one Landside Server, which is installed at the GSM-R Rt-Wah core location.

The mobile part of the Autonomous Scanner system is described in 3.2.

The Landside Server is described in 3.3.

### Measurements

The Autonomous Scanner system is used to cover the following two measurements:

- Coverage measurements

- C/I measurements

#### Coverage

The Coverage needs to be verified on the designated tracks, evaluated in 100m sections.

The Autonomous Scanner system collects at least every 10 cm one coverage measurement sample, based on Time Trigger. Because this Time Trigger is based on the maximum allowed train speed, most of the time more than 1 coverage measurement sample is collected per 10 cm.

For more info regarding the Coverage KPI, please refer to [NIT-Cov]

#### C/I

The C/I need to be verified on the designated tracks, evaluated in 100m sections.

The Autonomous Scanner system collects continuously the Coverage level & C/I downlink for each GSM-R channel, meaning that each measurement sample will contain up to 19 C/I values and their corresponding Coverage level (“up to” is quoted because in practice the scanner can never decode the C/I for all 19 channels at a single point in time, mostly due to interference or too low received levels).

The C/I is captured by the Autonomous Scanner system, based on Time Trigger.  The number of samples per 100 meters will depend on the train speed and on the fact that the Autonomous Scanner system also collects Coverage data.

In normal operation the Autonomous Scanner system collects around 10 samples per second.

For more info regarding the C/I KPI, please refer to [NIT-C_I]

## Autonomous Scanner mobile part

Figure 3-1 Autonomous Scanner mobile part

The mobile part of the Autonomous Scanner system is composed of:

- A R&S TSMA6 solution consisting of two parts:

  - R&S TSME6 Network Scanner combined with an integrated computer (R&S TSMA6)

  - Battery Pack (R&S TSMA6-BP)

- One 4G modem

- A GSM-R antenna with integrated GPS antenna

- An AC/DC power supply

The following figures show the R&S TSMA6 solution (front view and back view). The R&S TSMA6 (Network Scanner) is stacked upon the R&S TSMA6-BP (Battery Pack).

Figure 3-2 R&S TSMA6 solution front view

Figure 3-3 R&S TSMA6 solution back view

For more information about the R&S TSMA6 solution please refer to the [R&S TSMA6 Product Brochure]

### Network Scanner

The R&S TSMA6 consists of a R&S TSME6 (Network Scanner) and an integrated computer. For the ease of reading, in the remainder of this document, it is named Network Scanner. The Network Scanner can perform measurements in a user-configurable range between 350 MHZ and 6 GHz.

The C/I scan speed of the Network Scanner is 720 channels/sec.

The CW scanner functionality used for coverage measurements allows to measure up to 254.000 GSM channels/sec. This high scanning speed allows to collect even at high speed (360km/h) at least a sample every 10cm. To be in-line with the procedure used for the TSML-CW as part of the GSM-R Drive Test System, the sample speed will be adjusted for each Autonomous Scanner system to collect at least one sample per channel every 10 cm at highest operational speed of the train on which the Autonomous Scanner system is installed. At lower speeds the sample interdistance will then be lower as 10 cm. By adapting the sample speed of each Autonomous Scanner system to the highest operational speed of the train, unnecessary increases of the logfile sizes are prevented.

In general, an Autonomous Scanner system will generate around 700MB of data per measurement hour.

Below an overview of the connections, buttons and lights on the Network Scanner is listed.

Figure 3-4 Network Scanner connections and buttons

For the default operation, next to Docking Connector 15, only connections 4, 12 and 14 are used, see also Figure 3-1.

The Network Scanner is equipped with an integrated computer running on Microsoft Windows (Windows 10 IoT Enterprise). Below some specs of the internal computer are specified:

- Intel® Core™ i7-7567U, 3.5 GHz processor

- 8 Gbyte DDR4 RAM

- 256 Gbyte HDD

For the connectivity options of the integrated computer, please refer to Figure 3-4

### Battery Pack

The Battery Pack contains two hot-swappable batteries and are used to ensure that a running measurement is stopped in a correct way when the power supply of the train is shutdown.

After stopping of the measurement, the Battery Pack also provides power during the transfer of the measurement data to the remote server. Therefore, the Battery Pack also provides power to the 4G modem. Below some specs of the Battery Pack are provided:

- 2x Li-ion batteries (R&S MNT-BP89WH)

  - Capacity: 5510mAh per battery

  - Nominal Voltage: 14.4V

- Batteries are placed in parallel; ensuring redundant Battery power supply (hot-swappable).

- Typical 3,5 hours autonomous usage (no AC power)

The Battery Pack is automatically charged when the AC power supply of the train is switched on.

Below an overview of the connections on the Battery Pack is listed.

Figure 3-5 Battery Pack connections

### Software

On the Network Scanner the R&S Romes software is installed, similar to the R&S Romes software as installed on the GSM-R Drive Test System. Because of this similarity, post-processing the measurement files for the Autonomous Scanner is similar to that of the GSM-R Drive Test System. Both can be performed using the Automatic Post-Processing Software (APPS), either installed on the Romes PC of the GSM-R Drive Test System or on the Landside Server of the Autonomous Scanner.

Uploading of the post-processed data via the Multi-Vendor Interface (MVI) to Nokia Performance Manager (NPM) is also the same between the two systems.

Next to the installed Romes software, the Autonomous Scanner offers additional software, capable of configuring the Autonomous Scanner system via a web-GUI. Access to the Autonomous Scanner system is possible with a data connection via the 4G modem, see 3.2.4

The Autonomous Scanner system is designed to automatically start with the measurements when the power supply on the train is switched on (auto start). When the train shift is finished and the power supply on the train is switched off, the Autonomous Scanner (which then runs on the Battery Pack) automatically, after a predefined time, stops the running measurements and switches off (auto stop). This can be configured via the web-GUI.

Figure 3-6 Auto on/off configuration

### Remote access

For remote access to the Autonomous Scanner system, the connected 4G modem is used to access the Autonomous Scanner system. A Remote Desktop connection towards the Autonomous Scanner system is made from a remote PC. After the connection is established, the Web-GUI can be accessed by using Internet Explorer. Via the Web-GUI changes to the configuration can be made or checks on the Autonomous Scanner system can be performed. Remote access to the Autonomous Scanner is password protected.

The used protocol for remote access is Remote Desktop Protocol (RDP). For the communication between the remote PC and the Autonomous Scanner system, TCP port 3389 is used. On the remote PC the Remote Desktop Connection app is used for accessing the Autonomous Scanner system remotely.

Remote access to the Autonomous Scanner system is only possible when the Autonomous Scanner system and the connected 4G modem is powered on. The remote access is possible because the 4G modem is provided with a dedicated IP-address together with an active IPSec VPN connection towards the ProRail LAN. See 3.3.2 for more details.

### 4G modem

The 4G (LTE) modem is used for the automatic upload of the measurement data, as gathered by the Autonomous Scanner system. The 4G modem also enables the possibility for a remote control or configuration of the Autonomous Scanner via the Web-GUI of the Autonomous Scanner system.

As mentioned in 3.2.1, each measurement hour generates in general 700MB of measurement data. This will result in 5600MB of measurement data per day, based on an 8-hour shift. If the average upload speed of the 4G connection is about 10Mbps, this would take almost 10 minutes to upload the full amount of measurement data.

The 4G modem connects to the Autonomous Scanner system via the LAN port of the Network Scanner, see Figure 3-2

The measurement data is uploaded to the Landside Server within the GSM-R Intranet environment. This server also contains the APPS software. More information about the Landside Server is provided in 3.3

The 4G modem which is supplied is an Advantech/Conel LR77 v2 Libratum SL. The modem has the following relevant specs:

- 2x SIM card slot

- 2x 10/100 Mbit/s Ethernet ports

- 2x antenna connection (1x antenna and 1x diversity)

- 1x DC power supply connector

Figure 3-7 4G modem

For more information regarding the specifications of the 4G modem, please refer to [4G modem Datasheet]

### AC/DC Power Supply

A 230VAC/12VDC Power Supply is used for powering the Autonomous Scanner system and for charging the Battery Pack. The Power Supply connects to the Battery Pack, via the DC In port, see Figure 3-4

### GSM-R antenna including GPS

For the coverage and C/I measurements a GSM-R antenna is installed on the roof of the train. This GSM-R antenna includes an integrated GPS antenna.

The GPS receiver allows to log the position related to the measurement data. With the logged positions, the APPS postprocessing software is able to recalculate the time-based triggered measurements to specific distances (e.g. 100-meter sections) and is able to map the measurement data to the closest section (e.g. 100-meter section) on the track.

When entering a tunnel, the GPS signal will disappear, which will be noticed by the R&S Romes software. Once GPS signal is recovered, linear interpolation is performed between last valid GPS signal and newly received GPS signal in the R&S Romes software. The same methodology is used by the GSM-R Drive Test System.

The GSM-R antenna connects with the RF In connector on the R&S TSMA6, see Figure 3-4

The GPS antenna connects with the GPS antenna input (SMA) on the R&S TSMA6, see Figure 3-4

For the specification of the antenna and the installation requirements, please refer to [Specifications Measurement Vehicle].

## Landside Server

In the GSM-R LAN a Landside Server is installed. This Landside Server contains the APPS software, which postprocesses the received Autonomous Scanner system measurement files. For the Coverage measurements, this post-processing is based on the agreed AEP95 calculation method.

Figure 3-8 Landside Server

This postprocessing is automatically started when the APPS software detects new Autonomous Scanner system measurement files. The output files, which are created by the APPS software, consists of Coverage and C/I data which is divided in 100m sections. So, for each 100m section the Coverage (based on the AEP95 method) and C/I data is processed. These output files are XML files and these XML files contain around 1MByte /day of measurement data for Coverage and another 1Mbyte/day of measurement data for C/I.

Note: exact output file format may be subject to change in the project phase.

The NPM MVI interface for the measurement train will check if there are XML files available on the Landside Server. If there are files available. These files will be automatically uploaded via the NPM MVI interface into NPM. For more information regarding the MVI interface and/or NPM, please refer to [SSDD CPM]

Based on a reference code for each Autonomous Scanner system into the R&S Romes measurement data file, the APPS software will automatically correct the measurement data with an offset to handle the train specific cable attenuation and antenna gain. This offset is configurable in the APPS software for each specific Autonomous Scanner system.

### Hardware

The Landside server is based on a HP server with the following specs:

- HPE DL360 Gen10 server

- Intel Xeon-S 4110 processor (8 cores, 16 Threads, 2.10 GHz)

- 32GB RAM

- 2x 4TB HDD in RAID 1 (mirroring)

- Microsoft Windows Server 2016

- Redundant Power Supply

The server has 4Tb of HDD capacity. Redundancy is provided by adding two identical HDDs in a RAID 1 configuration. In this way the content of both disks is constantly kept identical.

The specifications of the server are based on the following assumptions:

- Minimum storage size of 2TB

- Minimum storage capacity for RAW measurement files for 15 days

- Minimum storage capacity of the postprocessed files for 15 days

- Only GSM-R band is measured

- Average daily hours of Autonomous scanner measurement: 10

### Measurement data handling

The Autonomous Scanner is compatible with the APPS software, currently running on the measurement PC of the GSM-R Drive Test System.

For the Autonomous Scanner, this APPS software is installed on the Landside Server.

After each measurement run of the Autonomous Scanner system, the measurement file is uploaded to the Landside Station via the 4G modem. The Autonomous Scanner system uploads the measurement data via SSH File Transfer Protocol (SFTP) towards the Landside Server in the GSM-R Intranet. All traffic to the Landside Server is routed via the the ProRail LAN. -

The SFTP connection is realized by means of a dedicated IPSec VPN connection with a bandwidth of minimal 100Mbps between the KPN Mobile network (provider of the SIM cards) and the ProRail LAN. The SIM cards for the 4G modem are provided with a fixed IP-address per SIM card and are connecting towards a private APN within KPN Mobile, providing an encrypted data transfer between de Autonomous Scanner and the Landside Server.

For the SFTP connection between the Autonomous Scanner system and the Landside Server, TCP port 22 is used.

See Figure 3-9 for an overview of the connectivity details of the Autonomous Scanner system.

Figure 3-9 Autonomous Scanner connectivity infrastructure

In order not to have big data uploads after each measurement run, the running measurement starts with a new file after 30 minutes, causing a small interruption of a few seconds, before continuing with a new measurement. Therefore, after 30 minutes one measurement file exists, and this file is then uploaded automatically to the Landside Server. Because each file is stored as a ZIP file, this will result in one file of around 17 Mb per 30 minutes. This will then also result in one data transfer of around 17 Mb per 30 minutes towards the Landside Server.

After the power supply is switched off, somewhere between 1 and 30 minutes afterwards the final file is then also stored as a ZIP file and then also uploaded towards the Landside Server. A new measurement is then started until the Autonomous Scanner system is powering down (40 minutes after the power supply is switched off). The measurement data of that interrupted measurement is uploaded the next time the Autonomous Scanner system is switched on again, but this measurement data is discarded by the Landside Server.

# Unattended Probes (UAP)

## Unattended Probe solution

The on-board probe is a standard 3U 19” equipment which is installed in trains. The main tasks of the on-board probe are to execute measurement scenarios, record the measurements, compute various metrics and KPI’s depending the running scenario and finally to periodically transmit the metrics to the Landside server.

The on-board Unattended Probe is housed in a 19’’ sub-frame as visualized in Figure 4-1

Figure 4-1 Expandium on-board Unattended Probe

Figure 4-2 represents the Unattended Probe system architecture.

The on-board probe automatically records all measurements and computes various metrics and KPIs related to a measurement scenario. For measurements performed data is being logged to be uploaded to the Landside Server.

To communicate in point-to-point with the on-board probes the Landside server uses ISDN connection (Ifix CS) to the MGW. The Landside server objective is to collect and send UAP metrics to the central QATS server and to download on-board probe results. The central QATS server correlates Landside Server received data from on-board probes with data’s collected by network interface probes.

The Landside server also uses IP connection to the GSM-R Packet Switch core network Gi/Ifix PS. The function for this connection is to manage transactions and data so Landside is able to test “IP RBC” procedures by answering UAP PS transactions.

SMS management is used in order to play scenarios and manage the pool of probes over the GSM-R network. Up to 15 on-board probes with each 2 modems can function simultaneously using a single ISDN PRI E1. Different call scenario’s may be sent to the UAP’s which will be executed according the time constraints.

QATS Drive test will automatically collect and process call data from the UAP’s and generate the defined KPI’s.

Figure 4-2 Unattended Probe system architecture

Further information can be found in [QATS Unattended Probes Product Description].

### Monitored interfaces

The fix side monitored interfaces remain the same as currently implemented for QATS Railway and QATS Signalling. Onboard train monitoring of Um interface is added.

The drawing below shows the common interfaces monitored and Landside probes added on both Core locations.

Figure 4-3     Monitoring overview with UAP elements added

### Solution dimensioning

The UAP solution exists of the following equipment:

- 5x Onboard Unattended Probes equipped with 2 GSM-R modems

- 2x Landside server ISDN connected

- 1x SMS Server

- QATS Drive Test software module installed on QATS Railway servers

- Onboard antennas and associated cables

### IT configuration

All equipment connected to the LAN uses 3 physical connections and consequently 3 IP addresses.

The VPN enabling access to QATS equipment accomplishes the UAP elements.

Information exchange between Landside and Dataweb servers is done via the LAN.

Figure 4-4 IT configuration detailing Unattended Probe system

### Connectivity UAP Landside

UAP operation is managed remotely by the end-user via a graphical user interface available when logged in to QATS Drive Test. Exchanges of UAP logged data is possible via GSM-R CSD calls or via public operator 3G/4G IP connection between Landside and Unattended Probe.

Depending on the features used, GSM-R CSD or 3G/4G data transfer is used.

As the NL GSM-R domain is not connected to public Internet, a dedicated TCP IP connection between the Unattended Probe and Landside server is used.

Both domains GSM-R/4G will be clearly separated and security is managed on the workstation. Connection will be done over a specific railway operator APN-VPN gateway, facilitated from the ProRail KA domain. Both domains GSM-R/4G are clearly separated and security is managed on the Landside server.

The Landside/UAP connection operation, prerequisites and security is further detailed in [3] Unattended Probe landside connectivity.

### UAP Data logging

The UAP local data logging depends on used features (see table) for the GSM-R modems. Storage capacity of the UAP is limited to 32GB but contains also OS and Software installed. The available storage capacity will limit the maximum of daily upload file size.

Data logging speed will be between 100 and 300 Kb/s when Modem Traces feature is used. The average daily file for 8h scenario run is 4,32Gb. If the average upload speed of the 4G connection is about 10Mb/s, it would take about 7,2 minutes to upload the full amount of measurement data.

| Feature | Data logging average (Kb/s) | Uploading |
| --- | --- | --- |
| Status and Fault management | 6 Kb | Batch upload every 10 minutes via 3G/4G |
| Tracking and tracing | 150 bytes/second (2 modems) | Realtime via 3G/4G |
| Congestion management | Uses Tracking & Tracing data (no additional data logging) | 3G/4G |
| Remote Maintenance | 750Mb for minor (monthly) releases 3Gb for major (1/2 year) releases | 3G/4G |
| Modem Traces (future functionality) | Activity based: 100Kb-300Kb | 3G/4G |

### Archive rules

The UAP archiving rules will be in line with general QATS archiving.

| Item | Duration |
| --- | --- |
| Raw capture files on Processing & Dataweb server | 3 months |
| Raw UAP capture files on Landside server | 3 months |
| Details on Reporting server (CDR + access to call trace) | 6 months |
| Aggregated data in database (KPI) | 1 year |

Table 4-1 Archive rules

## QATS Drive Test

The user interface is based on QATS Drive Test solution offering a range of user needs such as:

- Enabling user to remotely access drive test campaign through web-based user interface

- Detecting areas where coverage may not be compliant or find coverage degradation

- Merging downlink on-board data with uplink Abis measurement report (for CSD)

- Measuring establishment times

- Comparing data with connected call measurement reports

- Saving all trace files on a central Reporting server

The principle of the QATS Product Suite user interface is to access easily the information using relevant medias and to browse naturally the information in details (by 'drilling-down') as represented in the following drawing.

Figure 4-5 Drill-down principle in QATS Product Suite

QATS Drive Test Graphical User Interface (GUI) capabilities include:

- Sorting a selected column on field values

- Filtering a field value using the condition editor

- Copy and paste field values to the clipboard

- Displaying/hiding columns

- Displaying end-to-end call tracing related to any selected CDR

- Displaying message decoding

- Exporting CDRs list to an Excel spreadsheet

When displaying KPIs mostly, the following capabilities are possible:

- Zooming in or out with the mouse

- Drilling-down by right-clicking toward other features or focusing on detailed data including breakdowns

- Rolling-up from detailed data to summary information is also possible on some cases through the right-click options

- Displaying exact values for any graph dots the dynamic legend

- Selecting different graph parameters (e.g. aggregation period, KPI type, etc) using combo-boxes

Figure 4-6 Zooming & Panning

Further information can be found in [QATS Drive Test Product Description].

### Scenario management

Figure 4-7 Probe scenario management

The probe scenario management allows different call scenario’s to be send to the UAP’s which will be executed according the time constraints. There is no limitation in terms of scenario duration. As soon as the UAP is started, the actual scenario will be running. The scenario are seen and edited via the dedicated man machine interface.

Scenario management is performed via SMS messaging over the UAP GSM-R radio modems and CSD calls.

Further information can be found in [QATS Unattended Probes Product Description].

### Status and Fault management

Status and Fault management is to support the operational process. UAP’s will be used continuously, only switched off when train power is down. Status SMS’s are implemented to signal UAP status and to initiate service action in case of a defect.

A user interface is available to display those information’s. Information’s and status are sent through GSM-R SMS and 3G/4G bearers.

1) UAP management, current status sent through GSM-R SMS

This information will be displayed:

- power on/off

- service call: scenario is to be uploaded, file results are downloaded

- modem is running a scenario (details viewed in the scenario management window)

2) UAP management, sent through 3G/4G bearers

This information will be displayed:

- CPU temperature

- disk space

- memory usage

- GPS fix status

- battery balancing anomaly

### Tracking and tracing

For track specific scenario management, tracking and tracing of UAP in (near) real time is available. Different solutions are possible:

#### Autonomous (automatic) scenario management

A user-interface will describe scenario (call type, timeout, …) and geographical areas association and link (maximum 5 areas). The scenario/geographical areas links are uploaded to the Unattended Probes. As soon as the UAP crosses a scenario’s border, autonomous recognized via UAP GPS tracking, the probe automatically switches to the relevant scenario step/action. These features are integrated into QATS Drive test.

#### Controlled (manual) scenario management

A GIS system displays UAP location in near real time on a unique map. By clicking on an UAP, user can open the management interface to see / change / edit scenario. This dynamic control for scenario management is available in combination with tracking and tracing features.

For tracking and tracing a high data rate IP connection (3G or 4G) between the UAP and the Landside is used to transmit probe location in real-time.

Tracking and tracing to support controlled scenario management is part of the UAP licenses.

### Congestion management

To avoid GSM-R network congestion by traffic generated by UAP’s when located in the same area, location/cell information are sent from UAP to the central landside server. This is done by using the 3G/4G bearer. If number of modems that are using same combination (LAC/CI/Frequency) is more than a threshold (typically: 1 UAP (2 modems)) latest UAP is set on “Pause” mode.

With this method only a short period of time 1 or 2 additional modems will be active in defined LAC/CI/Frequency. The pausing time period will be few seconds, to send LAC/CI/Frequency to central server and to receive pause message by the Unattended Probe.

### Remote Maintenance

To reduce need of maintenance visits to probe trains the UAP’s support remote maintenance and software updates via 3G/4G connectivity. Remote maintenance may be performed by Expandium support under strict Mobirail S&B supervision and planned work procedures.

Regarding software upgrades a security procedure prevents for unwanted access:

- A SMS is sent to the probe with a command to be executed. The SMS requests to open a Reverse SSH with encapsulation. The SMS has an encrypted signature.

- The Unattended probes analyses the SMS, checks the signature and if OK, executes the Reverse SSH to the Expandium dedicated support server.

- Expandium dedicated support server connects to the probe with its login and password.

- New software packages are sent to the UAP via safe transfer protocol (scp, sftp, etc.). Once packages are available on the probe memory disk, the maintainer will connect to the probe via SSH using the 4G connection.

- Once logged within the system, remote support starts the upgrade “manually” by upgrading packages that need to be updated within a virtual terminal to avoid disconnection problems.

## Proposed scenario’s

The Unattended Probe system will consist of a number of on-board probes integrated on trains that periodically will drive on all main tracks and allow periodic reporting on KPI’s of generated traffic.

Up to five (5) measurement trains are equipped with Unattended Probes. The operation of these trains should offer sufficient track coverage to support periodic reporting.

In next chapters scenarios are proposed for usage of the probes in line with the Capacity & Performance needs.

### Radio coverage measurements

One purpose of the probe system is to perform continuous radio coverage and quality measurements enabling production of a periodic radio coverage report.

This will be reached by using on-board probes equipped with a GSM-R modem meant for this dedicated use.

A recurring scenario will be used to set-up a Circuit Switched Data call as soon the on-board probe is powered on.

The CSD call will be set-up and continued as long as possible but with a number of restrictions:

- The CSD call will be stopped when the train is stopped and is not moving for 15 minutes

- The CSD call is stopped according to congestion management rules when a probe train enters an area/cell with active probe the CSD call of one of the probes will be stopped

The probe system will measure radio coverage on a continuous base. Rxlev and Rxqual uplink signals are captured by QATS Railway. The Rxlev and Rxqual downlink signals are logged at the on-board probes together with the GPS positioning information. The on-board probe will transmit these metrics in a CSD call to the central server. Transmission of logged data and processing are done on demand, at scenario management or at every system power-on. The logged information of one day will be transmitted in about 15 minutes with CSD calls.

### Derived KPI fields

The table shows KPI fields generated by the Unattended Probe system which contribute to regular monitoring because they either:

- Correlate DL and UL by means of QATS Drive test

- Detect any abnormal radio behaviour of cell or train with QATS Reference analytics

- Extend the coverage of this KPI in time/space compared to Measurement train

- Complementary to coverage measurement made by measurement train and autonomous scanner do increase the "coverage" data in terms of space and time covered

- Offer the possibility to investigate with full details the cause(s) behind any radio coverage issue

| Scenario | KPI field |
| --- | --- |
| Main scenario Radio Coverage | Coverage & Quality |
| ETCS o CSD | ETCS o CSD |
| Main scenario Radio Coverage | Handover |

#### RXLev & RXQual

For calls performed by on-board probes, DL RxLev, UL RxLev, MS PWR, BS PWR, cell info and associated time will be picked up from data captured by existing QATS probes and sent to the central system. Abis measurement reports (CSD only) are preferred to on-board measurements as they enable to take into account Power control.

Coordinates (X, Y) and associated time will be captured by the on-board probes and sent to the central system. Coordinates will be associated to radio measurement samples using interpolation.

Radio problem count is the aggregation of behaviours that are classified in 3 classes:

Interference:

- UL RxLev > -85 dBm

- UL RxQual > 4

- DL RxLev > -80 dBm

- DL RxQual > 4

Weak coverage:

- UL RxLev < -100 dBm

- UL RxQual < 5

- DL RxLev < -95 dBm

- DL RxQual < 5

Lack of coverage:

- UL RxLev < -100 dBm

- UL RxQual > 4

- DL RxLev < -95 dBm

- DL RxQual > 4

Default threshold values can be modified on demand and may need fine-tuning based on learnings.

As a second set of features, a coverage map is built with all measurements reports samples collected by the QATS Probes The map / reporting with coverage measurement is made with aggregated measurements per position e.g. geohash or customer network definition resolution – for example 100 meters thanks to the GPS of QATS probes.

Furthermore, as a third set of features, QATS Reference analytics or comparison to reference measurement is used to provide results like abnormal variation of cell or train behaviour which cannot be easily detected by other means.

#### Secondary scenario’s

Besides the Unattended Probes main scenario for radio coverage measurements a set of secondary scenario’s will have important contributions for deriving CPM KPI’s.

The secondary scenarios can be performed by an on-board probe with a single GSM-R modem when a concession is done to perform continuous coverage measurements. By using on-board probes equipped with two GSM-R modems, the main scenario and secondary scenarios can be executed at the same time.

Call setup scenario

This call setup scenario enables call setup generation while duration of calls can be configured. By performing this call scenario along the tracks call success rate and connection loss rate figures will be gathered.

| Scenario | KPI field |
| --- | --- |
| Call setup scenario | Call setup / Connection loss |

GPRS scenario

The GPRS scenario enables GPRS features and collection of ETCS over GPRS KPI’s. The table below shows the KPI fields which can be derived. A number of additional (EXP) KPI’s will have a meaningful contribution to the CPM Chain.

| Scenario | KPI field |
| --- | --- |
| GPRS CSD scenario | GPRS features, ETCS over CSD |
| GPRS PS scenario | GPRS features, ETCS over GPRS |

Voice quality scenario

This voice quality scenario enables call generation according to PESQ standard.

| Scenario | KPI field |
| --- | --- |
| Voice scenario | Voice quality PESQ |

## Hardware dimensioning

### On-board Unattended Probe

### UAP Hardware

The main hardware components of the on-board Unattended Probe unit are:

19" probe module, 3U height

Up to 2 x GSM-R modem (Triorail TRC-5RM-Ext)

1 maintenance modem 4G/3G/2G

1 CPU card

1 Dead Reckoning GPS

1 AC/DC power supply

1 UPS (uninterruptible power supply) based on capacitor batteries

All HW listed above is certified:

- Low Voltage Directive, 2006/95/EC

- EMC Directive, 2004/108/EC:

- R&TTE Directive, 1999/5/EC:

- Further information can be found in [QATS Unattended Probes Hardware User Manual].

### UAP Software

The main software components of the on-board probe enable it to:

- Executing various scenarios

- Acquiring automatically all the measurements

- Transmitting periodically the metrics to the central server

- Managing train power supply switch-off to avoid dropped calls

Unattended Probes require operation of QATS Drive Test to be installed on QATS Railway Dataweb servers. QATS Drive Test and the Landside probes configuration are described in SSDD QATS v2.53+. Further information on QATS Drive Test can be found in [QATS Drive Test Product Description].

### UAP - UPS power supply

By automatically detecting when train power supply is switched-on, on-board power supply system starts Unattended Probe activity without any human intervention.

The unattended on-board probe backup battery technology is based on capacitive batteries allowing less maintenance.

By automatically detecting when train power supply is switched off, the on-board power supply system avoids dropped calls on the network. Active scenarios are stopped and hardware disconnected before entering in stand-by mode.

The power supply module is designed to power autonomy for more than four (4) minutes. The sequence to stop active scenarios and enter stand-by mode is fifty (50) seconds. This 50 seconds period detects a train being powered off, stop on-going scenarios then to power-off UP’s without creating any dropped calls or failure. The UPS system also manages very short and multiple power-offs (less than 3 seconds) to ensure a seamless operation.

### UAP software upgrade

In case of 4G disconnection, the “screen” terminal still running and then the upgrade continue even without SSH connection. Once the connection is available again, the maintainer will retrieve the terminal as if no disconnection.

In case of power off during the upgrade (usually we check it with the purchaser before but if it appends), as we use standard Linux Package installer (deb via apt), the system will keep the previous package version and we will have to restart the upgrade process after next power on.

### UAP Data Export functionality

The UAP system is primarily controlled and used via QATS Drive Test user interfaces. For integration to NPM an UAP Data Export mechanism is used. Data tables related to defined UAP KPI’s needed to support CPM are exported by means of a TSV file mechanism. The TSV files are exported to the 15’ Reporting Server (R1) part of the QATS Landside Server environment.

More details about the 15’ Reporting Server can be found in [SSDD QATS v2.53+].

Details of the QATS CPM-NPM Interfacing can be found in [QATS CPM-NPM Interfacing].

Details of TSV file tables structure can be found in [QATS Reporting Drive Test Database Description].

# References

| Specifications measurement vehicle | Specifications measurement vehicle.docx |
| --- | --- |
| Floor plan RTD | GSM-R Floorplan Rotterdam.pdf |
| Floor plan ASD | GSM-R Floorplan Amsterdam.pdf |
| Rack plan ASD | Rackplan Phase2 ASD2A.xlsx |
| Rack plan RTD | Rackplan phase 2 Rotterdam.xlsx |
| SSDD RAS | SSDD_Remote_Access_System.doc.docx |
| NIT-Cov | NIT-Cov – vx.x.docx |
| NIT-C_I | NIT-C_I – vx.x.docx |
| SSDD CPM | Sub-System Design Document CPM |
| R&S TSMA6 Product Brochure | TSMA6_bro_en_5215-8241-12_v0400.pdf |
| 4G modem Datasheet | conel_ur5iv2_lr77v2_libratum_datasheet.pdf |
| SSDD QATS | Sub-System Design Document QATS |
| QATS Unattended Probes Product Description | EXPDM0816 MK QATS Unattended Probes Product Description v1.2.2 |
| QATS Record Unattended Probes User Manual | EXPDM0879.MK.QATS Record Unattended Probes User Manual v1.2.2 |
| QATS Drive Test Product Description | EXPDM0761 MK GSM-R QATS Drive Test Product Description v1.5 |
| QATS Unattended Probes Hardware User Manual | EXPDM0990.MK.QATS Unattended Probes Hardware User Manual v1.2.4 |
| QATS CPM-NPM interfacing | EXPDM1608.PS.QATS CPM-NPM interfacing v1.1 |
| QATS Reporting Drive Test Database Description | EXPDM1322.CC.QATS Reporting Drive Test Database Description v1.3 |

Appendix A: Abbreviations

APN	Access Point Name

APPS	Automatic Post-Processing Software

CPM	Capacity and Performance Management

CSSR	Call Setup Success Rate

CW	Continuous Wave

DQA	Data Quality Analyzer

ETCS	European Train Control System

ERTMS	European Rail Traffic Management System

GPS	Global Positioning System

HPE	Hewlett Packard Enterprise

IPSec	Internet Protocol Security

KPI 	Key Performance Indicator

LTE	Long Term Evolution

MGW	Media Gateway

MOC	Mobile Originating Call

MS	Mobile Station

MSS	Mobile Switching centre Server

MTC	Mobile Terminating Call

MTM	Mobile to Mobile call

MVI	Multi-Vendor Interface

NPA 	Network Problem Analyzer

NPM	Nokia Performance Manager

QDT	QATS Drive Test

QR	QATS Railway

QS	QATS Signalling

QoS	Quality of service

RBC	Radio Block Centre

RDP	Remote Desktop Protocol

REC	Railway Emergency Call

R&S	Rohde & Schwarz

SFTP	SSH File Transfer Protocol

SSH	Secure Shell

SQA 	Speech Quality Analyzer

UAP	Unattended Probe

Ue	User Equipment

VPN	Virtual Private Network

Appendix B: List of licenses for GSM-R Drive Test System

Appendix C: Calibration for GSM-R Drive Test System

A calibration contract is in place for the system, including:

- Calibration of the scanners (TSML-CW & TSMW) by Rohde & Schwarz

- Verification by Mobirail of the radio modules as part of the system

- Verification of the Measurement train antenna system

Calibration of the scanners

The scanners TSML-CW and TSMW will be sent yearly to Rohde & Schwarz for calibration. 
A calibration report will be provided.

Verification of the radio modules

A real calibration service does not exist for the GSM-R modules as part of the system; however a yearly verification of the radio modules will be executed by Mobirail. If during this verification issues are noticed, the corresponding unit would be replaced.  
Possible issues:

- No correct operation of the module

- Degraded output power

- Rxlev reporting not corresponding to the measured downlink signal

- Rxqual not corresponding to C/I measured

A report will be provided.

Verification of the measurement train antenna system

Yearly check

Every year, the following will be verified:

- Feeder cables losses

- State of the feeders

- Visual inspection of the antennas on the roof

A report will be provided.

Biennial verification

A complete verification of the antenna patterns will be executed every 2 years, and a report will be delivered showing the actual antenna pattern for each antenna, as measured.

Appendix D: List of NIT for GSM-R Drive Test System

Here-below is the list of NIT that are measured with the GSM-R Drive Test System.

- NIT-CSSR+CST-REC: 	Call setup time & CSSR (REC)

- NIT-CSSR+CST-MOC: 	Call setup time & CSSR normal voice call MOC

- NIT-CSSR+CST-MTC: 	Call setup time & CSSR normal voice call MTC

- NIT-CSSR+CST-MTM: 	Call setup time & CSSR normal voice call MTM

- NIT-CSSR+CST-VGC: 	Call setup time & CSSR group call

- NIT-S093-CLR:		Connection loss rate

- NIT-C_I:		C/I Downlink

- NIT-COV: 		Coverage

- NIT-Lev-Idle-R: 		MS received level in idle mode

- NIT-Lev-Dedi-R: 	MS received level in dedicated mode

- NIT-Qual-R: 		MS received quality

- NIT-HOTime-R: 		Maximum interruption time at handover

- NIT-HOSR-R: 		Handover success rate

- NIT-MOS: 		Speech quality

- NIT-S093-NetReg: 	Network registration delay

- NIT-S093-CED+CEER: 	Call setup time data call

- NIT-S093-TD: 		Data transmission delay

- NIT-S093-TI+EFP: 	Transmission interference & error-free period

- NIT-GI:			Gebiedsindeling

History

| Version | Date | Author | Short description of changes |
| --- | --- | --- | --- |
| 0.1 | 20-04-2015 | Alexandre Amant | First draft |
| 0.2 | 11-05-2015 | Alexandre Amant | Internal review |
| 0.3 | 11-06-2015 | Alexandre Amant | Incorporated ProRail review comments |
| 0.4 | 27-07-2015 | Alexandre Amant | Incorporated ProRail & KPN review comments |
| 0.5 | 18-09-2015 | Alexandre Amant | Incorporated ProRail & KPN review comments |
| 0.6 | 16-10-2015 | Alexandre Amant | Incorporated ProRail review comments |
| 0.7 | 09-12-2015 | Alexandre Amant | Added jumper cable losses |
| 1.0 | 30-03-2016 | Alexandre Amant | Final release |
| 1.1 | 21-04-2016 | Alexandre Amant | Modified NIT naming in Appendix D |
| 1.2 | 27-04-2016 | Alexandre Amant | Added information about the Pegasem radar |
| 1.3 | 05-04-2019 | Richard Trip | Incorporated the Autonomous Scanner and the Unattended Probes Renamed to SSDD Measurement Systems |
| 1.4 | 21-05-2019 | Richard Trip | Incorporated review comments of ProRail. Incorporated the changes of the SSDD Drive Test System versions V1.0 - V1.2 |
| 1.5 | 05-07-2019 | Richard Trip | Incorporated review comments of ProRail and KPN S&B. |
| 1.6 | 18-07-2019 | Richard Trip | Incorporated review comments of ProRail. |
| 1.7 | 13-12-2019 | Richard Trip | Incorporated review comments of ProRail. |

Connectivity 3G/4G need to be detailed in a workshop for UAP as well Scanner solution. The SSDD document will be extended when solution details are known.