# Sub-System Design Document

# CPM

Table of contents

1	Introduction	3

1.1	Scope	3

1.2	Guide to document	3

2	CPM overview	4

2.1	Design criteria and decisions	4

2.2	CPM Tooling Landscape	4

3	CPM Architecture	6

3.1	Nokia Performance Manager	7

3.1.1	NPM Components	8

3.1.2	NPM Real-time performance Management	10

3.1.3	NPM Reporting Suites	10

3.1.4	NPM Access	11

3.1.5	NPM MVI Architecture	12

3.2	NPM Interfaces	13

3.2.1	NetAct	13

3.2.2	Netviewer	14

3.2.3	Radio Commander	15

3.2.4	MVI QATS	15

3.2.5	MVI Measurement Tools	20

3.3	Nokia APS	23

3.3.1	APS Software	24

3.3.2	TSV/XML Conversion Software	26

3.3.3	3rd Party File Handling	26

4	Components	27

4.1	HPE BladeSystem c7000 Enclosure	29

4.1.1	HP BL460c Gen9 Blade Server	29

4.1.2	HPE Brocade 8Gb SAN Switch	30

4.1.3	HPE Virtual Connect Flex-10/10D Module	30

4.2	Dell EMC VNX Storage	31

4.2.1	Dell EMC VNX5400 Primary Storage	31

4.2.2	Dell EMC VNX5200 Backup Storage	32

4.3	HPE 5510 24G 4SFP+ HI Switch	32

4.4	APS HW	33

5	Glossary	34

5.1	Referenced documents	34

5.2	Abbreviations	34

5.3	History	36

6	Appendix	36

6.1	NPM version	36

6.2	Hardware	36

6.3	Software for Hardware components	37

6.4	Software and Licenses for NPM	38

# Introduction

## Scope

This document describes the Capacity and Performance Management (CPM) solution of the Dutch GSM-R network. Since the Nokia Performance Manager (NPM) is the central part of the CPM Tooling, this document mainly describes the NPM system and all interfaces towards the NPM system.

## Guide to document

Chapter 2 gives an overview of the CPM solution

Chapter 3 describes the System Design of the CPM solution.

Chapter 4 describes the components of the CPM solution.

The design described uses the following terms:

- Rt-Wah as defined in [SDD] for core site Rotterdam Waalhaven;

- Asd-RAI as defined in [SDD] for core site Amsterdam RAI;

- Site OMC or OMC site as defined in [SDD];

# CPM overview

## Design criteria and decisions

The Dutch GSM-R network is a comprehensive network and has to meet high availability requirements and deliver an outstanding performance.

To be able to guarantee the end-users Quality of Services (QoS), the Capacity and Performance Management solution is used in order to be able to interpret and present the key performance data (KPI’s) of the network, based on the collected data (Performance indicators via counters) of the underlying systems, like NetAct, Netviewer and Radio Commander. Also, the data gathered by the Drive Test System, Autonomous Scanners, QATS UAP’s and the 3rd Party UAP’s are uploaded to NPM via the Multi-Vendor Integration (MVI) Measurement Tools interfaces. The Performance indicators of the live traffic are uploaded to NPM via the MVI QATS.

A central part in the Capacity and Performance Management Solution is the Nokia Performance Management system (NPM).

## CPM Tooling Landscape

At the heart of the CPM tooling the NPM system is used for the creation, storing and reporting of the KPI’s, and associated reportings. The NPM system is located in Rt-Wah. For more details regarding this location, please refer to [SSDD Sites].

See Figure 2-1 for an overview of the CPM tooling landscape, which shows the NPM system as well as the different systems which are connected to the NPM. These systems are uploading all necessary data towards NPM. The uploading of this Raw data takes place with an interval of 15 minutes, as explained in chapter 3.1.1.2.

Two MVI’s are in place, the MVI QATS and MVI Measurement Tools, to be able to support the upload of all QATS data, captured via the network interface probes,  and the data coming from the various measurement tools, like the GSM-R Drive Test System, the Autonomous Scanners (AS), the Unattended Probes (UAP) and the UAPs of a 3rd Party.

Figure 2-1 System overview CPM

# CPM Architecture

The CPM solution is designed to have an important role in the following three management related tasks:

- Capacity Management (CM)

- Performance Management (PM)

- (Keten) Incident Management (KIM)

In order to support these three management tasks, special Tooling equipment is needed. The NPM system plays a central role to support these three management tasks. The NPM gathers all relevant info from:

- NetAct

- Netviewer

- Radio Commander

- QATS Railway/Signalling (via the Multi-Vendor Integration module)

- Measurement tools, like AS and (3rd Party) UAPs (via the Multi-Vendor Integration module)

## Nokia Performance Manager

The NPM software architecture is based on a modular structure. The following figure provides an overview of NPM’s modular structure.

Figure 3-1 Nokia Performance Manager High-level architecture

The PM portal is the main entry point providing central access to all modules inside the NPM system and enables as such a quick access to all network information. The NPM tool doesn’t allow direct access to the underlying tools.

The reporting layer consists of several modules:

Pre-defined content packages including useful KPIs and reports for all needed technology areas.

Advanced Content packages provides KPIs and dashboards based on multi-vendor experience and the combination of multiple technologies.

Ad-hoc reports can be created, out of regular network counters, pre-defined KPIs or custom made KPIs.

The Dashboard is used for combining and showing several independent report (e.g. KPIs) panels in one online overview. This Dashboard online overview (webpage) can be showed on client PC’s within the GSM-R environment and via client PC’s which connect via RAS or via the ProRail GTN KA browser. Refer also to chapter 3.1.5

GIS reporting, as part of Reporting Suites, introduces specific map views, where KPIs and objects can be represented in a geographical way. The Product related Reporting Suites includes GIS reporting functionality on network element level. The ProRail Railway Reporting Suite, includes the possibility to report on 100m section, for the following KPI data, obtained via the GSM-R Drive Test System and/or the Autonomous Scanner:

- AEP95 Coverage data

- C/I Measurement data

For the KPI’s of which the KPI data is obtained via the GSM-R Drive Test System and/or the Autonomous Scanner and the UAP, a NIT document is created which describes and defines the reported KPI or a combination of KPI’s.

### NPM Components

There are four components in the NPM system:

- Data collection and processing

- Long term data storage and partitioning

- Self Management

- User Management

Figure 3-2: Nokia Performance Manager platform components

#### Data collection and processing

Data is collected from multiple systems and placed into one centralized data warehouse. If applicable (see the NIT documentation), the data is also processed in this step. See step 1 in Figure 3-3 above.

The data collection granularity is provided in [KPI lijst]. This KPI list e.g. specifies per KPI on which Object level the data for each KPI is reported and maps each KPI to a specific User Group or to multiple User Groups.

#### Long term data storage and partitioning

NPM provides long term storage (from months to several years) for raw and aggregated network performance historical data ready to be used in the calculation of Key Performance Indicators (KPI) and in the production of meaningful reports.

The NPM solution is dimensioned for a long-term period of 5 years, with the following aggregation scheme in days:

- Raw data – 30 days (1 month)

- Raw data – 720 days (2 Yr. Approx..) for ProRail Railway Reporting Suite

- Hourly – 90 days (3 months approx.)

- Daily – 365 days (1 Yr. approx.)

- Weekly – 720 days (2Yr. approx.)

- Monthly – 1825 days (5Yr. approx.)

Each reported value is directly written into the different Raw tables.  When the aggregation time is reached the data is aggregated in time and network level. Those results are stored in separate tables. As example reporting suites with 30 days retention period, will aggregate Performance Indicator  data at end of the month and store the monthly result in separate tables, and the raw table will be cleared to hold the raw data of the new month.

#### Self Management

Self Management functionality includes operating system and database resource supervision, as well as database clean up.

Alarms are reported towards NetAct for several reasons such as DB connectivity failures, hung aggregation processes, rejected files (due to topology mismatches, syntactic errors, among others) and aggregation process internal errors.

For more details, please refer to [NOKIA PM].

#### User Management

NPM supports single sign-on to all reporting applications. User authorization is configured on application level via the Permission Manager tool.

### NPM Real-time performance Management

The Real Time PM functionality provides online monitoring of the network resources. The network can be effectively monitored for bottlenecks using tables and charts that are updated in real- time (one minute granularity).

The Real Time PM functionality (one minute granularity) is available for the following network elements:

- NT HLR

- One- NDS

- Packet Core

The functionality is designed to analyze and optimize the overall network availability and efficiency. It allows to:

- Continuously monitor the quality of service

- Recognize early upcoming problems and bottlenecks

- Prevent situations leading to service outage and consequent loss of revenue

- Efficiently plan the usage of network resources.

The Real Time PM online monitoring is integrated seamlessly with the existing Nokia Performance Manager reporting tools. Real time views or reports can be created using the Report Creator wizard.

The maximum number of real time KPIs monitored at the same time in NPM is 200.

For more details, please refer to [NOKIA PM].

### NPM Reporting Suites

Product specific Reporting Suites are installed on the NPM system, providing Performance and Capacity KPI’s, per product.

Besides the product oriented Reporting Suites a Railway Service oriented Reporting Suite is installed for ProRail. The latter includes a number of KPI’s with associated graphs, allowing specific user group reports (“Diensten rapportages”).

Besides the pre-defined reports, flexible reports can be created by the users, allowing adaptation in terms of reporting period, reported KPI’s and/or graphical presentations.

As part of the NPM functionality the different reports can be exported manually or automatically on regular time intervals (i.e. monthly). The reports to be automatically exported and time interval are user configurable.

For ProRail there are 35 different reports defined which are generated automatically every month. These automatically generated reports are categorized in two categories: Diensten Rapportages and Capaciteit Management. These reports are further specified in [KPI lijst]

The exported reports will be stored on the NPM storage and can be further integrated into office documents.

### NPM Access

The NPM access is via a web based graphical user interface and applications based on Java 2 Enterprise Edition, via the existing RAS infrastructure or via the GTN network of ProRail.

Figure 3-3 NPM Access

The Client PC must meet the following minimum requirements:

- CPU: Intel Core i3 or equivalent.

- RAM: 4 GBs

- Java SE 7 or higher

- Microsoft Internet Explorer 10 or higher

- Firefox version 38.0 or higher

The NPM system can host up to 20 concurrent users.

### NPM MVI Architecture

For loading Performance Indicator data obtained from the QATS Railway and Signaling system, which continuously monitors the GSM-R network interfaces, and from the GSM-R Measurement Tools into the NPM system, specific integration modules are designed on the NPM system. These integration modules are based on NPM’s standard Multi-Vendor Integration Module framework.

The MVI Modules in the overall NPM architecture are visible in Figure 3-4.

Figure 3-4 NPM MVI Architecture

More details about the two available MVI interfaces for QATS and Measurement Tools can be found in the chapter describing the NPM interfaces, chapter 3.2.

#### NPM MVI Module framework

Performance Indicator data of different systems, with each a product/vendor specific format, can be loaded into NPM via the NPM Southbound interfaces. Specific system mediations are used for this purpose.

Figure 3-5 below illustrates the MVI module framework, composed of Mediation, Performance Management loading and Aggregation. The as such stored NPM PI-data is then available for reporting.

Figure 3-5 MVI - High Level MVI module overview

## NPM Interfaces

This chapter describes the interfaces towards the existing O&M systems and to the MVI QATS and the MVI Measurement Tools.

### NetAct

NetAct connects to the NPM system via the Intranet. The used protocol is a proprietary protocol.

NetAct is used to manage the following network components (of which the output data/files are important for the NPM system):

- BSS network Nokia (BTS, BSC, TCSM3i)

- EPC Core network Nokia (Flexi NG, Flexi NS)

- R4 Core network Nokia (MSS, MGW, CDS)

- NT-HLR (including One-NDS)

- Cisco and Juniper routers/switches

### Netviewer

Netviewer is used to manage the Flexi Packet microwave equipment. For details refer to [SSDD OMC]. The interface between NPM and Netviewer is a standard IP connection (Ethernet) via the Intranet. The used protocol is a proprietary protocol.

Figure 3-6 Netviewer interface

### Radio Commander

Radio Commander is used for managing the Siemens BSS network. For details refer to [SSDD OMC]. Radio Commander connects to NPM via the Intranet. The used protocol is a proprietary protocol.

### MVI QATS

This chapter describes the Nokia Performance Manager (NPM) Multi-Vendor Integration (MVI) Module for the integration of the QATS 15 minutes Reporting database [SSDD QATS] into the NPM system for Capacity and Performance Management (CPM).

#### NPM MVI interface for QATS

The connection between NPM and QATS is realized via a standard IP connection (Ethernet) over the GSM-R Intranet.

Figure 3-7: NPM-QATS database: interfacing

The MVI module will load QATS Performance Indicator data into the NPM database structure via SQL queries towards the 15’ Reporting Server DB (based on PostgreSQL, triggered from the MVI per 15 minutes interval in line with the QATS CPM Reporting update period).

These SQL queries will upload all new available data within the 15 minutes QATS database, since last upload into NPM, via SQL towards the NPM server.  This will be performed via PostgreSQL, whereby all data will be transferred to NPM via queries of the type SELECT * FROM table WHERE timestamp > LastLoadedTimestamp. As such clear split is kept between both systems, whereby each system performs maintenance, like archiving, etc. on its own database. The QATS datatables are described in [QATS 15’ Reporting].

#### QATS 15 minutes database

QATS 15 minutes PostgreSQL database is installed on the QATS R1 server. This is further described in [SSDD QATS].

For the connectivity of the NPM system towards the QATS 15 minutes database ODBC connectivity is used. A specific configuration file, stored on the NPM system and holding the ODBC database connectivity, is used from NPM side.

#### MVI Configuration files

The MVI QATS contains configuration data for the SQL query, like i.e. overview PLMN/BSC/BTS/Cell topology information, APN’s for e.g. Infoplus application.

This information is stored in the MVI QATS in specific Configuration files. These Configuration files are also adapted towards specific QATS MVI configuration files on the NPM system.

This approach allows for maximal flexibility in the CPM solution.

#### NPM Loading of QATS PI-data

The NPM mediation process will, after SQL-query, transform the QATS PI-data, enriched with for instance topology data, into specific OMeS (XML) files within the NPM system.

The as such obtained XML files are then parsed by the ETL (Extract, Transform, Load) engine into raw data tables that are accessed and further processed by the aggregation engine.  Aggregation (summarization) is done into object and time dimensions and is not further described in this document.

Figure 3-8: NOKIA Performance Manager PM Pipe

The NPM raw and aggregated tables, loaded via the NPM MVI interface are following the NPM design rules, allowing same analysis and reporting functionalities as available within NPM for Nokia products, and associated reporting suites.

#### MVI QATS Topology

The topology as indicated in Figure 3-9 will be used by the MVI for the mapping of the PI-data obtained from the QATS 15 minutes reporting database.

Figure 3-9: NPM Topology for MVI QATS

#### MVI QATS User Groups

The user groups as indicated in [KPI Lijst], will be used by the QATS MVI, for the mapping of Performance Indicator data obtained from the QATS 15 minutes reporting database. This latest reports in IMSI/MSISDN basis.

The QATS MVI performs the linking between the Performance Indicators collected from the QATS 15 minutes database and the different user groups. For this purpose, the QATS MVI will make use of an internal configuration file (see 3.2.4.3), based on the required reporting’s in NPM (user groups) and on the other side SIM overview data.

| 1 | ATB-VV |
| --- | --- |
| 2 | Conn Sec |
| 3 | HHT Hzl&BR |
| 4 | InfoPlus borden |
| 5 | Infoplus omroep |
| 6 | ITMC |
| 7 | Klokken |
| 8 | Lift Intercom |
| 9 | NSR cab sec |
| 10 | NSR cal |
| 11 | NSR int |
| 12 | Orbit |
| 13 | POSS |
| 14 | Quo Vadis |
| 15 | S&A Zuilen |
| 16 | TB AU CS |
| 17 | TB BR CS |
| 18 | TB Hzl CS |
| 19 | TB HSL CS |
| 20 | WvW Mon |
| 21 | Pb all ETCS |
| 22 | Pb all GPRS |
| 23 | Pb all SPV |
| 24 | Pb all |
| 25 | Pb all V&D |
| 26 | Pb Dienst andere V&D |
| 27 | Pb Dienst portofonie |
| 28 | Pb UAP |
| 30 | Pb Meettrein |
| 31 | Use Cases |

Table 3-1 MVI QATS User Groups

Note: An additional User group reporting as “Undefined” will be present in the MVI, holding the KPI’s of users which can’t be matched to a certain user group, due to missing information to perform the mapping to a user group.

### MVI Measurement Tools

This chapter describes the Nokia Performance Manager (NPM) Multi-Vendor Integration (MVI) Module for the integration of the GSM-R Measurement tools into the NPM system for Capacity and Performance Management (CPM).

The measurement PI-data to be uploaded is specified in a separate XML measurement tools specification, similar to Nokia’s OMeS, however adapted to measurement tools instead of network elements. The XML Measurement specification is described in [XML Measurement tools].

Besides the measurement systems as provided within the CPM scope and described in [SSDD Measurement Systems], the MVI allows integration of other measurement tools, like i.e 3rd party UAPs.

The measurement tools and associated interfacing to NPM is visualized in Figure 3-10.

Figure 3-10 Measurement Tools overview

The Mobirail delivered systems are described in the [SSDD Measurement Systems].

#### NPM MVI interface for GSM-R Measurement tools

The GSM-R measurement tools providing PI-data for NPM are classified as follows:

- Providing data via Nokia Automatic Post-processing Server (Nokia APS) platform towards the NPM platform:

  - Autonomous Scanners

  - GSM-R Drive Test System (ROMES)

  - QATS Unattended Probes (UAP)

  - Unattended Probes 3rd party

All mentioned measurement tools will provide PI-data via a project standardized XML file, allowing identical integration into the NPM system.

The NPM system will monitor for, via SFTP PUT method, uploaded files from measurement tools and handle the files for PI-data load into NPM as indicated in Figure 3-13.

Figure 3-13 MVI SFTP Put method

#### NPM Loading of Measurement tools PI-data

The received XML measurement files will be parsed by the ETL (Extract, Transform, Load) engine into raw data tables that are accessed and further processed by the aggregation engine.  Aggregation (summarization) is done into object and time dimensions and is not further described in this document.

Figure 3-14 NPM PM Pipe

The NPM raw and aggregated tables, loaded via the NPM MVI Measurement Tools interface are following the NPM design rules, allowing same analysis and reporting functionalities as available within NPM for Nokia products, and associated reporting suites.

#### MVI Measurement Tools Topology

The topology as indicated in Figure 3-15 will be used by the MVI for the mapping of the PI-data obtained from the XML files Measurement tools. On the left the standard topology is shown, while on the right the alternative topology is shown.

Figure 3-15 NPM Topology for MVI Measurement tools

## Nokia APS

The Nokia Automatic Post-processing Server (APS) consists of a server on which several applications are running. These applications are:

- APS Software for the automatic post-processing of the RSCMD data from the GSM-R Drive Test System (ROMES) and the Autonomous Scanner system

- TSV/XML conversion software for the QATS Unattended Probes (UAP).

- File handling for the 3rd Party UAP’s (not part of the current project scope)

Below a schematic overview is shown of the different parts belonging to the Nokia APS:

Figure 3-16 Nokia APS Schematic overview

All three applications are uploading the measurement data towards NPM via so-called XML files. These XML files are uploaded every 15 minutes and are divided in three categories:

- Range measurements

- Non-Range measurements

- Driving Direction measurements

The Standard Topology is based on the GSM-R network elements topology, as also described in 3.2.5.3. The XML file then contains measurement data differentiated on CID (Cell-ID) topology level. If a higher network topology is needed (e.g. BSC), this is internally handled by NPM.

Alternative topology is used when the ProRail Section (100m) information is also needed for the measurements. Then the XML file contains measurement data differentiated on SECTION (100m Section) topology level.

For more information, refer to [XML Measurement tools].

### APS Software

The APS Software is used for the automatic post-processing of all measurement data which is uploaded to the Nokia APS Server by the GSM-R Drive Test System (ROMES) and/or the Autonomous Scanner system.

The output of the post-processing consists of XML files which are then uploaded towards NPM, via the MVI Measurement Tools. These XML files do not only contain the relevant Performance Indicator data. Also, additional information regarding the Section, User Group and Topology (overview PLMN/Track/line/100m Section or overview PLMN/BSC/BTS/Cell) is added to these XML files, if needed. For adding this information, so called Lookup Tables are defined.

As described in 3.3, the output of the APS Software consists of XML files which are containing specific information such as:

- Measurement data

- User Group

- Standard or Alternative topology

- Date information

- Time information

The XML files are then uploaded to NPM via the MVI Measurement Tools, refer to 3.2.5.

#### Section Lookup Table

For those measurements which require alternative topology, the Section Lookup Table is used. This Section Lookup Table consists of the ”spoorbaanhartpunt”  file received of ProRail and contains the whole rail network of the ProRail, divided in to 100m sections. For each 100m section, the exact GPS location of the middle of each 100m section is stored in this Lookup Table, see Figure 3-17

Figure 3-17 Section Lookup Table principle

Based on the included GPS information in the measurement files, the APS Software is able to map the measurement data to the appropriate 100m section.

#### User Group Lookup Table

In NPM it is possible to create reports, based on several pre-defined User Groups In order to make it possible for the APS Software to select the right User Group, a User Group Lookup Table is available for the APS Software. This User Group Lookup Table is then used for the mapping of the Performance Indicator data to the correct User Group. The possible User Groups are listed in the tables below.  E.g. the mapping of a specific UAP to the correct User Group is done by checking the MSISDN number. For the Autonomous Scanner, this mapping is done, based on the filename of the RSCMD file.

This User Group info is then added to the XML data which is uploaded to NPM. In this way NPM can select all relevant Performance Indicator (PI) data for each specific User Group.

| UA* | Unattended Probe |
| --- | --- |
| AS* | Autonomous Scanner |
| MT* | Measurement Train |

Table 3-2 User Group Lookup Table

The user groups will be sub-devised to all reports per specific measurement tools:

Measurement Train

MTT (Tuning)

MTV (Validation)

Table 3-3: User Group Lookup Table Sub-division Measurement Train

Autonomous Scanner

AS0 - AS4

Table 3-4: User Group Lookup Table Sub-division Autonomous Scanner

Unattended Probe

UA0 - UA4 (QATS UAP)

UA5 - UA9 (UAP 3rd party)

Table 3-5: User Group Lookup Table Sub-division UAP

Performance Indicator data, without IMSI/MSISDN indication or invalid filename, will be mapped to a separate user group “Undefined” by the APS.

### TSV/XML Conversion Software

On the Nokia APS Server, next to the installed Nokia APS Software, a second software tool is installed. The output of the QATS Unattended Probes (UAP) consists of so called TSV files. As an XML-MVI interface type is available on NPM these TSV files need to be converted to the XML format.

To achieve this conversion a so-called TSV/XML conversion tool is developed.

The XML output for the TSV/XML Conversion Software is in fact similar to the XML output as provided by the APS Software. Also, the same Lookup Tables are in place as a reference, as is depicted in Figure 3-16.

See also [XML Measurement tools] and [QATS TSV Reporting] for more information.

### 3rd Party File Handling

Every 3rd Party UAP (or corresponding Landside Server) uploads the measurement files towards the Nokia APS Server in a designated directory. The measurement files must be of the XML format and therefore match the exact same format as required by NPM. Further details for this solution are out-of-scope for the CPM project.

# Components

The NPM system consists of several components which will be described in this chapter. First an overview of all the different components and the way these components are linked together.

Figure 4-1 NPM components

Below the rack layout of the NPM system is showed.

Figure 4-2 NPM rack layout

## HPE BladeSystem c7000 Enclosure

Figure 4-3 HPE BLc7000 Enclosure

This cabinet  provide space for al modular server, storage and interconnection equipment which is needed to support the NPM system. The main (relevant) NPM system components which are housing in this cabinet are listed below.

### HP BL460c Gen9 Blade Server

| Type: | Amount: |
| --- | --- |
| HP BL460c Gen9 Blade Server | 4 |

In order to support the NPM system, 4 BL460C Blade servers are installed. These servers are modular installed and together they are running the NPM system applications.

Figure 4-4 HPE BL460c Blade server

### HPE Brocade 8Gb SAN Switch

| Type: | Amount: |
| --- | --- |
| HPE Brocade 8Gb SAN Switch | 2 |

This switch interconnects the 4 HPE BL460c Blade Servers with the installed EMC VNX5x00 Storage solution. There are 2 HPE Brocade SAN switches installed at the backside of the HPE BladeSystem c7000 Enclosure.

Figure 4-5 HPE Brocade 8Gb SAN Switch

### HPE Virtual Connect Flex-10/10D Module

| Type: | Amount: |
| --- | --- |
| HPE Virtual Connect Flex-10/10D Module | 2 |

This module interconnects the NPM system with the installed HPE 5510 switches. There are 2 HPE Virtual Connect Modules installed at the backside of the HPE BladeSystem c7000 Enclosure.

Figure 4-6 HPE Virtual Connect Flex-10/10D Module

## Dell EMC VNX Storage

For the storage of all the data and databases the Dell EMC VNX storage solution is used.

### Dell EMC VNX5400 Primary Storage

| Type: | Storage: |
| --- | --- |
| Dell EMC VNX 5400 | 9 TB |

The primary storage is used for housing the databases and for the short-term storage of data. The primary storage offers high performance capacity with up to 35.000 IOPS (Input/Output Operations per Second).

The installed disks are redundantly installed using a combination of RAID 10, RAID 5 and RAID 6.

Figure 4-7 Dell EMC VNX5400

### Dell EMC VNX5200 Backup Storage

| Type: | Storage: |
| --- | --- |
| Dell EMC VNX 5400 | 24 TB |

The backup storage is used for housing the long-term storage of data. The backup storage offers a performance capacity with up to 1000 IOPS (Input/Output Operations per Second).

The installed disks are redundant installed using a combination of RAID 10, RAID 5 and RAID 6.

Figure 4-8 Dell EMC VNX5200

## HPE 5510 24G 4SFP+ HI Switch

| Type: | Amount: |
| --- | --- |
| HPE 5510 24G 4SFP+ HI Switch | 2 |

This HPE 5510 switch interacts between the NPM system and the Intranet of the GSM-R network.

Figure 4-9 HPE 5510 24G 4SFP+ HI Switch

## APS HW

| Type: | Amount: |
| --- | --- |
| HPE DL360 Gen10 Xeon-S 4110 (8C, 2.10 GHz) - 2x 16GB 2Rx8 PC4-2666V-R (RAM) - 2x HPE 4TB SAS 7.2K HDD (RAID 1) | 1 |

# Glossary

## Referenced documents

| SSDD Sites | Sub-System Design Document Sites |
| --- | --- |
| SDD | System Design Document |
| SSDD BSS | Sub-System Design Document BSS |
| SSDD OMC | Sub-System Design Document OMC |
| SSDD QATS | Sub-System Design Document QATS |
| NIT | Network Integration Test (NIT) documentation |
| KPI lijst | KPI lijst v6.31 draft |
| SSDD Measurement Systems | Sub-System Design Document Measurement Systems |
| XML Measurement tools | XML Specification GSM-R Measurement tools |
| SSDD SIM Overview | SSDD SIM and Network Subscriber Configuration |
| QATS 15’ Reporting | QATS 15 minuten Reporting document (via SSDD QATS) |
| QATS TSV Reporting | QATS DT/UAP TSV Reporting document (via SSDD QATS) |
| NOKIA PM | NOKIA Performance Manager Product Description |

## Abbreviations

| 3GPP APS ATB-VV AU BR BSC BSS BTS CM CPM CS DB DCN EIRENE EMS EPC ETCS ETL FTP GPRS GSM-R GTN HHT HLR HSL HZL IN IOPS IP KIM KPI KPN MGW MS MSC MSS MVI NIT NPM NSR OMC OMeS Pb PI PLMN PM POSS QATS QoS R&S RAID RAN REC S&A SFTP SGSN SIGTRAN SPV SQL SSDD SSH TB TDM VM UAP USSD VLR WvW Mon XML | 3rd Generation Project Partnership Automatic Post Processing Server Automatische treinbeinvloeding – Verbeterde versie Amsterdam-Utrecht Betuweroute Base Station Controller Base Station System Base Transceiver Station Configuration Management Capacity and Performance Management Circuit Switched Database Data Communication Network European Integrated Railways Radio Enhanced Network Element Management System Evolved Packet Core European Train Control System Extract, Transform, Load File Transport Protocol General Packet Radio Service GSM for Railways Generiek Toegangs Network Hand Held Terminal Home Location Register High Speed Line Hanzelijn Intelligent Network Input/Output Operations per Second Internet Protocol Keten Incident Management Key Performance Indicator Koninklijke PTT Nederland Media Gateway Mobile Station Mobile Switching Center MSC Server Multi Vendor Integration Network Integration Test Nokia Performance Management NS Reizigers Operation and Maintenance Center Open Measurement Standard Product beheer Performance Indicator Public Land Mobile Network Performance Management Preventief Onderhouds en Storingsdiagnose Systeem Quality Assurance and Troubleshooting Solution Quality of Service Research & Development Redundant Array of Independent Disks Radio Access Network Railway Emergency Call Service en Alarm SSH File Transfer Protocol Serving GPRS Support Node Signaling System No. 7 over IP Spoorveiligheid Structured Query Language Sub-System Design Document Secure Shell Treinbeveiliging Time Division Multiplex Virtual Machine Unattended Probe Unstructured Supplementary Services Visitor Location Register Wisselverwarming Monitoring Extensible Markup Language |
| --- | --- |

## History

| Version | Date | Author | Short description of changes |
| --- | --- | --- | --- |
| 0.1 | 03-04-2017 | Richard Trip | Initial document setup |
| 0.2 | 03-05-2017 | Richard Trip | Update due to internal review |
| 0.3 | 02-06-2017 | Richard Trip | Update due to review comment ProRail and KPN S&B |
| 0.4 | 06-07-2017 | Richard Trip | Update due to review comment ProRail and KPN S&B |
| 0.5 | 15-10-2019 | Richard Trip | Update due to NPM extension, ProRail Reporting Suite, MVI’s etc. |
| 0.6 | 17-10-2019 | Johan famaey | Small adaptations |
| 0.7 | 22-11-2019 | Richard Trip | Update due to review comment ProRail and KPN S&B |
| 0.8 | 12-12-2019 | Richard Trip | Update due to review comment KPN S&B |
| 0.9 | 10-11-2020 | Richard Trip | Update due to NPM SW upgrade to V19.10 |

# Appendix

## NPM version

| Product | Version | MTBF (h) | MTTR (h) | Availability (%) |
| --- | --- | --- | --- | --- |
| NPM | NPM 19.10 | - | - | - |

## Hardware

| Product Number: | Product Description: | Quantity: |
| --- | --- | --- |
| OSHW0334 | HP 642 42U i-Series Shock Rack 4PDUs | 1 |
| OEMHPHW5029 | HP LCD 8500 1U Console INTL Kit | 1 |
| OEMHPHW5033 | HP Nokia USB-A to USB-A 1m Cable | 1 |
| OSHW0241 | HP BLc7000 Enclosure | 1 |
| OEMHPHW3090 | HP BLc VC Flex-10/10D Module Opt | 2 |
| OEMHPHW3136 | HP BLc 10Gb SR SFP+ Opt | 4 |
| OEMHPHW3091 | HP BLc VC 1Gb RJ-45 SFP Opt Kit | 8 |
| OEMHPHW5019 | Brocade 16Gb/28c Embedded SAN Switch | 2 |
| OEMHPHW5021 | HP Premier Flex LC/LC OM4 2f 2m Cbl | 25 |
| OEMHPHW3092 | HP Ethernet 14ft CAT5e RJ45 M/M Cable | 25 |
| OSHW0308 | Cisco 2951 K9 Router | 1 |
| OEMHPHW3425 | HPE 5510 24G 4SFP+ HI Switch | 2 |
| OEMHPHW3426 | HPE 5500 150WAC Power Supply | 4 |
| OEMHPHW3265 | HP X240 10G SFP+ to SFP+ 1.2m DA CC | 2 |
| OEMHPHW5008 | HP OEM BL460c Gen9 10GB/20GB CTO Blade | 4 |
| OEMHPHW5009 | HP BL460c Gen9 E5-2680v3 12c FIO Kit | 4 |
| OEMHPHW5010 | HP BL460c Gen9 E5-2680v3 12c Kit 2nd CPU | 4 |
| OEMHPHW5082 | HP 32GB 2Rx4 PC4-2133P-R Kit | 32 |
| OEMHPHW5018 | HP FlexFabric 20Gb 2P 650FLB FIO Adptr | 4 |
| OEMHPHW5015 | HP QMH2672 16Gb FC HBA | 4 |
| OEMEMCHW195 | EMC Pair of 2m MINI SAS cable | 3 |
| OEMEMCHW190 | EMC 5M MM FIBRE CABLE LC-LC | 4 |
| OEMEMCHW511 | EMC VNX2 Disk Array Enclosure 2.5 RTF | 2 |
| OEMEMCHW527 | VNX 200GB SSD 25X2.5 DPE/DAE | 3 |
| OEMEMCHW544 | ONX 1.2TB 10 SAS 25X2.5 DAE | 30 |
| OEMEMCHW503 | EMC VNX 5400 2,5 NO DISKS | 1 |
| OEMEMCHW546 | VNX 400GB FAST VP SSD 25X2.5 DPE/DAEE | 26 |
| OEMEMCHW514 | EMC VNX 5200 2.5 8 x 900GB 10k | 1 |
| OEMEMCHW512 | EMC VNX2 Disk Array Enclosure 3.5 RTF | 2 |
| OEMEMCHW531 | EMC VNX 900GB 2.5 10K SAS Disk | 7 |
| OEMEMCHW527 | VNX 200GB SSD 25X2.5 DPE/DAE | 3 |
| OEMEMCHW532 | EMC VNX 3TB 3.5 NL SAS Disk | 30 |
| OEMEMCHW195 | EMC Pair of 2m MINI SAS cable | 3 |
| OEMEMCHW190 | EMC 5M MM FIBRE CABLE LC-LC | 4 |

## Software for Hardware components

| Product Number: | Product Description: | Quantity: |
| --- | --- | --- |
| OEMEMCSW527 | VNX OE PERFORMANCE UPGR PER TB 200GB | 3 |
| OEMEMCSW544 | ONX OE PERF PER TB | 30 |
| OEMEMCSW546 | VNX OE PERF PER TB | 26 |
| OEMEMCSW551 | EMC FAST Suite for VNX5400 | 1 |
| OEMEMCSW531 | VNX OE PERFORMANCE UPGR PER TB 900GB | 7 |
| OEMEMCSW527 | VNX OE PERFORMANCE UPGR PER TB 200GB | 3 |
| OEMEMCSW532 | VNX OE CAPACITY UPGR PER TB 3TB | 30 |
| OEMEMCSW550 | EMC FAST Suite for VNX5200 | 1 |
| OSSW5155 | Oracle DB for NetAct Cluster LTU | 1 |
| OSSW6582 | VMware vCenter Server 6.x RLIC | 1 |
| OSSW6585 | VMware vSphere Enterp Plus 6.x 2CPU RLIC | 4 |
| OSSW4287 | IBM SW Bundle NetAct Unify Cluster LTU | 1 |
| OSSW2909 | OSI Stack LTU | 1 |
| OSSW1462 | RogueWave Multiplatform New LTU | 1 |
| OSSW5379 | RogueWave JViews Telco Graph Objects LTU | 1 |

## Software and Licenses for NPM

| Product Number: | Product Description: | Quantity: |
| --- | --- | --- |
| 134-071935.00 | Performance Manager | 1 |
| OSSW6030 | NetAct 19.x SW | 1 |
| OSSW7411 | NPM19 NetAct std SW p LK | 1 |
| OSSWNR0160LK | New Delivery of NetAct 19 SW LK | 1 |
| OSSW6602 | Reporting Suites 19 LK | 1 |
| OSSW6324 | RepSuites Nokia NetActCore&MBH SW emedia | 1 |
| OSSW4888 | NPM Base SW LTU | 1 |
| OSSW4620 | Reporting Suite Nokia Radio GSM-R LTU | 1 |
| OSSW4627 | Reporting Suite Nokia Core Flexi NG LTU | 1 |
| OSSW4628 | Reporting Suite Nokia Core Flexi NS LTU | 1 |
| OSSW4629 | Rep Suite Nokia Core Mobile Voice LTU | 1 |
| OSSW4630 | Reporting Suite Nokia Core MGW LTU | 1 |
| OSSW4636 | Reporting Suite Nokia Core NT HLR LTU | 1 |
| OSSW4637 | Reporting Suite Nokia Core One-NDS LTU | 1 |
| OSSW4641 | Rep Suites GIS & NW Health Adv Pack LTU | 1 |
| OSSW4893 | Dashboard LTU | 1 |
| OSSW4895 | GIS integration LTU | 1 |
| OSSW4897 | NPM Thresholding & Profiling LTU | 1 |
| OSSW5075 | NPM Thresholding & Profiling LK | 1 |
| OSSW4899 | SNMP alarm interface support LTU | 1 |
| OSSW4900 | Real Time PM support LTU | 1 |
| OSSW6475 | NPM - export to pptx SW emedia | 1 |
| OSSW6457 | NPM 19 app SW emedia | 1 |
| OSSW6660 | Reporting Suites 16 Self PM SW emedia | 1 |
| OSSW6323 | RepSuites Nokia Radio LA SW emedia | 1 |
| OSSW6325 | RepSuites Nokia Adv Content SW emedia | 1 |
| OSSW6459 | NPM Real Time PM support LK | 1 |
| OSSW6462 | NPM Dashboard LK | 1 |
| OSSW6464 | NPM GIS integration LK | 1 |
| OSSW6467 | NPM SNMP alarm interface support LK | 1 |
| OSSW6612 | Reporting Suites IP / TX LTU | 1 |
| OSSW6373 | ReportingSuites19 Self PM LK | 1 |
| - | NPM ProRail Reporting Suite | 1 |
| - | NPM ProRail GIS | 1 |
| - | NPM Measurement Tool adapter (XML) (Standard XML MVI capable of loading PI-data from Measurement train, UAP Expandium and UAP 3rd party into NPM as input for the ProRail NPM reporting Suite) | 1 |
| - | NPM QATS MVI (15 minutes) (The RFC4 part includes only the adaptations to the original offered MVI, mainly for GPRS/EDGE KPI’s extensions and ETCSoGPRS, which were not part of original NPM integration.) | 1 |
| - | Post-Processing Software upgrade including XML output format and additional Performance Indicators (difference KPI list v6.31 versus 1.3) | 1 |
| - | Conversation software to adapt TSV output files from Expandium UAP to be compatible with XML standard for MVI Measurement Tools | 1 |