# Specifications

Measurement Vehicle

Table of contents

1	Introduction	4

2	Vehicle types	5

3	Driving capabilities	6

3.1	Trains	6

3.2	On-Road vehicle	6

4	Antenna system in Dedicated Measurement Trains	7

4.1	Introduction	7

4.2	Antenna devices	8

4.3	RF-distribution panel	8

4.4	Antenna jumper cable	9

4.5	Antenna installation on the roof	10

4.5.1	Number of required antennas	10

4.5.2	Ground Plane	11

4.5.3	Grounding	12

4.5.4	Antenna rooftop location	12

4.6	Antenna system Quality and maintenance	14

5	Mounting of antennas	15

5.1.1	mPJules	15

5.1.2	Other dedicated measurement trains	15

6	Antenna system in other vehicles	16

6.1	Antenna devices & jumper cables	16

6.2	Antenna installation	16

6.3	Antenna isolation against other transmission systems	16

7	Space and location requirements	17

7.1	Measurement equipment	17

7.2	Autonomous Scanner	18

7.3	Unattended Probe (UAP)	19

8	Power Supply	19

8.1	Power requirements for the mPJules	19

8.1.1	Measurement equipment	19

8.1.2	Autonomous Scanner	19

8.1.3	Unatttended Probe (UAP)	20

8.2	Power requirement for dedicated measurement trains	20

8.2.1	Autonomous Scanner	20

8.2.2	Unatttended Probe (UAP)	20

8.3	Power Requirements for other types of vehicles	21

9	Odometer	21

10	Environmental requirements	22

10.1	Environmental requirements measurement equipment	22

10.1.1	R&S ROMES Drive test equipment	22

10.1.2	Autonomous Scanner:	22

10.1.3	Unattended Probe:	22

10.2	Environmental requirements measurement vehicles	23

10.2.1	Measurement train mPJules	23

10.2.2	Dedicated measurement trains:	23

10.2.3	Other vehicles:	23

# Introduction

An GSM-R network needs mobile measurements to be executed on-board a test train and/or a test car, for the purposes of tuning, optimizing and validating the radio network.

This document aims at giving requirements for a measurement vehicle.

Such a vehicle shall have

- a number of GSM-R measurement antennas

- an GPS antenna

- a device to measure the distance traveled

- an uninterruptible and filtered power supply

- an GSM-R measurement set

Based on the measurement equipment used by Mobirail for radio network tuning and validation, this document gives guidance on:

- Antenna types for GSM-R measurements

- Installation of the GSM-R measurement & GPS antennas

- Power Supply requirements for the measurement equipment

- Space required to install, during measurement campaigns, the measurement equipment

- Space required to install the Autonomous Scanner

- Space required to install the Unattended Probe (UAP)

- Limitations towards other systems on-board the measurement vehicle

- The required in-vehicle environmental conditions

# Vehicle types

From a practical point of view, it is not possible to use a single vehicle to execute all types of measurements required at all the necessary places on the network.

While other types may be used, the following types of vehicles have been defined:

- Dedicated Measurement Train -mPJules

- Dedicated measurement train for autonomous/automatic measurements

- Other vehicles, like i.e High-Speed Train, Special Areas Train, On-Road vehicle

Unless otherwise specified, the requirements listed in this document will apply to all sorts of vehicles.

# Driving capabilities

## Trains

It is recommended that any measurement train can drive in two directions, without the need of turning the train around.

The Dedicated Measurement Train mPJules should be capable of running on most tracks as part of the GSM-R national contract.

The dedicated Measurement Train for the autonomous scanner/unattended probe should preferable be capable of running on most tracks as part of the GSM-R covered lines. Because 3 to 5 types of different trains might be considered, the requirement is not so strict to each individual train type, as long as the combination of all of those different types of trains can fulfill the requirement.

Special vehicles, such as High-Speed Trains, should be capable of running on all tracks where the speed exceeds the maximum allowed speed of the Dedicated Measurement Train.

## On-Road vehicle

An On-Road vehicle shall be capable of driving on all the roads of the country.  
It shall be capable of reaching quickly the maximum allowable speed of any road in the country.

# Antenna system in Dedicated Measurement Trains

## Introduction

Special attention is required on the antenna installation on the train, as for optimization of radio-network parameters and as well as to be useful for measuring contractual key performance indicators, the antenna pattern should be fully Omni-directional.

Besides selecting the correct antennas for the train, it is extremely important to follow the guidelines set out by the antenna manufacturer to obtain the Omni-directional antenna pattern.

A distinction is done here between the Dedicated Measurement Train, which shall have fixed antennas installed on the roof, and the other vehicles, where antennas will only be installed on a temporary basis, for the time of the measurements.

## Antenna devices

The train antennas should be capable of operating in the GSM-R frequency band.  
Mobirail propose to use the following Kathrein antenna types:

- 741009, very low profile antenna of 0 dBd (95 mm high)

- 87010003, low profile multiband antenna with integrated GPS antenna (81 mm high)

These antennas are protected against contact with high-voltage; quoting Kathrein: “This antenna (…..) is DC grounded to protect against lightning and high-tension lines.”

## RF-distribution panel

As the antennas need to be connected to different measurement systems it is advised to terminate the antennas on an RF-distribution panel inside the measurement coach/train.

Preferably the antenna connectors on this RF-distribution panel are female N-type connectors.

The RF distribution panel should for safety reason be grounded.

The VSWR of the antennas measured at the RF-distribution panel should be lower than 1.5 within their operating frequency.

The antenna distribution panel should be located close to the location of the measurement equipment, allowing cable lengths between these of maximum 2 m.

The RF-distribution panel should be easily accessible, even during a drive, as the engineer performing the measurements, will change antenna / equipment configuration during the drive, whereby certain antennas and or equipment will be disconnect and/or connected towards other antennas, i.e. based on driving direction, type of measurement to be performed. This is applicable to all antennas intended for GSM-R measurements and GSM-R measurement equipment (drive test equipment, Autonomous scanner and Unattended probes, or any additional measurement equipment). Tuning and validation has higher priority in such case as automatic verification drives.

Dedicated measurement trains to be equipped only with fixed installed, planned to remain for a long time within the trains, Autonomous scanner and/or Unattended probes, do not require an RF-distribution panel. A direct connection between antenna and scanner and/or probe is allowed.

## Antenna jumper cable

The antenna jumper cable, between the antenna and the RF-distribution panel, or between antenna and Autonomous scanner/Unattended probe, should be as short as possible and properly installed, without any cable damage.

The total loss between the antenna and the distribution panel should not exceed 2 dB at 900 MHz.  Its screening should exceed 85 dB at 900 MHz.

The feeder cables should be routed completely separately from all other vehicle wiring and not tied to them. Ideally, the minimum separation should be 150 mm and, where required to cross over other wiring, the cables should be aligned in the perpendicular.

For the purpose of yearly verification/calibration it is recommended that the cable is easily accessible at both ends.

The cables should be protected against contact with the overhead line under all circumstances as such contacts could result into lethal voltages from the contact line to persons on board, as well as cause damage to the equipment on board.  The antennas are protected against contact with high voltage (see 4.1.1).

## Antenna installation on the roof

### Number of required antennas

A distinction must be made between the mPJules and other dedicated measurement trains. The latter are the ones to be equipped only with fixed installed Autonomous scanner and/or Unattended Probe solution.

#### mPJules

This measurement train is planned to be used for all kind of GSM-R measurements, including tuning, validation and monitoring. Especially for the purpose of tuning and validation, the number of antennas need to be maximized, to be able to perform different kind of measurements in parallel. However, to be able to perform reproducible measurements, a correct planning of the measurements is needed, in order to prevent mutual influence of the different measurements, which even can cause damage on measurement equipment. Therefore, the guidelines as specified in this document should be followed. If the mPJules is used for tuning and/or validation purposes, the antenna positions for the Autonomous scanner and/or Unattended Probe may change or become temporarily unavailable.

#### Other dedicated measurement trains

This chapter refers to trains on which only Autonomous scanners and /or Unattended probes will be installed, on a permanent basis.

The following number of antennas are required:

- 1 combined GSM-R/GPS antenna for the Autonomous scanner

- 1 combined GSM-R/GPS antenna and 1 GSM-R antenna for the Unattended probe (equipped with 2 GSM-R radiomodules)

For the mounting of the 4G antennas, no specific requirements exist, except the fact that the 4G antenna should be placed more than 1 meter away from the Autonomous Scanner system.

### Ground Plane

Train mounted antennas are designed as λ/4 radiators and need for proper functioning to be mounted on a conductive surface creating a ground plane for the antenna.

Figure 4-1 electrical field lambda/4 antenna

Antennas easily will lose VSWR performance, and radiation pattern may change dramatically if no appropriate ground plane is available.

The ‘ideal’ ground plane for a typical train antenna is a large, flat, horizontal plate of metal located above the height of any other object on top of the train, with the antenna mounted centrally on it. The practical ground plane should come as close as possible to the ‘ideal’ ground plane.

Any obstacle close to the antenna may influence the radiation pattern and radiated waves, and as the results the measurements, performed via the antennas

### Grounding

In case of an accident or a failure of the overhead line high voltage and current might be applied to the antenna. To protect personal and equipment, connector voltage of the antenna is supposed not to exceed 60 V. To guarantee low connector voltages, antenna flanges need to be grounded thoroughly.

To achieve best conductivity mounting surfaces at the antenna socket and the mating surface of the train should be clean. Any paint residues or other pollution needs to be removed prior to the mounting process. Further an unpainted area is needed near the four mounting holes of the antenna flange.

### Antenna rooftop location

Generally, antennas need to be mounted several meters away of pantographs. 
The reason for this separation is based on safety grounds, to prevent flashover to the antenna and to reduce broadband noise. A raised pantograph also can have a negative effect on the antenna radiation pattern.

#### mPJules

Based on the measurement equipment that will be supplied by Mobirail for the measurement train, and specifically for the train ‘mpJules’, the following antenna distribution is recommended:

In case the ground plane has a length in between 4.2 and 5.9 m, see Figure 4-2

In case the ground plane has a length of at least 5.9 m, see Figure 4-3

Figure 4-2: Antenna distribution plate 4.2m to 5.9m long

Figure 4-3: Antenna distribution plate length minimum 5.9 m

In case the metallic rooftop plate is longer than 4.2 m but smaller than 5.9 m the length on top 4.2 m should be divided by 2 and added on top of the 1.7 m between adjacent antennas.  Similar if the rooftop plate length is more than 5.9 m, the length on top of 5.9 m should be divided by 3 and added to the 1.7 m as antenna inter distance.

On each plate one antenna of the type 87010003 has to be installed, and this on the closest location to the “rijweerstanden”.  The other antenna types will be of the type 741009.

As rooftop obstructions, have a significant impact on received signal levels, with a greater magnitude as normal radio fading environment experienced on a train within multipath environments, no obstructions outside the above indicated antennas are allowed on the ground plate of the antennas.

#### Other dedicated measurement trains (AS/UAP)

This chapter refers to trains on which only Autonomous scanners and /or Unattended probes will be installed, on a permanent basis.

Recommended to install the antennas at the train rooftop, at a height of about 4m above the track.

The Autonomous scanner antenna and the roof edge(s) should not be closer than 0,5 meter.

Special attention should be given towards the antenna to be used for the Autonomous scanner, in terms of isolation versus other radiating (active) antenna elements. The latter may easily influence the C/I measurements, when installed in close vicinity to the scanner antenna. Also, to be able to use the full dynamic range of the coverage scanner, sufficient isolation should be obtained between the scanner antenna and any other radiating (active) antenna.

To reach enough isolation, some isolation calculation is needed. For any other operator which is carrying out measurements on the same train as where the Autonomous scanner is installed, the following maximum level (dBm) requirements have to be taken into account in the calculations for defining enough antenna interdistance between the antenna of the Autonomous scanner and the used antenna by the other operator:

- GSM-R frequency band: -10 dBm, measured at the patch panel connector of the Autonomous scanner.

- All other frequency bands: +10dBm, measured at the patch panel connector of the Autonomous scanner.

If those calculations are not available, the general rule is, based on the used GSM-R band and output powers of typical radio-modules, that an antenna interdistance of at least 4m is advised between de antenna of the Autonomous scanner and any other radiating (active) antenna.

Antennas for other scanners or passive equipment must respect an interdistance of 0,5 meter with the Autonomous scanner antenna. See also Figure 4-4.

Figure 4-4 Antenna interdistance overview

## Antenna system Quality and maintenance

After installation each antenna in combination with its jumper cable should be quality tested by means of a VSWR (Voltage Standing Wave Ratio) measurement (over the full ER-GSM band: 873 – 880MHz and 918 – 925MHz). The VSWR measurement should result in a value <1,6 (<5.2% reflection). At commissioning the measured VSWR should be recorded together with the measurement date and labelled at the coax-end at which the measurement was performed. It is required to verify this at least every 2 years by re-measuring the VSWR and compare results to the value recorded at commissioning. When the VSWR is increased to 2,0 or more, repair or replacement of connectors, cable and/or antenna should be carried out.

In addition to the VSWR measurement at commissioning and repeated at least every 2 years the entire antenna and cabling must be visually verified for proper mounting and absence of quality degradation due to chafing or other impairments.

# Mounting of antennas

### mPJules

Recommended mounting of the antennas is directly to the metallic ground plate which for this purpose will be equipped with integrated mounting bolts.

Mounting screws or nuts should not protrude more than 15 mm from the mounting surface.

### Other dedicated measurement trains

Recommended mounting if the antennas is directly to the metallic roof of the train or the use of special ground plates as used on the mPJules.

Mounting screws or nuts should not protrude more than 15 mm from the mounting surface.

# Antenna system in other vehicles

These other vehicles are deemed not to have any fixed measurement antenna installed for the purpose of the measurement equipment.

Temporary antennas have thus to be installed, following the requirements listed below.

## Antenna devices & jumper cables

Omni-directional antennas shall be exclusively used.

Small portable antennas with magnetic base are recommended.

Such types of antennas are usually provided with their own jumper cable.  The cable loss usually compensates for the antenna gain; therefore, the whole antenna system can be considered as a 0 dBi gain antenna.

A minimum of one antenna shall be installed on the vehicle; however, when possible, several antennas shall be installed to allow for parallel measurements without the need of a power splitter.

## Antenna installation

Where possible, the antennas shall be installed on the roof of the vehicle, as centrally as possible, as high as possible and clear from any obstruction on the roof.

When more than one antenna is installed, a minimum distance of 0.5 m shall be kept between antennas.  Further, the distance between the antennas and the roof edge shall be at least 0.5 m.  The distance between all antennas has to be maximized, taking into account these constraints; special attention will be required regarding the tests executed in order to minimize influence between the tests running on different antennas.

In some vehicles, the antenna installation could be a challenge, and installation on the side of the vehicle for example could be the only possibility.  In such a case, this kind of sub-standard installation shall be tolerated.

## Antenna isolation against other transmission systems

When the measurement vehicle incorporates a GSM-R CAB Radio for its own purposes, a minimum distance of 4 m should be kept between the measurement antennas and the antenna(s) connected to the train’s GSM-R cab radio.

For measurement vehicles equipped with other transmission systems (that are using external antennas), a separate recommendation about antenna isolation can be provided upon request.

For more information about antenna interdistance requirements, see also 4.5.4.2

# Space and location requirements

## Measurement equipment

To be able to use the measurement train for network optimization tasks and contractual KPI verification, a space should be reserved to install properly the measurement equipment.

The measurement system is housed in two 3U 19” rugged portable racks, with covers (front, rear) and robust handles. The racks have an anti-vibration frame for plug-in components.  The size of an individual rack is (HxDxW): 22x58x54 cm.

To carry the auxiliary equipment (laptop, docking station, cabling, power supply etc.) to and from the train, a trolley case is used. During the performing of the measurements, this trolley case is securely stored in the train.

A place within 2 m cable distance towards the RF distribution frame should be available to locate in a secure way the measurement racks.  They have to be stacked on top of each other; the sides must be kept free for ventilation purposes.

Additionally, a location to host 2 operators, operating simultaneously one laptop each is required in the near surroundings to the measurement equipment.

In case the measurement vehicle is a car, a reduced configuration with only one operator is allowed.

## Autonomous Scanner

To be able to use the measurement train for network monitoring tasks, a space should be reserved to install properly the Autonomous Scanner equipment.

The Autonomous scanner as proposed by Mobirail is housed in two stackable units, as presented in Figure 7-1.

Figure 7-1: Autonomous scanner including Battery Pack (frontside)

The dimensions of one single unit is (WxHxD): 204 x 45 x 171 mm, with associated battery pack the dimensions become (WxHxD): 204 x 91 x 171 mm. The backside of the Autonomous scanner is presented in Figure 7-2

Figure 7-2: Autonomous scanner including Battery Pack (backside)

When installed into a 19” rack, by means of a mounting plate a height of 3U need to be taken into account.

## Unattended Probe (UAP)

The Unattended Probe Train equipment is housed in an standard 3U-84HP 19”rack.

Rack sizes (HxDxW): 132 x 340 (power connected) x 482 mm; Weight: 6 Kg.

Figure 7-3: Unattended Probe

# Power Supply

The measurement equipment can be powered using two different voltages: 
230 V AC or 12 V DC.

All load requirements listed below must be understood as continuous (not peak), meaning that the power supplies must be able to provide the required current continuously.

## Power requirements for the mPJules

### Measurement equipment

A standard power outlet of 230 V, 2 A minimum, equipped with a dual-conversion non-interruptible power supply device having a 15 minutes lasting time at 460 VA load shall be provided aboard the vehicle for the purpose of connecting the measurement equipment.

Additional 230 V standard power outlets (2 A min.) shall be available to connect further equipment (e.g. laptops which have an own battery). In reality the auxiliary equipment (e.g. laptop) is also connected to a standard power outlet of 230 V, 2 A minimum, equipped with a dual-conversion non-interruptible power supply device.

### Autonomous Scanner

A standard AC power outlet of 230 V, 2 A minimum, shall be provided aboard the vehicle for the purpose of connecting the Autonomous Scanner. Alternatively, a 12V DC power outlet (11V to 18V DC -0%/+10%) with max 4A fuse can be used for powering the Autonomous scanner.  The nominal power consumption is 30W.

The used power outlet 230V AC or 12V DC automatically must provide current after starting up the train and during every ride with the train.

Since the Autonomous Scanner has its own battery packs, no non-interruptible power supply device is needed.

### Unatttended Probe (UAP)

A standard power outlet of 230 V, 2 A minimum, shall be provided aboard the vehicle for the purpose of connecting the Unattended Probe. The used power outlet automatically must provide current after starting up the train and during every ride with the train.

The Unattended Probe is equipped with a back-up battery using super capacitors. Charging/discharging time is 4’20”/4’30” nominal consumption. No additional non-interruptible power supply is needed. The peak current at startup is 0.7A max.

## Power requirement for dedicated measurement trains

This chapter refers to trains on which only Autonomous scanners and /or Unattended probes will be installed, on a permanent basis.

### Autonomous Scanner

A standard AC power outlet of 230 V, 2 A minimum, shall be provided aboard the vehicle for the purpose of connecting the Autonomous Scanner. Alternatively, a 12V DC power outlet (11V to 18V DC -0%/+10%) with max 4A fuse can be used for powering the Autonomous scanner.  The nominal power consumption is 30W.

The used power outlet 230V AC or 12V DC automatically must provide current after starting up the train and during every ride with the train.

Since the Autonomous Scanner has its own battery packs, no non-interruptible power supply device is needed.

### Unatttended Probe (UAP)

A standard power outlet of 230 V, 2 A minimum, shall be provided aboard the vehicle for the purpose of connecting the Unattended Probe. The used power outlet automatically must provide current after starting up the train and during every ride with the train.

The Unattended Probe is equipped with a back-up battery using super capacitors. Charging/discharging time is 4’20”/4’30” nominal consumption. No additional non-interruptible power supply is needed. The peak current at startup is 0.7A max.

## Power Requirements for other types of vehicles

In other measurement vehicles, the equipment shall use either standard 230 V AC power or 12 V DC power, depending on the situation and what is available on-board.

The power supply must be able to sustain 460 VA.

A non-interruptible power supply device is required in any kind of vehicle to prevent problems due to any power fluctuation.

# Odometer

Odometer pulses shall be provided to drive a scanner into a distance-triggered operation.
Such pulses are only required for the mPJules or other vehicles by which distance triggered measurements will be performed

The pulses shall have a square shape with a voltage within the range +/- 3.3 V DC until +/- 12 V DC.

The pulses shall be representing a distance of movement short enough so that one or several pulses can trigger a measurement from the scanner TSML each 10 cm.  
In other words, the distance driven between two pulses should not exceed 10 cm and should be as close as possible to an exact divider of 10 (1, 2, 5, 10).  For the sake of accuracy, the smaller the distance, the better.

When a measurement vehicle does not have an on-board odometer, the Pegasem GSS25C can be used; it can achieve an accuracy of 1 cm up to 300 km/h. Only the TSML scanner as part of the measurement train drive test-system can handle distance triggers.

Autonomous scanner and unattended probes measurements are time based triggered measurements and thus are not using an odometer.

# Environmental requirements

Different type of equipment have their own environmental requirements as specified in paragraph 10.1. The resulting environment requirements towards the different vehicles are then specified under paragraph 10.2.

## Environmental requirements measurement equipment

### R&S ROMES Drive test equipment

The measurement vehicle should be a safe work location for the staff.

An operating temperature between +5°C and +35°C is required by the measurement equipment.

The measurement equipment must be stored within the temperature range from -20°C to +65°C.

The measurement equipment should never be exposed to direct sunlight.

It is highly recommended that a temperature control device be available when possible in the measurement vehicle.  If such a device is not present, drives could be cancelled when extreme temperatures are encountered.

### Autonomous Scanner:

An operating temperature between +5°C and +45°C is required by the Autonomous Scanner.

The Autonomous Scanner must be stored within the temperature range from -25°C to +70°C.

### Unattended Probe:

The Unattended Probe may be operated at temperatures between +5°C and +45°C.

The Unattended Probe will withstand temperatures between -25°C to +55°C.

## Environmental requirements measurement vehicles

### Measurement train mPJules

The measurement vehicle should be a safe work location for the staff.

The operating temperature should remain between +5°C and +35°C inside the rack housing the equipment and or installation location of the equipment. To remain within this temperature range the measurement equipment should never be exposed to direct sunlight.

It is highly recommended to have a temperature control device, with associate heater and cooler inside the train. If such a device is not present, drives could be cancelled when extreme temperatures are encountered.

### Dedicated measurement trains:

The operating temperature should remain between +5°C and +45°C. Advisable is a power source for the scanner and/or unattended power, which is power controlled, by means of temperature sensors. This should assure automatic powering of the equipment when the train is used, and the temperature at the location of the Scanner and probe is within the specified temperature range.

If temperature within the train could go lower as -25°C, it is advised to take matters to protect the equipment.

### Other vehicles:

Same temperature limitations as for the mPJules are applicable.

Annex A: Antenna datasheets

Kathrein Antenna 870100003

Kathrein Antenna 741009

Annex B : Specifications Unattended Probes

History

| Version | Date | Author | Short description of changes |
| --- | --- | --- | --- |
| 0.1 | 27-03-2015 | Alexandre Amant | First draft |
| 0.5 | 08-04-2015 | Alexandre Amant | Changed document name |
| 0.6 | 25-09-2015 | Alexandre Amant | Incorporated review comments |
| 1.0 | 30-03-2016 | Alexandre Amant | Final release |
| 1.1 | 30-08-2018 | Richard Trip | Added information for Autonomous Scanner |
| 1.2 | 21-09-2018 | Kees Leijerzapf | Unattended Probe requirements added |
| 1.3 | 11-01-2019 | Johan Famaey | Train type based requirements added |
| 1.4 | 29-03-2019 | Richard Trip | Incorporated review comments and changed formatting entire document |
| 1.5 | 05-04-2019 | Richard Trip | Incorporated review comments |
| 2.0 | 23-07-2020 | Harmen Schreuder | Added chapter 4.6 |
| 2.1 | 07-09-2020 | Harmen Schreuder | VGB version |