Market Consultation

Frequency comb laser system

Datum versie: 5 July 2022

Contents

1. Background information regarding the application………………………………………………………………………..…3

2. Required specifications of the Frequency Comb Laser..........................……..…………………………..…………3

3. Procedure and timetable of the Market Consultation …………………………………………………..…………………5

4. Other terms and conditions ………………………………………………………………………………………....…………………6

## Background information regarding the application

The Vrije Universiteit Amsterdam (VU) is a large innovative university which actively contributes to developments in education and research.  For more information about the VU, see https://www.vu.nl/en

The section of Quantum Metrology and Laser Applications, in the Physics and Astronomy department, has the intention to acquire a state-of-the-art Frequency Comb Laser system for advanced precision measurements and transfer of optical frequencies.

The purpose of this market consultation is to find potential suppliers of such an instrument.  Candidates are invited to respond on the basis of the information provided in this document regarding functional specifications, delivery, service provision and other conditions.

If, after requesting the information, you decide not to respond further, we kindly request you to notify us.

To be clear: this is NOT a quote request.

## 2. Required specifications of the Frequency Comb Laser

### 2.1 General description

The Frequency Comb Laser should be a turnkey system, that is modular and can be extended to work at multiple wavelength ranges in the infrared, near-infrared and the visible. It should have a high enough repetition rate frequency (mode spacing), and the system should be such that one of the optical modes can be optically locked to an ultrastable laser (<2 Hz) at 1542.5 nm that is already present. The system minimum requirements are listed below.

### 2.2 The Frequency Comb Laser should include the following features:

Hardware

• The system should be a complete set of hardware and software according to the specifications below, and rack mounted.

• The repetition rate of the frequency comb oscillator laser can be anywhere between 190 MHz and 260 MHz, with a few MHz tuning range by the user.

• Base operation wavelength range should be near 1560 nm (Er-fiber based) with a bandwidth of >=20 nm, and should enable optical locking of the comb near 1542.5 nm. Optical output power >=10 mW per output port, and several available for extending the system in the future.

• PM fiber input and outputs (preference for FC/APC)

• The system should have an optical beat detection / locking unit at 1542.5 nm, accepting input from an external 1542.4 nm ultra-stable laser source that is already available (power available of a few mW). Moreover, the system should accept as an RF reference the 10 MHz output from a commercial Cs atomic clock (also already available).

• The system should extendable with module(s) for operation (making beat notes) in a range of 500 – 2000 nm. Specific wavelengths currently of interest are near 1450 nm, near 800 nm, and 626 nm.

• The integrated phase noise (from at least 100 Hz to a few MHz) should be below 100 mrad.

• The stability should be better than 10-16 in 1 second, and better than 10-18 in 1000 seconds.

• The accuracy should be 10-17 or better for integration times of 100 s or longer.

• The linewidth (optically locked) of the frequency comb modes should be close (<1 Hz in addition) compared to the optical reference (which is <2 Hz).

• The frequency comb laser system should have an inbuilt computer with accompanying software, and should be controllable (to a certain degree, to be negotiated) and enable read-out by an external computer.

2.3 Other features

- Warranty of at least 1 year for the instrument.

- Free software upgrades during warranty

- Phone and remote support are always available.

### 2.4 Delivery

Preferred delivery of the Frequency Comb Laser is early or Spring 2023.

### 2.5 Price

The purchase price includes all costs, such as shipping & handling, installation and implementation, guarantees, training and documentation, and any additional costs for travel, accommodation, training  and testing.

## 3. Procedure and timeline of the market consultation

During the market consultation, all communication with regard to this subject is conducted via the purchasing management department of the VU University. Contact details, see below:

| Name: | Purchasing Management Department Dhr. Tarik Ouchan |
| --- | --- |
| Faculty / Department : | Finance Department |
| Address: | Van der Boechorststraat 1 1083 BT Amsterdam |
| E-Mail: | aanbestedingen.fpc@vu.nl |
| Citing | Market Consultation: Frequency Comb Laser QMLA |

Timetable Market Consultation:

| Description | Deadline |
| --- | --- |
| Publication on www.tenderned.nl | 08-07-2022 |
| Publication Memorandum of Information | 11-07-2022 |
| Deadline for questions | 15-07- 2022, 11:00 a.m. |
| Last moment to submit answers | 19-07- 2022, 11:00 a.m. |
| Sending summary to participants and closing the market consultation process | 22-07- 2022 |

You are requested to respond to this market consultation no later than 11:00 am on 19-07 2022. Responses received after this period will no longer be considered.

The VU would like to give you the opportunity to ask any questions you may have about this market consultation.

You can do this by e-mail until 15-07 2022, 11:00 a.m. at the latest. You should send your application and questions by e-mail to aanbestedingen.fpc@vu.nl  for the attention of the Purchasing Management department.

When submitting your response, please include "Frequency Comb Laser QMLA” in the subject line.

Answers to the questions will be published on Tenderned by means of an information memorandum by 19-07- 2022 at the latest.

The VU will keep the option open to invite parties, in response to the submitted responses, to provide additional explanations to your answers orally and/or by means of a presentation. The VU will contact these parties and schedule a meeting in consultation.

The results of the market consultation can be used as input for any tender to be followed. After the market consultation has ended, written feedback of the market consultation is provided to all respondents in the form of a summary of the answers, taking into account any commercial interests of the parties. This feedback does not affect the obligation of the market consultation. Prices and/or rates are excluded from publication.

Candidates who have participated in this market consultation and who have answered the questions will receive the summary by e-mail, no later than 22-07-2022.

This summary will also be published as an appendix to any tender to be followed.

4. Other terms and conditions

In addition to the terms and conditions stated elsewhere in this document, this market consultation is subject to the following terms and conditions:

• Parties cannot derive any (mutual) obligations or rights towards the VU from this market consultation;

• Participation in this market consultation does not guarantee a role as supplier;

• Any costs for participating in this market consultation will not be reimbursed by the VU;

• The official language during this market consultation is in principle English;

• the VU is in no way bound by the results of the market consultation or obliged to realize and/or tender for the subject to which the market consultation relates;

• Claims about the use of information, confidentiality or requests for compensation in connection therewith will not be honored;

• The VU reserves the right to suspend this market consultation temporarily or permanently.