Site Studies Wind Farm Zones

Various Sites

SCOPE OF WORK

Geological Desktop Study
Colophon

| Project name | Geological desktop study for various sites |
| --- | --- |
| Project number |  |
| Contact person Author Reviewed Approved | Menno Hofstra M +31 6 11981357 Menno.hofstra@rvo.nl Nationale Projecten Croeselaan 15 ¦ 3521 BJ Utrecht Postbus 8242 ¦ 3503 RE Utrecht Menno Hofstra, Peter-Paul Lebbink Peter-Paul Lebbink, Janus Thal (Arcadis) Matté Brijder |

SCOPE DESCRIPTION OF STUDIES TO BE CARRIED OUT FOR OFFSHORE WIND TENDER

| GEOLOGICAL DESK STUDY’s SECTIONS | TASKS |
| --- | --- |
| - PURPOSE | Get insight in expected soil conditions, geological features and bathymetry of three Wind Farm Zones (Nederwiek WFZ, Lagelander WFZ and Doordewind WFZ) and Search Area 8 based on existing literature, publicly available data and data provided by the Contracting Authority. These studies will serve as basis for the scope of the planned geophysical surveys and geotechnical surveys as well as integrated ground models. |
| - MAIN OBJECTIVES | - Provide literature overview including a geological framework that can be used as the basis to define the scope of a geophysical and geotechnical survey (including but not limited to the seismic profiling survey and the scoping for suitable geotechnical locations). - Development of a legacy data ground model based on the integration of publicly available data and data (including seismic) provided by the Contracting Authority and applying the following process: - Reprocessing of publicly available 3D seismic data - Interpretation and gridding of key subsurface interfaces and features in the publicly available 3D seismic and provided reprocessed 2D seismic data - Providing both a GIS archive and a 3D geological visualization model including all relevant information. - Provide in an early stage of development information on areas that might be less suitable for the construction of offshore wind farms. - Provide bathymetric, seismic, geotechnical and geological information/overview required for the Environment Impact Assessment (EIA). - Provide a recommendation per site for preliminary geotechnical locations where possible based on the available data. |
| - ASSESSMENT SHOULD INCLUDE | - Overview of all publicly available geological, geotechnical, geophysical and bathymetric data for all Wind Farm Zones and Search Area 8 including borehole and CPT data. Bathymetric data should be in Lowest astronomical Tide (LAT) - Overview of all available relevant literature on the geology and geomorphology of the area. - Assessment of the suitability of publicly available data for carrying out the assessment. Assessment shall emphasise discrepancies between different sources of information if they exist (including spatial/mapping discrepancies at map edges). - Delivery and initial population of suitable seabed topography and sediment classification schemes. - Selecting and reprocessing of publicly available 3D seismic data cubes for the Lagelander WFZ, Nederwiek WFZ and Search Area 8, with the intention to increase the data quality within the top 150 m below seafloor (BSF). Reprocessed 2D seismic data for all sites and reprocessed 3D seismic data for the Doordewind WFZ will be provided by the Contracting Authority. It is up to the Tenderer to propose a suitable efficient processing strategy with the intention to improve the interpretability of the top 150 m BSF. - Mapping of the key subsurface interfaces representing boundaries of distinct geological units in the available 2D and 3D seismic data up to 150m BSF in the Lagelander WFZ, Nederwiek WFZ and Search Area 8. Mapping of major geological features (e.g. palaeovalleys, channels, potential shallow gas etc…) up to 150m BSF when present and observable. The subsurface mapping will be based on, but not limited to, a seismic data archive provided by the Contracting Authority. The number of interfaces (horizons) should be proportionate to what is observable within the resolution and quality of the available data. Currently we estimate 5 geological horizons (including seafloor) to be mapped across the sites. For the Doordewind WFZ key subsurface interfaces have already been mapped (see 4.6 of this document) - Gridding of key subsurface interfaces in the depth domain representing boundaries of distinct geological units. It is up to the successful Tenderer to decide in agreement with the Contracting Authority which interpreted horizons on which seismic dataset are suitable enough for gridding. In case the interpreted horizons only exist in the time domain, a simple (e.g. two layer) time-depth conversion method is deemed sufficient. - Development of a stratigraphic framework and legacy data ground model based on available public data, seismic interpretation and literature up to a depth of 150m BSF. The stratigraphic framework shall be developed with the aim of defining key interfaces and expected geological units within the depth of interest during subsequent geophysical and geotechnical interpretations. The stratigraphic framework shall therefore describe the geology to the formation level for the stratigraphic sequence within the depth of interest and to the formation-member level where required. When uncertainty exists in developing the stratigraphic framework due to conflicting data sources, the most appropriate shall be selected (and this selection justified), in order to provide an unambiguous framework for the geological models to be developed as a result of subsequent geotechnical and geophysical investigations. The geotechnical/engineering significance of the different formations and members shall be described. - Where available, the indicative spatial rate of change of depth to geological interfaces and/or geotechnical properties shall be provided, together with the dominant structural strike, with a view to guidance for the design of geophysical and geotechnical investigation. - Identification of possible constraints for the installation of offshore wind farms in all Wind Farm Zones and Search Area 8 as a result of the soil conditions. Potential geohazards identified from the literature shall be described. - Recommendations and collateral data for each site to support the development of the scope of a geophysical survey (planned in 2023); including but not limited to the seismic survey, and to support the development of the scope of a geotechnical survey shall be provided. Collateral data should include consideration of probable seabed materials to support the specification of seismic and sub-bottom profiling methods, and the spatial variability of sub-seabed structures developed under section 5. These materials for support of the development of the scope of a geophysical and geotechnical survey should be delivered in a memo for each site (four in total) separately from the report. |
| - STARTING POINTS & BOUNDARY CONDITIONS | - Archaeology will be addressed in a separate archaeological desk study. - UXOs will be addressed in a separate UXO desk study. - The morphodynamics will be addressed in a separate morphodynamical desk study. - The Wind Farm Zone boundaries are made available by the Client through a shapefile included in this tender. This also includes other user functions in the area, active and out of use cables and pipelines in the area. - Reports of previous desk studies are available on the RVO offshore website, examples of these studies are: Soil IJmuiden Ver · Offshorewind RVO and Soil TNW Offshorewind RVO - Seismic data are available at the following links DDW report and data (1) DDW data (2) DDW OpendTect NW, LL and SA8 report and data - Further information is provided in Annex 7 – Starting Points and Assumptions |
| - MEETINGS | Supplier must be available for meetings with client at: - Start of the assignment (kick-off) – in person. - After finalizing Tasks 1, 2 and 3 of the assessment (Section 3). Overview of publicly available data and available literature and its assessment for suitability for all sites - online. - Propose and present a suitable efficient seismic processing strategy of task 5 (Section 3) – online. - After finalizing Task 6 (section 3) to present identified major surfaces and geological features from the seismic mapping part. Either as two meetings (one for NW and one for all other sites) or one meeting for all sites – online. - Workshop with the contractor for the archaeological desk study to discuss findings that are relevant for the archaeological desk study and improve integration between the two. - Presentation of the end results – in person. |
| - PLANNING | Due to time constrains for the Contracting authority, there are first, second and third priority areas identified. 1st Priority: Nederwiek WFZ 2th Priority: Doordewind WFZ 3th Priority: Lagelander WFZ and Search area 8 A preliminary draft report and associated data deliverables including all requested elements for the Nederwiek Wind Farm Zone should be issued within 10 weeks after the assignment has been signed. A preliminary draft report of the Doordewind Wind Farm Zone should be issued within 15 weeks after the assignment has been signed. The remaining draft reports for Lagelander Wind Farm Zone and Search Area 8 should be issued within 20 weeks after the assignment has been signed. The total review process and the finalization of the report is planned according to the schedule below: Execution first draft reports NW 10 weeks Mid December (e.g. 12‐12‐22) Delivery first draft report NW 1 day Mid December (e.g. 12‐12‐22) Review first draft report NW 1 week Mid to end December (e.g. 19-12-22) Issue second draft report NW 3 weeks Early January (e.g. 6-1-23) Review second draft report NW 1 week Mid January (e.g. 13-1-23) Issue final report NW 1 week Mid to end January (e.g. 20-1-23) Execution first draft reports DDW 15 weeks Mid January (e.g. 16-1-23) Delivery first draft report DDW 1 day Mid January (e.g. 16-1-23) Review first draft report DDW 1 week End January (e.g. 23-1-23) Issue second draft report DDW 2 weeks Early February (e.g. 6-2-23) Review second draft report DDW 1 week Mid February (e.g. 13-2-23) Issue final report DDW 1 week Mid to end February (e.g. 20-2-23) Execution first draft reports LL and SA8 20 weeks Mid to end February (e.g. 20‐2‐23) Delivery first draft reports LL and SA8 1 day Mid to end February (e.g. 20‐2‐23) Review first draft report LL and SA8 1 week End February (e.g. 27-2-23) Issue second draft report LL and SA8 3 weeks Mid to end March (e.g. 20-3-23) Review second draft report LL and SA8 1 week End March (e.g. 27-3-23) Issue final report LL and SA8 1 week Early April (e.g. 3‐4‐23) The final report for NW should be completed not later than 16 weeks after the assignment has been signed. The final report for DDW should be completed not later than 20 weeks after the assignment has been signed and the remaining final reports (LL and SA8) should be completed not later than 26 weeks after the assignment has been signed. Draft reporting shall include all deliverables including all digital deliverables. |
| - REPORT DELIVERABLES | - Descriptive separate report for each site (in English) describing at least: - Dutch and English Management Summary. - Introduction including a specification of objectives and constraints. - Description of the methodology and analysis carried out. - Discussion, results and recommendations on the topics specified under Section 3. - Data that have been used. - Characteristics/parameters that have been defined for carrying out the suitability assessment. - Conclusions of the assessment. - Two orthogonal geological cross-sections showing the geological sequence of each Wind Farm Zone (including all formations and members as referred to in Section 3, Task 3). Any processing required to produce this geological cross-section should be included in the price. - A Holocene subcrop map, i.e. a geological map of the material immediately underlying the base of the Holocene age deposits based on the most up-to-date information publicly available. - Map of each Wind Farm Zone and Search Area 8 showing water depth (LAT) over the site, including the seabed morphology. - Map of each Wind Farm Zone and Search Area 8 showing the locations of publicly available boreholes and shallow cores. - Maps of the produced grids of the key subsurface interfaces, images of timeslices depicting major geological features and maps of the distribution of interpreted geological features. - Any presentations given during the course of the Supplier’s work shall be provided as both PowerPoint and pdf files. The descriptive report shall be provided both in Word and pdf format. - Recommendations for the scope of the geophysical and geotechnical survey. A memo for each site with recommendations should be delivered separately as Word and pdf files. The recommendations for the geotechnical survey should include 10 proposed locations at a minimum per site. - All deliverables must be according the guidelines in the document Starting Points and Assumptions |
| - DIGITAL DELIVERABLES | - Export of all interpreted horizons in time and/or depth domain. This includes both unit boundaries and geological features, in a format which is compatible with conventual seismic workstation software including IHS Kingdom (e.g. X Y line trace time) - Export of all produced XYZ-grids in the depth domain (m BSF and m LAT). All grids should be clipped to the area of interest. - Four GIS archives (one for each area) to accompany the Supplier’s report, containing a geodatabase and a map document. All spatial data products developed as part of the work (e.g. no-go areas defined during the work, inferred Formation extent polygons, merged bathymetry data, produced unit boundary grids, locations of any cross-sections displayed in the Supplier’s report) shall be compiled into a single ESRI geodatabase. This geodatabase shall also contain information about key background spatial datasets used by the Supplier during the course of the work (e.g. Wind Farm Zone boundaries, locations of previous geotechnical, geophysical and hydrographic Site Investigations) that have not been superseded by new information produced by the Supplier, clipped to the area of interest where appropriate (excluding those items where licensing/copyright arrangements prevent reproduction of these items by the Supplier). The geodatabase shall be accompanied by an ESRI ArcMap document (*.mxd) in which all information in the geodatabase shall be displayed in appropriate grouped layers. The map shall use relative pathnames to the geodatabase content. All information in this map document shall be symbolized consistently with maps presented in the Supplier’s report and shall have layer names that are consistent with references in the Supplier’s report. Naming and symbol convention shall be consistent for all GIS archives. Appropriate metadata shall be produced such that the provenance and usage limitations of each feature class and raster within the geodatabase are indicated by the metadata for each item. A specification for the metadata shall be provided by RVO. - Four interactive visualisations (one for each area) of a 3D legacy data ground model providing the most major information and data collected and produced by the successful Tenderer, including (but not limited to) unit boundary grids, bathymetric data and the location and depth of the provided geotechnical data. The Tenderer is free to propose the software package used for this deliverable (e.g. OpendTect), however the end result needs to be accessible and fully functional with free-to-use. The Tenderer is free to suggest more relevant types of information that can be incorporated in the visualisation. |
| - INTERFACES | - Information will be input for the EIA study to be carried out. - Information gathered in this desk study should provide sufficient information to inform the development of the scope of the geophysical surveys. - Information gathered in this desk study should provide sufficient information to inform the development of the scope of the geotechnical surveys, in particular the determination for suitable locations. - Information from this study will be used to define boundaries of the concession areas. |
| - INPUT DEVELOPMENT PROCESS | ☒ Scope of the planned geophysical surveys ☒ Scope of the planned geotechnical surveys ☒ Scope of the planned integrated ground models ☒ EIA Study |