# Request for Quotation

# Dryer Gas Treatment system 500+

# Requested by

# Blue Phoenix Group B.V.

# July 2022

| Version | Author | Date | Status | Remarks |
| --- | --- | --- | --- | --- |
| 1 | PN | September 5, 2022 | Concept |  |
| 1 | MVS | September 9, 2022 | Approved |  |
| 2 | MVS | September 29, 2022 | Updated |  |
|  |  | Click here to enter a date. | Choose an item. |  |
|  |  | Click here to enter a date. | Choose an item. |  |
|  |  | Click here to enter a date. | Choose an item. |  |

Conﬁdentiality Notice: This document is conﬁdential and contains proprietary information and intellectual property of Blue Phoenix Group. Neither this document nor any of the information contained herein may be reproduced or disclosed under any circumstances without the express written permission of Blue Phoenix Group.

Copyright Notice: © 2022 by Blue Phoenix Group All rights reserved. Any form of reproduction, dissemination, copying, disclosure, modification, distribution and or publication of this material is strictly prohibited.

- Table of Contents

Table of Contents	2

1.	Introduction	3

1.1	Information contracting entity	3

1.2	Contact information	3

1.3	General information Blue Phoenix Group	3

1.4	Project Information	3

2.	Commercial Bid Requirements	4

2.1	Instructions for bidding	4

2.2	Order of precedence	4

2.2.1	Terms & Conditions	4

2.2.2	Confidentiality and Non-Disclosure	5

2.2.3	Award Contract Agreement	5

2.2.4	Codes & Standards	5

2.3	Selection criteria	5

2.4	Ordering procedure	6

3.	Scope of Work	6

3.1	General design	6

3.2	Scope	6

3.2.1	Detail engineering	6

3.2.2	Fabrication	6

3.2.3	Delivery & installation	7

3.3	Project Time schedule	7

4.	Attachments	7

Introduction

Information contracting entity

Delivery Address:

AVR Duiven

Rivierweg 20

6921 PZ Duiven

The Netherlands

Invoice Address:

Blue Phoenix Group B.V.

Watermanweg 106a

3067 GG Rotterdam

The Netherlands

CoC		: 24448294

VAT Nr. 	: NL821359940B01

Project Code: 	: LIFE19/CCM/NL/001219

Contact information

All communication regarding this RFQ proceeds preferably by email with the Project Manager, as per applicable communication protocol with reference LMF-PLN-001-Communication Rev.#.

General information Blue Phoenix Group

Blue Phoenix Group recycles ash and extracts ferrous and nonferrous metals, while producing a final clean mineral for reuse such as aggregates. As the world’s largest ash processor, we achieve operational excellence through our proven processes, professional teams and the highest commitment to health, safety, and the environment.

Our mission to build a sustainable future, challenges our group to constantly improve and adapt to local demands and legal requirements. Our R&D team works hard to find different applications for IBA to ensure that nothing goes to waste. We invest highly in research and technology to transform our ideas into sustainable realities.

Project Information

The LIFE MIBA Filler project aims to demonstrate our breakthrough climate-change mitigation technology that converts MIBA minerals into a low-carbon binder filler for the concrete.

Laboratory-scale research has resulted in a process flow diagram, which we now want to upscale in this MIBA Filler Pilot Plant (demo phase I) before having a full-scale demonstration plant (demo phase II).

This MIBA Filler Pilot Plant will be operational for an estimated operational running time of approx. 3000 hours over a lifetime of 1-1,5 years.

The EU is subsidizing the first demonstration plant under the project name: “LIFE MIBA Filler” with grant agreement reference number LIFE19 CCM/NL/001219, Version LIFE January 2020.

Level 1 time schedule:

Start procurement:	Q1 2022

Start site construction:	Q3 2022

Start commissioning:	Q4 2022

Start operational test period:	Q1 2023

The LIFE MIBA Filler pilot plant will be located in The Netherlands on the premises of AVR Duiven.

Commercial Bid Requirements

The quotation shall be submitted as per this RFQ and the quoted supplies / works shall comply with the referenced specifications, data sheets, standard, etc.

Instructions for bidding

- Bid due date: 14th of October 2022

- The quotation and associated documents may be in the Dutch / English language.

- The quotation shall have a validity of at least 2 weeks.

- The quotation shall be in EUR, excluding VAT.

- All prices shall be quoted including packaging, and delivery DDP Duiven, The Netherlands as per incoterms 2020.

The quotation shall at least have following items:

- Detailed company information

- Reference projects

- Required certificates

- Comprehensive description of the scope of supply, including technical specifications with relevant technical data, to enable a proper evaluation.

- Any deviations from this RFQ and/or the applicable specifications shall be clearly written down in the quotation in a separate paragraph headed “exceptions and deviations”. Otherwise, compliance / inclusion is assumed.

- The quotation shall be priced per item, including all relevant unit rates

- Priced list of options

Order of precedence

In case of any discrepancy between documents, the following order of precedence shall apply:

- Laws, regulations, codes, and standards applicable in The Netherlands

- The Purchase Order & general terms & conditions

- Non-Disclosure Agreement

- Award Contract Agreement

- This RFQ and its attachments

- Bidder’s quotation

Terms & Conditions

The Blue Phoenix Group general terms and conditions shall apply, of which the latest version is attached. The bidder is free to make comments, exclusions and additions to the terms and conditions, but may not reject them as a whole.

The Blue Phoenix Group general terms and conditions shall apply as default, of which the latest version is attached. The file can be found in the tender documents under the name: ‘General_Terms_and_Conditions_Inashco_v4-20170420.

The bidder is free to make comments, exclusions and additions to the terms and conditions.

Alternatively, the bidder may offer in accordance with industry specific terms and conditions, i.e. ORGALIM, FME, Metaalunie.

Bidder’s standard terms and conditions are specifically rejected.

Any mutually agreed deviations to our terms and conditions will be clearly written down in the Purchase Order.

Confidentiality and Non-Disclosure

If not yet in place already, bidder and Blue Phoenix Group shall sign a Non-Disclosure Agreement as per Blue Phoenix Group template prior to order placement.

The file can be found in the tender documents under the name: ‘LMF-TEMP-002 – Template Non-Disclosure Agreement Version ER20210401’

Award Contract Agreement

The bidder shall handover to Blue Phoenix Group a filled-out, duly signed Award Contract Agreement before entering into an agreement. The Award Contract Agreement describe the required back-to-back articles of out LIFE19 Grant Agreement.

The file can be found in the tender documents under the name: ‘LMF-TEMP-001 – Template Award Contract Agreement June 2021’.

Codes & Standards

Even if not mentioned specifically, all supplies and services shall comply with the local codes, standard, rules, regulations and laws in The Netherlands as well as the applicable site safety standards.

Further detailed requirements can be found in the attached specification: LMF-SPEC-035 - Dryer Gas Treatment system 500+ Rev.#, LMF-DWG-011 - PFD Dryer Gas Treatment system 500+ Rev.#, and all other attachments.

Selection criteria

The quotation shall be reviewed on following selection criteria:

- Proven experience in engineering, procurement, and fabrication of filtration equipment equipment, preferably for abrasive and corrosive powders.

- Technology

- Delivery time

- Costs

- Quality of the offered item(s), compliance with the request and completeness

Ordering procedure

We ask bidders in this stage of the project to base their quotation on the currently available specifications, drawings, lists, and other available information and that a total price is offered based on that scope.

Based on this, Blue Phoenix Group will evaluate the received quotations and intends to work towards a contract with the final winning bidder.

Scope of Work

General design

Following basic requirements are set for the design, the plant/equipment/components are designed based on following:

- In full accordance with the requirements of the applicable National and European directives (e.g. Machinery Directive 2006/42/EC).

- The design of the plant will be based on having it fully automated and it shall require as less as possible human interference.

- Designed for 24/7 operation; >8000 h/y (real operational hours are expected to be around 3000 hours per year)

- High availability, only a minimum amount of planned maintenance is required. Simple and robust design.

- Maximum transportation width: 3300 mm, building door opening height is 3500mm.

Scope

Detail engineering, procurement, fabrication, delivery, of dryer gas treatment system components for LIFE MIBA Filler pilot plant as per attached specification: LMF-SPEC-035 - Dryer Gas Treatment system 500+ Rev.#, and all other attachments.

Any items which are not written in this RFQ and/or the applicable specifications, but which are required for the safe, reliable, and stable execution and/or functioning of the supplied equipment shall be deemed to be included in bidder’ scope.

LIFE MIBA Filler pilot plant intents to test various dust extraction technologies in series or stand alone, in order to determine the most efficient and cost-effective solution. Therefore, Pos.500 Scrubber and Pos.520 Dust filter are categorized a 2 separate lots. Tenderers may submit a proposal for each or both lot(s) or propose an alternative solution for either or both lots that achieves the same performance.

Detail engineering

All drawings, calculations of dimensions, wall thickness, material types, power ratings, etc. for tubes, motors, equipment, supports, frames, and instruments. Including advise / comments on our Piping & Instrumentation Diagram (P&ID) concerning functionality, process monitoring & control, material use, maintenance and well working.

Fabrication

- Equipment

- Pumps, valves, auxiliary equipment

- Fans

- Instruments

- Factory Acceptance Testing

Delivery & installation

Transport charges & responsibilities: All items are delivered DDP Duiven, The Netherlands (Incoterms 2020).

Option: Complete installation of the supplied system, including connections to upstream and downstream process equipment.

Project Time schedule

The bidder shall submit a project time schedule from order to handover. The project time schedule contains the most important activities and the key milestones.

Blue Phoenix Group attaches great value to completing the project according to preliminary Key Milestones.

| Key Milestone | Preliminary date |
| --- | --- |
| Detail Engineering completed | 30-11-22 |
| Procurement & Fabrication completed | 10-02-23 |
| FAT completed | 24-02-23 |
| Delivery and Documentation handover | 03-03-23 |

Attachments

The proposal shall be based on this document and following documents and items:

- LMF-SPEC-035 – Dryer Gas Treatment system 500+ Rev.#; describing the specific and technical specifications for the delivery

- LMF-DWG-011 – PFD Dryer Gas Treatment system 500+ Rev.#; giving a schematic overview of the equipment requested for the delivery

- LMF-TEMP-002 – Template Non-Disclosure Agreement Version ER20210401.pdf; describing the required Non-Disclosure Agreement.

- General_Terms_and_Conditions_Inashco_v4-20170420.pdf; describing the general purchase terms and conditions of Blue Phoenix Group.

- LMF – Template Award Contract Agreement June 2021 Rev.#.pdf; describing the required back-to-back articles of out LIFE19 Grant Agreement.

- LMF-PLN-001 – Communication Rev.#.pdf

- LMF-SPEC-001 – Documentation and Drawings Rev.#.pdf; describing the requirements of the requested documentation.