Appendix A05

ConformityList PoR

# CNC Milling Machine 5 Axis

Reference	:	2022 FPL INK 96

Date 		:	11-07-2022

Bijlage A05: Conformitylist PoR

By means of the list of conformity below, it will be assessed whether the Tender meets the requirements as stated in Chapter 6 The Tenderer must indicate the conformity in the table below for the requirements per item.

If one or more of the requirements as stated in chapter 6 are not met, the Tender will be excluded from further assessment. Answering a question with 'agreement', initialing it, means that in the event of the award of the contract, failure by the Tenderer to meet the requirement set will be regarded as non-compliance with the obligations arising from the Agreement to be concluded.

| Requirement | Requirement description | Agreed Tenderer Yes/No |
| --- | --- | --- |
| General machine set-up |  |  |
| Requirement 8.1.1. | The machine design must be a ‘’gantry design’’ | Yes/No |
| Requirement 8.1.2 | The machine must be suited with a X, Y, Z, A and C axis. | Yes/No |
| Requirement 8.1.3 | The machine must have the capability to do 5 axis simultaneously milling jobs. | Yes/No |
| Requirement 8.1.4 | The A-axis must be supported on both sides. | Yes/No |

| Workpiece dimensions, machine travel |  |  |
| --- | --- | --- |
| Requirement 8.2.1. | The X x Y x Z travel must be at least 600 x 500 x 450 mm | Yes/No |
| Requirement 8.2.2 | The milling table must have at least the following dimensions: ⌀ 450 mm , two flat sides with a distance of minimal 350 mm is allowed. | Yes/No |

| Swiveling and rotation range |  |  |
| --- | --- | --- |
| Requirement 8.3.1 | The A-axis must have a (total) swivel travel of at least 220 degrees. | Yes/No |
| Requirement 8.3.2 | The C- axis must be able to rotate 360 degrees. | Yes/No |
| Requirement 8.3.3 | The collision circle must be >750mm | Yes/No |
| Requirement 8.3.4 | The minimal workpiece capacity must be > Ø 450 mm x 300 mm | Yes/No |

| Precision of the machine |  |  |
| --- | --- | --- |
| Requirement 8.4.1. | The position accuracy, in X – Y – Z, must be ≤0.008 mm, measured by VDI/DGQ 3441 standards. | Yes/No |
| Requirement 8.4.2 | Positioning of the A axis must be P = ≤10 arcsec | Yes/No |
| Requirement 8.4.3 | Positioning of the C axis must be P = ≤8 arcsec | Yes/No |

| Spindle and drives |  |  |
| --- | --- | --- |
| Requirement 8.5.1. | The main spindle must be suitable for an HSK A 63 tool shank. | Yes/No |
| Requirement 8.5.2 | The main spindle must be direct driven. | Yes/No |
| Requirement 8.5.3 | The spindle main power must be ≥20 kW | Yes/No |
| Requirement 8.5.4 | Rapid linear traverse must be ≥30 m/min | Yes/No |
| Requirement 8.5.5 | The main spindle speed must be ≥16.000 RPM | Yes/No |

| Workpiece probe system |  |  |
| --- | --- | --- |
| Requirement 8.6.1 | The machine must have a wireless infrared 3D probe system for probing the workpiece. | Yes/No |
| Requirement 8.6.2 | The probe system must have a accuracy of ≤ 2.5 µm. | Yes/No |
| Requirement 8.6.3 | Control and software of the probe system, must be the same brand as the machine controller. | Yes/No |

| Tool measurement system |  |  |
| --- | --- | --- |
| Requirement 8.7.1 | The machine must have a laser measurement system for tool size and length measurements. | Yes/No |
| Requirement 8.7.2 | The laser measurement must have a repeatability of minimal ±0.5 µm 2σ. | Yes/No |
| Requirement 8.7.3 | Control and software of the measurement system, must be the same brand as the machine controller. The probe software/plugin, must be able to directly write their measured parameters to the machine controller parameters/ workpiece offsets. | Yes/No |

| Machine controller, and electronics. |  |  |
| --- | --- | --- |
| Requirement 8.8.1 | The machine control and measurement scale system must be from the same manufacturer. | Yes/No |
| Requirement 8.8.2 | The machinecontrol must be available in the Dutch and English language. | Yes/No |
| Requirement 8.8.3 | The machine must have a MPG which can be used in handheld modes. | Yes/No |

| Toolmagazin |  |  |
| --- | --- | --- |
| Requirement 8.9.1 | The machine must have a toolmagazin, with the capacity of minimal 30 pockets. | Yes/No |
| Requirement 8.9.2 | The toolmagazin should have the capability to be loaded from an external door on the side or behind the machine. | Yes/No |

| Rigidity of the machine |  |  |
| --- | --- | --- |
| Requirement 8.10.1 | The machine base/foundation should be made out of an mineral casted material, such as expoxy granite. | Yes/No |
| Requirement 8.10.2 | The machine base must be able to place on the floor without any anchoring. | Yes/No |

| Accessories |  |  |
| --- | --- | --- |
| Requirement 8.11.1 | The machine must have a filter mist extraction. | Yes/No |
| Requirement 8.11.2 | The machine must have a separate coolant hose with pistol, which can be used by the operator. | Yes/No |

| Coolant, environment, and chip management |  |  |
| --- | --- | --- |
| Requirement 8.12.1 | The machine operation space must be fully enclosed by a rigid and stable enclosure, including chip drawer. The top of the enclosure must be able to open. | Yes/No |
| Requirement 8.12.2 | The machine must be delivered without a chip conveyor. | Yes/No |
| Requirement 8.12.3 | The machine must have a coolant system, pump with a pressure of minimal P ≥ 3bar. | Yes/No |
| Requirement 8.12.4 | The flow of the external coolant system must be ≥45L/min | Yes/No |

| Ergonomics and safety of the machine |  |  |
| --- | --- | --- |
| Requirement 8.13.1 | The machine control panel must be adjustable in height (± 10cm) | Yes/No |
| Requirement 8.14.2 | The machine control panel must stand on a swivel base, it should be swiveled to the tool loading position as well. | Yes/No |
| Requirement 8.14.3 | The machine milling table must be fully reachable from a overhead crane. | Yes/No |
| Requirement 8.14.4 | The machine enclosure top must have the possibility to be opened, when using the overhead crane. | Yes/No |
| Requirement 8.14.5 | The machine, and all accessoiries, must meets the European CE and safety standards. | Yes/No |

| Energy- and air-supply |  |  |
| --- | --- | --- |
| Requirement 8.15.1 | The power consumption of the CNC milling machine must be ≤30kVA | Yes/No |
| Requirement 8.15.2 | The machine should be suitable to fit the main connection by the Dutch standards. (3x400v 50Hz) | Yes/No |
| Requirement 8.15.3 | The machine should be suitable for a 8 – 10 bar air supply. | Yes/No |

| Documentation |  |  |
| --- | --- | --- |
| Requirement 8.16.1 | The supplier must deliver all the (service)manuals, instructions, measurement reports, (CE)certificates in paper or PDF. | Yes/No |

| Delivery, installation, and acceptance |  |  |
| --- | --- | --- |
| Requirement 8.18.1 | The machine supplier, or subcontractor, must be able to do also the electrical and pneumatic installation of the machine, to our excisting electrical and pneumatic installation. TNO provides a CEE form wall outlet and pneumatic outlet (10 bar Orion) on the location of the machine. | Yes/No |
| Requirement 8.18.2 | For maintenance and installation of the machine, all the maintenance personell which enters the TNO facility, must have a valid certificate of conduct, with the following aspects: 11, 12, 37, 41 and 61. | Yes/No |
| Requirement 8.18.3 | The machine supplier, and/or a subcontractor, is responsible for a correct installation of the machine and accessoires. | Yes/No |
| Requirement 8.18.4 | The machine supplier must produce a complete part on the installed machine, which proves the capabilities and accuracy of the machine. This may be in combination with the training. | Yes/No |
| Requirement 8.18.5 | The Tenderer must provide a description for a maintenance contract in which the following elements have been named: 1.detailed description of preventive and curative maintenance work for the milling machine and the frequency with which they should be performed. 2. specification of the maintenance work that can be done by the users themselves executed. 3.specification of the materials included in the maintenance contract. materials and/or parts that are not included in the maintenance contract must be included in Appendix A04 - Price sheet. 4. when outsourcing to a third party, the Tenderer must state this third party. 5. contact persons (account manager and contact person who is authorized to make decisions for contractual and commercial matters + a replacement account manager. 6.how evaluation is given. 1x per year regarding user experience, status of maintenance, malfunctions, solving complaints and developments at the supplier and/or TNO, etc., etc. | Yes/No |
| Requirement 8.18.6 | In its tender, the Tenderer makes a detailed proposal for a purchase protocol in which the correct operation (in accordance with the schedule of requirements) of all functionalities and tolerances are demonstrated. One of the executable parts of the acceptance protocol will be the milling of one or more workpieces, at least one so-called NAS workpiece. This acceptance protocol shows: • The method of how a specification will be verified/tested; • Ensure an efficient method of verification and testing (in terms of time and costs); • Measures to prevent errors in the verification. TNO has the option at all times to make changes to the proposed acceptance protocol. The acceptance protocol will be used to determine whether after delivery can proceed to final acceptance by TNO. In this context, see also Appendix C02 of the General Purchasing Conditions TNO 2022. | Yes/No |

| Training and support |  |  |
| --- | --- | --- |
| Requirement 8.19.1 | The supplier must give a full machine training, on the delivered machine in our workshop, for at least 5 persons. | Yes/No |
| Requirement 8.19.2 | The costs of this user training(s) must be included separately in Appendix A04 - Price sheet. | Yes/No |

| Delivery time |  |  |
| --- | --- | --- |
| Requirement 8.20.1 | The estimated delivery time, starting at the final decision of this tender, must be given in advance. | Yes/No |

Tenderer:

Company name					: ……………………………………………………

Name Person					: ……………………………………………………

Position						: ……………………………………………………

Signature						: ……………………………………………………

Place …………………………….………………, date ……………………………………………………