# Specification 035

# Dryer Gas Treatment system 500+

# Requested by

# Blue Phoenix Group B.V.

# September 2022

| Version | Author | Date | Status | Remarks |
| --- | --- | --- | --- | --- |
| 1 | PN | September 5, 2022 | Concept |  |
| 1 | MVS | September 9, 2022 | Approved |  |
| 2 | MVS | September 30, 2022 | Updated |  |
|  |  | Click here to enter a date. | Choose an item. |  |
|  |  | Click here to enter a date. | Choose an item. |  |
|  |  | Click here to enter a date. | Choose an item. |  |

Conﬁdentiality Notice: This document is conﬁdential and contains proprietary information and intellectual property of Blue Phoenix Group. Neither this document nor any of the information contained herein may be reproduced or disclosed under any circumstances without the express written permission of Blue Phoenix Group.

Copyright Notice: © 2022 by Blue Phoenix Group All rights reserved. Any form of reproduction, dissemination, copying, disclosure, modification, distribution and or publication of this material is strictly prohibited.

- Table of Contents

Table of Contents	2

1.	Introduction	3

2.	System Overview	3

2.1	Objective	3

2.2	Dryer gas treatment system	4

2.3	Scope	4

3.	Product characteristics	5

3.1	Off-gas from Dryer	5

4.	Capacity & Dimensions characteristics	5

4.1	General	5

5.	Deliverables	5

5.1	Engineering & fabrication	5

5.2	Documents	6

Introduction

This document describes the specific and technical specifications for the Dryer Gas Treatment system 500+

This document is part of the Request for Quotation LMF-RFQ-020 - Dryer Gas Treatment system 500+

The scope consists globally of two filter systems, pumps, valves, instrumentation, and tubing.

System Overview

Objective

The purpose of the system is to filter the off-gas stream from the dryer. The off-gas contains air, water (damp/vapour) and dust particles. Since this is a pilot-plant, multiple routings through the dryer gas treatment system shall be tested to determine the most effective filtering method.

The dryer gas treatment system may consist of 4 process steps:

- Wet scrubber: remove pollutants from the water vapour that is released by a thermal dryer system

- Heating element (if required): re-heat the air flow to avoid condensation of water vapour in the dust filter

- Dust filter: remove pollutants from the wet air

- Fan (if required): extract air from the dryer/scrubber and discharge treated gas into the air / recirculate into dryer system

There are 2 potential outlets of the dryer gas treatment system:

- Discharge the treated gas into the air

- Recirculate the treated gas into the dryer system as sweep gas

The different routing options should consist of the following:

- Wet scrubber  Discharge to the air.

- Wet scrubber  Recirculate to the dryer system.

- Wet scrubber + Heater + Dust filter  Discharge to the air

- Wet scrubber + Heater + Dust filter  Recirculate to the dryer system

- Dust filter  Discharge to the air

- Dust filter  Recirculate to the dryer system

- Untreated  Discharge to the air

The different options of the off-gas streams will be regulated with hand operated valves.

LMF-DWG-011-PFD Dryer Gas Treatment system 500+ shows the schematic view of the desired off-gas treatment system.

Dryer gas treatment system

This specification applies to the following equipment items:

- Pos. 500 Scrubber

  - Air flow in: ±116 Air kg/h + 290 kg/h water + 10 kg/h dust particles

  - Temperature in: ± 105°C

  - Cooling spray water in: ± 40°C

  - Condensate will recirculate and excess water will be directed to the dewatering

  - Air flow out will go to:

    - Direct exhaust

    - Recirculated back to the dryer as sweep gas

    - Dust filter

- Pos. 520 Dust filter:

  - Air flow in:

    - By-pass Pos.500 Scrubber: ±116 Air kg/h + 290 kg/h water + 10 kg/h dust particles

    - Exhaust from Pos.500 Scrubber: To be determined by supplier

  - Temperature in:

    - By-pass Pos. 500 Scrubber: ±105°C

    - Exhaust from Pos. 500 Scrubber: 50-60°C (estimated)

- Pos. 510 Heating element:

  - If necessary for a better functionality of the dust filter.

- Pos. 530 Fan:

  - Exhaust and/or recirculation

- Pos. 540 Fan:

  - Exhaust and/or recirculation

A quotation of the complete assembly or the selection of the offered equipment items should be given.

Scope

- Engineering of complete assembly or the selection of the offered equipment items is included in the scope.

- Fabrication, assembly and installation of all equipment, tubing, valves, instrumentation, and auxiliaries is included in the scope.

- Installation includes making all connections to up-/downstream equipment and supports for tubing, and equipment.

Product characteristics

Output: dust emissions into open air

The residual dust emission after dryer gas treatment shall be well below the dust emission limit value of 5 mg/Nm3.

Input: Off-gas from Dryer

| Off-gas | ±116 Air kg/h + 290 kg/h water + 10 kg/h dust |
| --- | --- |
| Flow | 500 m3/h |
| Temperature | ±105°C |
| Dust | Treated bottom ash |
| Chloride content in dust | 1% |
| Dust particle size: mean | 12 µm |
| Dust particle size: D90 | 18 µm |
| Dust particle size: D50 | 7 µm |
| Dust particle size: D10 | 1.8 µm |
| Dryer pressure | -1 mbarg - -4 mbarg |

Capacity & Dimensions characteristics

General

Detailed construction engineering, in full accordance with the requirements of the applicable National and European directives (e.g. Machinery Directive 2006/42/EC).

Material:	The material used must be resistant to product characteristics and process conditions.

Features:	Designed for full automation and minimal human interference. 
The system components must be easy to clean and maintain. Preference for automatic cleaning.

Supports:	Support for tubing, and equipment.

ATEX:	No ATEX requirements
ATEX safety studies is still ongoing, if safety studies conclude that ATEX requirements are applicable, this will be considered a scope change.

Deliverables

Engineering & fabrication

Detailed engineering, procurement, fabrication, installation of Dryer gas treatment system 500+. In accordance with the Machinery Directive 2006/42/EC, applicable standards and specifications.

Preservation, according to the specification 'Preservation', optional supplier standard

Required/recommended spare/wear parts.

Motors: make SEW (preferred), optional supplier standard

Documents

- Documentation & certificates & EC Declaration of Conformity according to Machinery Directive 2006/42/EC.

- Document deliveries according to specification LMF-SPEC-001 – Documentation and Drawings Rev.#.