TITLE: METH045_ARISE tissue DNA extraction

| STATUS | Under review |
| --- | --- |
| LAST UPDATE |  |
| DOCUMENT OWNER | Nafiesa Ibrahim |
| APPROVER / GOEDKEURDER |  |

| VERSION / DATE | REASON for NEW VERSION |
| --- | --- |
| 1.0 / 1-2-23 | Updated to a new template. |

1	INTRODUCTION

1.1	SCOPE

The extraction procedures are based on silica coated magnetic nanoparticles. DNA binds to the silica in the presence of high concentration salts and is eluted from the silica in water or low-salt buffer. This DNA purification principle can be used after an effective DNA lysis.

NOTE: The DNA extraction protocol described in this document is based on a homemade protocol.

Important!

When preparing chemicals, wear protective clothing: lab coat, gloves and safety glasses.

Be aware of the dangers of each chemical. This is indicated on the label and more information can be found in the Safety Data Sheets (SDS).

Use the powder weighing cabinet and fume cabinet to weigh out and prepare toxic chemicals.

Use the table of content to go to the desired process. Before performing an extraction, make sure there are enough reagents.

This document applies when a DNA extraction from tissue should be performed by using homemade/ commercial beads and reagents in combination with the KingFisher extraction robot.

1.2	OBJECTIVE

This method describes the process of a high-throughput DNA extraction from tissue using the KingFisher extraction robot.

1.3	DEFINITIONS AND ABBREVIATIONS

ARISE 		Authoritative and Rapid Identification System for Essential biodiversity information

DNA 		Deoxyribonucleic acid

KF		KingFisher extraction robot

protK		Proteinase K

O/N		Overnight

EtOH 		Ethanol

MilliQ 		Ultrapure water

MU		Modular Universal

BOMB 		Bio-On-Magnetic-Beads

Serasil beads	SeraSil-Mag 400 silica coated superparamagnetic beads

2	RESPONSIBILITIES & PEOPLE

SHEET025_WORKFLOW_OVERVIEW_SOPs_SD

2.1	RESPONSIBILITIES

| FUNCTION | ROLES and RESPONSIBILITIES |
| --- | --- |
| (Senior) Research Technician, Lab workers (students, guests) | Is able to follow the procedure, gives feedback in case of important changes. |
| Author | Revise this procedure when there are essential updates. |
| Head Laboratories | Is end responsible for operational all methods. |

2.2	LIST OF PEOPLE ADDRESSED

SHEET010_OVERVIEW_ROOM_MANAGERS_&_TELEPHONE NUMBERS

Optionally include specific people e.g. apparatus managers.

3	FLOW CHART

N/A

4	Method

4.1	Buffers

Reagents, solutions, and chemicals, see 4.2 Making MU-DNA reagents

  - Lysis Buffer A

  - Lysis Buffer B

  - ProtK (VWR, 1.24568.0500)

  - 15% tween20

  - Tissue binding Buffer

  - EtOH 96%

  - isopropanol

  - SeraSil-Mag™ 400 magnetic beads (Sigma, GE29357371)

4.1.1	Tissue lysis

Important: Heat lysis B up if the buffer is crystallized.

- Place up to 40 mg of tissue into a 1.5 ml collection tube (96 well, QIAGEN) or 1.5 ml microtube

- Add 50 μL* of Lysis Buffer A, 50 μL* of Lysis Buffer B, 7.7 μL* 15% tween and 7.7 μL* ProtK

  - *Volumes can be adjusted depending on the amount of starting material

- Optional: Grind tissue with a tube pestle or cut the tissue with a scalpel, vortex briefly.

  - IF NON-DESTRUCTIVE DO NOT CUT OR GRIND

- Incubate at 55°C, 400 RPM on a shaker incubator or with occasional vortexing(>3 hours or overnight)

~ Possible stop store at 4 °C ~

4.1.2	Continue lysis step

- Centrifuge the plate/tube at 2 minutes at 10.000 xg or 5 minutes at 2250 xg at room temperature

- For the Manual extraction (<45 samples): Transfer 75 µL* supernatant to a new 1.5 ml tube without disturbing the pellet, and proceed with step 1 (4.1.3).

- For the KingFisher extraction (>45 samples): Transfer 75 µL* supernatant to a KF deep well plate without disturbing the pellet and proceed with step 1 (4.1.4).

*Volumes can be adjusted depending on the amount of starting material

Troubleshooting

- The salts present in the Lysis Buffer B and the Tissue Binding Buffer can crystallize, heat the buffers at ~55°C for 10 minutes, or until the salts are dissolved.

4.1.3	Manual DNA extraction

- Add 20 µL SeraSil-Mag™ 400 beads and 150 µL* (2.0x the Lysate volume) of Tissue Binding Buffer to each sample, shake at 500 rpm for 5 minutes at RT.

- Settle the beads on the magnetic rack and discard the supernatant

- Remove the tubes from the magnetic rack and add 500 µL isopropanol. Shake at RT at 500 rpm for 2 minutes

- Settle the beads on the magnet and discard the supernatant

- Wash twice with 400 µL 96% ethanol

- Remove all ethanol and dry the beads for approximately 5-10 minutes

- Silica-coated beads should be completely dried (at 50 °C).

- Add 50-70 µL* elution buffer/ MilliQ to each tube and shake for at least 5 minutes .

- Settle the magnetic beads on the magnet and transfer the supernatant to a new tube/working plate.

- Continue to ‘4.1.4 Measuring and storing DNA’ for measuring and DNA storage.

4.1.4	KingFisher DNA extraction

- Add 20 µL SeraSil-Mag™ 400 beads and 150 μL* (2.0x the Lysate volume) of Tissue Binding Buffer to each sample

- Fill one KF deep-well plate with 500 µl isopropanol, two KF deep-well plates with 400 µl 96% ethanol and one KF elution plate with 50-70 µl* of elution buffer/ MilliQ

- Choose the right protocol on the KF (KF_BOMB_tissue_96_2x_wash)

  - (For how to use the KingFisher extraction robot see *WI_AP007_KF-01.)

- Continue to ‘4.1.6 Measuring and storing DNA’ for measuring and DNA storage

* The entire process is scalable depending on initial sample weight or transferred lysate amount. Where relevant, scalable volumes are shown in brackets next to reagent volumes used, e.g. (2 X volume).

# 4.1.5	Measuring and storing DNA

- Measure the purity/concentration on Lunatic

- When the KF is finished the eluate can be divided among a DNA Work Plate and a DNA Storage Plate. For this you can use the CyBio Selma Liquid Handler see *WI_AP028_SELMA-01.

- The DNA Work Plate contains 15 µl of each sample and should be placed in the fridge.

- DNA Storage Plate contains the rest (± 55 µl) of each sample and should be placed in the -80◦C DNA-bank

- Scan the NCBN plate with the BioMicroLab Sample Scan-Mini see METH027_SCAN_SAMPLE SHEET_PCR_BASECLEAR, fill NGS1_SampleRegistration_v0.8_TEMPLATE in and store according to WI_AP036_FREEZER_&_FRIDGE_MANAGEMENT

4.2	Making MU-DNA reagents

4.2.1	Chemicals

- Guanidine thiocyanate (CAS: 593-84-0; DA.DW2.L.02.03.CABS-10)

- Trisodium phosphate dodecahydrate (CAS: 10101-89-0; DA.DW2.L.02.03.CABS-09)

- Sodium chloride (CAS: 7647-14-5; DA.DW2.L.02.03.CABS-09)

- Tris base (CAS: 77-86-1; DA.DW2.L.02.03.CABS-09)

- Disodium EDTA dihydrate (CAS: 6381-92-6; DA.DW2.L.02.03.CABS-10)

- Calcium chloride dihydrate (CAS: 10035-04-8; DA.DW2.L.02.03.CABS-10)

- Guanidine hydrochloride (CAS: 50-01-1; DA.DW2.L.02.03.CABS-10)

- Proteinase K (CAS: 39450-01-6)

- 100% Ethanol (CAS: 64-17-5; DA.DW2.L.02.03.CABS-13)

- TWEEN® 20 (CAS: )

4.2.2	Equipments

- Glass bottles

- Parafilm

- Magnetic stirrer with heating

- pH meter

- Balance

4.2.3	Stock solution

The reagents are prepared from the stock solutions. Check if there is some stock left before you make it.

1 M Tris HCl (pH 8):

- Dissolve 15.7 g of Tris base in 50 mL milliQ

- Adjust to pH 8 with 5 M HCL

- Bring the total volume to 100 mL with milliQ

0.5 M EDTA (pH 8):

- Add 18.6 g of disodium EDTA dihydrate in 50 mL milliQ

- Add a magnetic flea and place on a magnetic stirring plate to mix the solution

- Adjust to pH 8 with 5 M NaOH (The EDTA salt will not go into solution until the pH reaches 8.0)

- Bring the volume to 100 mL with milliQ

5.5 M Guanidine HCl:

- Dissolve 52.6 g guanidine hydrochloride in 40 mL milliQ

- Stir until dissolved

- Adjust the volume to 100 mL with milliQ

4.2.3	Reagents

Lysis Buffer A:

- Add 13.4 mL 1 M Tris HCl (pH 8) to 50 mL milliQ

- 10.6 mL of 0.5 M EDTA (pH 8)

- Stir the mixture until all solids are dissolved

- Adjust to pH 9.0 with 5 M HCl

- Bring to a final volume of 100 ml with milliQ

Lysis Buffer  B:

- Add 3.4 g guanidine thiocyanate to 50 ml milliQ

- 17.4 g trisodium phosphate dodecahydrate

- 0.4 g sodium chloride

- Stir on a heated magnetic stirrer until all solids are dissolved

- Adjust to pH 9.0 with 5 M HCl

- Bring to a final volume of 100 ml with milliQ

ProtK solution:

- Add 2.08 mL PB buffer to the glass vial containing 75 mg of proteinase K

- Thoroughly mix the Proteinase K until completely dissolved

- The ProtK buffer should have a final concentration of 36 mg/mL

15% Tween:

- Add 15 ml of Tween20 to 85 ml  MilliQ

- Stir the mixture gently to mix (mixing hard creates bubbles)

Tissue Binding Buffer:

- Add 50 ml 5.5 M Guanidine HCl to 50 ml 100% ethanol (1:1)

- Vortex briefly and adjust the pH to 4.0 according to the instructions below

- Prepare a Lysis mixture by mixing 2,17 ml lysis buffer A, 2,17 ml lysis buffer B, 330 µL Pb buffer (MN) and 330 µL 15% Tween.

- Make a “calibration solution” by adding 10 ml tissue binding solution to 5 ml prepared Lysis Mixture in a 50 ml tube

- Adjust to pH4 - pH4,5 with 5M HCl and take note of the volume 5M HCl added (this should be approximately 350 µL 5 M HCl).

    - Make sure the temperature of the mixture is at Room Temperature.

- Write down the final volume of 5M HCl added, and dispose of the calibration solution.

- Now take the desired volume of Tissue Binding Buffer and add the same volume 5M HCl added to the Ph test solution for every 10 ml of tissue binding buffer. (B.v if 350 ul was added to the pH test solution you would add 10 times 350 ul (3.5 ml) to 100ml of tissue binding solution.)

- Verify you got the right concentration by adding 10 ml of binding solution to 5 ml of pH test solution and measure the pH.

If a new lysis buffer B is made, the verification step must be performed again with the new lysis buffer B in the pH test solution to check that the buffers still combine to a pH of 4,0 - 4,5.

Elution/TE Buffer:

- Add 1 mL 1 M Tris HCl (pH 8) to 75 mL milliQ

- Add 0.2 mL 0.5 M EDTA (pH 8)

- Bring to a final volume of 100 ml with milliQ

- Vortex/stir briefly to mix

5	RELATED/ASSOCIATED DOCUMENTS

See WI_AP007_KF-01 for instructions on how to use the KingFisher extraction robot

See WI_AP028_SELMA-01 for instructions on how to use the Selma

See METH027_SCAN_SAMPLESHEET_PCR_BASECLEAR and WI_AP036_FREEZER_&_FRIDGE_MANAGEMENT for instructions on how to store DNA extracts in DNA stockplate

See Lunatic For instructions on how to use the Lunatic

SHEET010_OVERVIEW_ROOM_MANAGERS_&_TELEPHONE NUMBERS

SHEET025_WORKFLOW_OVERVIEW_SOPs_SD

6	LITERATURE

Oberacker P, Stepper P, et al. ‘Bio-On-Magnetic-Beads (BOMB): Open platform for high-throughput nucleic acid extraction and manipulation’. PLoS Biol. 2019;17(1):e3000107. Published 2019 Jan 10. doi:10.1371/journal.pbio.3000107