User requirements specification for Laboratory Information Management system (LIMS)

Version 1.0

| DOCUMENT APPROVAL / authorisation |  |  |
| --- | --- | --- |
| Role | Name | Signature and Date |
| Author | Hassan Barre |  |
| SME | Maikel Broekgerrits |  |
| Process Owner | Ferdinand van Sloten |  |

Version history

| Version | Date | Description |
| --- | --- | --- |
| 0.1 | 08-aug-2022 | Initial document |
| 0.2 | 19-oct-2022 | Updated after review |
| 0.3 | 24-oct-2022 | Updated after review |
| 1.0 | 26-oct-2022 | Final |

Index

1.	Background	3

2.	Purpose	3

3.	Process Description	3

4.	Abbreviations	3

5.	Requirement Definitions	4

6.	System Requirements	4

6.1	Regulatory Requirements (RE)	4

6.2	General Requirements (GR)	5

6.3	Operational and Functional Requirements (FU)	6

6.4	Data Migration Requirements (DM)	11

6.5	Audit Trail Requirements (AU)	11

6.6	Human Interface Requirements (HU)	11

6.7	Reporting Requirements (RE)	12

6.8	Security, Electronic Record and Electronic Signature Requirements (ES)	13

6.9	System Interface Requirements (IN)	13

6.10	IT (Infrastructure) Requirements (IT)	13

7.	Scope of Supply (SC)	14

8.	Related & Referenced Documents	14

Background

Aqualysis is an accredited laboratory for water research. We conduct hydrobiological, bacteriological and chemical

research using modern analysis techniques. To manage the information workflow a Laboratory Information Management System (LIMS) is used. The current LIMS (QMA) application dates from 2007 and is no longer supported by updates/upgrades. This is a liability for the business continuity and the system is in need of replacement.

Purpose

The purpose of this document is to specify the user requirements for LIMS for use at the Aqualysis Facility.

The User Requirement Specification (URS) captures the business needs. It describes the features or capabilities needed to meet the business goals and objectives of the Laboratory information management system

This User Requirements Specification provides a basis for:

- Communicating the expectations of the solution to the vendor.

- Providing a platform for system design, configuration, build and testing; The URS is the basis for the functional specification that specifies how the required functionality is implanted.

- Identification of requirements subject to validation / qualification testing

- Contractual agreement between Aqualysis and the supplier.

Process Description

For a detailed process description refer to the document: “Processen en Functionaliteiten LIMS”

Abbreviations

| Term | Definition |
| --- | --- |
| ELN | Electronic Laboratory Notebook |
| DTAP | Development, testing, acceptance and production |
| URS | User Requirement Specification |
| CoA | Certificate of analysis |
| SOP | Standard operating procedure |
| OOS | Out of Specification |
| OOL | Out of Limit |
| CLE | Confirmed Laboratory Error |
| CET | Central European Time |
| QMA | Quality Manager Aqua |
| SME | Subject Matter Expert |
| Aquo | The Aquo standard (Aquo) is a Dutch standard for the exchange of data within the water sector |

Requirement Definitions

Below are the definitions for business priority designation

| Priority | Meaning |
| --- | --- |
| H | Mandatory: critical business process requirement, regulatory requirement, direct impact on quality |
| M | Beneficial business process requirement, indirect impact on quality |
| L | Nice to have improvement |

The supplier must adhere to the content of this document; any deviations proposed must be clearly identified and justified by the supplier. Compliance with the agreed contents is a contractual requirement of the purchase order and any costs associated with meeting the agreed requirements must be included in the purchase price.

System Requirements

Requirements are grouped per topic.

  - Regulatory Requirements (RR)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| RR001 | The system must be fully compliant with NEN-EN-ISO/IEC 17025 requirements | H |
| RR002 | Written procedures must be provided for the operation and maintenance of the computerised system | M |
| RR003 | Audit trail records must be read only. They shall not be editable, and shall be protected from accidental or intentional deletion or modification. | H |
| RR004 | Authorized user must be able to configure reporting templates according to Aquo rules, facilitating exchanging of data conforms Aquo standard. | H |

  - General Requirements (GR)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| General |  |  |
| GR001 | The system must be Commercial of the Shelf (COTS) | H |
| GR002 | The system must provide functionality to transfer newly developed functionality or static data from one environment to the other in a controlled manner (DTAP approach). | H |
| GR003 | The system must provide a mechanism to allow a customer read-only access to stored data. (Customer is only able to see own data) | H |
| GR004 | The system must be accessible through the network of Aqualysis | H |
| GR005 | The system must be able to manage (import, export, reports or as an attachment) different file types (e.g. pdf, xlsx, CSV txt, or JPEG) | H |
| GR006 | The system should contain functionality to automatically alert or notify users of required actions through email | M |
| GR007 | An authorized user should be able to expand the standard statuses in the system with user-defined statuses that can be used in the configuration of the workflows | M |
| GR008 | The system should contain the possibility of introducing an Electronic Laboratory Notebook (ELN) (or equivalent) functionality to facilitate paperless workflows in the future. | M |
| GR009 | It should be possible to configure different workflows for different sample types. (e.g. Hydrobiologie, Chemie) | H |
| GR010 | It should be possible for the system to validate user input in fields against configured (field) limits. (e.g. a pH measurement result entry cannot be lower than 1 or higher than 14) | L |
| GR011 | The system must be installed using Dutch as the base language. | M |
| GR012 | The system should provide Offline Mode functionality to address sampling areas that are not internet enabled | M |
| GR013 | The system must be able to cope with different price units (Euro/Points) | H |
| GR014 | Authorized user must be able change object status (e.g. project cancellation/reactivation) | H |
| GR015 | It must not be possible to delete data from the system, it should only be possible to deactivate data by changed the status to: (e.g canceled, rejected, disable) | H |
| GR016 | Authorized user must be able to manage access using role-based access control | H |

  - Operational and Functional Requirements (FU)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| Customer |  |  |
| FU001 | Authorized user must be able to create a customer | H |
| FU002 | A customer must contain (but not limited to) the following customer information: - Customer name - Customer description - Status - Customer group - Billing information like: - Taxes information - Debtor account number | H |
| FU003 | The system should contain customer portal functionality to facilitate: - Project planning initiation from the customer site - Project reporting like: - Project progress report - Project annual reports | M |
| Project |  |  |
| FU004 | Authorized user must be able to create a project | H |
| FU005 | Authorized user must be able to create a sub project | H |
| FU006 | A project must contain (but not limited to) the following project information: - Project code - Project description - Status - Project type (Parent/Child) - Contacts - Project attributes like: - Project start date - Project end date - Client - Fixed project price - Project reduction (%) - Project raise (%) - Project priority - Samples delivered by - Billing information: - Price per Point - Price UOM - Transportation costs - Invoice Reference - invoice frequency - invoice validity period - Taxes information - Reporting information: - When to report - Reporting methods | H |

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| Planning |  |  |
| FU007 | The system must contain planning module or have equivalent functionality | H |
| FU008 | Authorized user must be able to import planning data into the system by direct import of (data) file types: e.g. excel, CSV, txt, xml. | H |
|  | The system must have the ability to generate an daily, weekly, or monthly planning data, including sampling ranges based on existing object data, like (but not limited to): - Client - Project code - Sampling frequency - Sample-code - Description - Sampling location - Coordination (WGS84) (x y) - Sampling date and time - Sampled by AQL (YES/NO) - Sample priority - Sample start date and time - Sample end date and time | L |
| FU009 | It should be possible for authorized users to adjust the plan period | M |
| FU010 | The system must have the ability to generate status for the data sent to the planning tool (ServiceWare) used by AQL | M |
| - Sample reception |  |  |
| FU011 | The system must be able to use the barcode on a label to trigger automatic actions in the system. For example, sample receival by scanning a barcode. | H |
| FU012 | Authorized user must be able to change the priority of a sample | H |
| FU013 | Authorized user must be able to change the priority of a analysis | H |
| FU014 | Authorized user must be able to import data/information/results into the system by direct import of (data) file types, e.g. excel, pdf, CSV, txt, xml. | H |
| FU015 | It should be possible for a user to generate an automatically filled request letter for an external lab when a sample or analysis for an external lab is received. | M |
| FU016 | The system should be able to provide a comprehensive view of all projects, samples and analysis in the system, using a color-coded status view of the current and scheduled samples through user-configurable templates, all without requiring additional programming. | M |
| FU017 | The system should be able to generate an incremental lab-number when sample is received | M |
| - Analysis |  |  |
| FU018 | The system should be able to give an automated warning to pre-defined users when all samples of a specific project have been taken. (Not yet received) | M |
| FU019 | The system should be able to give an automated warning to pre-defined users when all samples of a specific project have been analysed and are approved | M |
| FU020 | It should be possible for users to get an overview of unperformed analysis, and an overview of performed analysis, pending or approval, sorted by due date and priority. | H |
| FU021 | The system must automatically generate a report containing results of analysis according to a pre-determined template. (CoA, not containing all analysis of the sample, but only pre-defined analysis). | H |
| FU022 | It should be possible for an analyst to generate a configurable structured list of work (“analytical batch”) to perform. | M |
| FU023 | Authorized user must be able to initiate a repeat or re-analysis of an analytical batch or an individual analysis when the analysis has failed internal controls (invalid analysis). | H |
| FU024 | The analytical batch must contain standard and reference samples (Control samples / Blanco’s) if so configured. | H |
| FU025 | The sequence of the samples, standard and reference samples should be defined in an configurable template for the analytical batch. | H |
| FU026 | Authorized user must be able to remove or add a sample or analysis from an analytical batch. | H |
| FU027 | Result fields must allow at minimum the following: - numerical, alpha-numerical, or pre-set values. | H |
| FU028 | Authorized user must be able to configure rounding rules on result fields. The non-rounded input must be retained as well for calculation purpose | H |
| FU029 | Authorized user must be able to configure different conservation date for different sample types. | H |
| FU030 | Authorized user must be able to configure different conservation date for different analysis | H |
| FU031 | When a user has entered a result, it should be possible to automatically round to a pre-configured number of significant numbers. | M |
| FU032 | Authorized user must be able to re-enter results. Both the old and the new version must be retained. A justification for the change must be mandatory | H |
| FU033 | Out of trend rules should be configurable by authorized users. | H |
| FU034 | The detection limit should be configurable by authorized users. | H |
| FU035 | It should be possible for the system to automatically add an analysis to a sample based on a result from another analysis. | L |
| FU036 | It should be possible to generate a report listing all samples to be discarded | M |
| FU037 | It should be possible for users to assign a storage location to samples. | L |
| FU038 | It should be possible to configure groups of analysis and assign the group to a sample. | M |
| FU039 | It should be possible for authorized users to add extra analysis to a sample. | H |
| FU040 | It should be possible to configure a sequence of analysis, making sure that one analysis is performed before another. | L |
| FU041 | Authorized users must be able to change or add extra information to a sample or analysis. (e.g. comments, or attachments) | H |
| FU042 | It should be possible to select default pre-programmed comments and add to a sample or analysis. | M |
| FU043 | It should be possible to add internal comments to a sample or analysis which is not visible for users with visitor (read-only access) role. | M |
| FU044 | Authorized user must be able to configure a workflow for ad-hoc analysis | M |
| FU045 | The system should be able to group ad-hoc analysis requests based on extra information like a project/ description number so that it is visible in the system that they belong to the same request, project. | L |
| FU046 | The system must indicate the volume or quantity that needs to be sampled for each analysis | H |
| FU047 | It should be possible to add the necessary packaging to an analysis | M |
| FU048 | The system should indicate the storage condition for the sample. And max hold time if needed | M |
| FU049 | It should be possible for users to assign priority levels to, samples and analysis that need to be performed. | M |
| FU050 | It should be possible in the system to split samples into pre-configured sub samples. | M |
| FU051 | It should be possible to link different samples together, whereby reporting only follows when all samples have been approved by the supervisor. | M |
| FU052 | User-input fields must be set to read-only after approval workflow steps have been performed. | H |
| FU053 | Authorized user must be able to change the status of a sample (e.g. from approved to online or from online to pending) | H |
| FU054 | Authorized user must be able to change the status of a analysis (e.g. from approved to online or from online to pending) | H |
| FU055 | It must be possible to specify if the analysis is performed by an external lab | H |
| - Specifications |  |  |
| FU056 | The system must visually signal out of spec and out of limit results on analysis and component level. | H |
| FU057 | The system should be able to send notifications to specific users when out of specification or out of limit results are found. | M |
| FU058 | It should be possible to configure a value that is displayed by the system when an entered result is outside of a detection limit (for example ‘<20’ when the limit is 20 and the entered result 18) | M |
| FU059 | It should be possible for the system to automatically check results against “out of trend” situations and visually signal the end-user that an out of trend has occurred. | M |
| FU060 | The system should automatically check against detection limits when a result is saved. | M |
| - Trending |  |  |
| FU061 | The system must automatically process control sample results into a configurable Control chart. | H |
| FU062 | The system must be able to apply control chart rules, (NEN6603) with associated statistical calculations such as F and T testing and uncontrolled quality. | H |
| FU063 | The system should be able to apply Z-control chart rules according to: NEN6603 | M |
| FU064 | It must be possible to configure automatic actions when an out of control is detected by the system. | H |
| FU065 | It should be possible to classify out of control results as systematic or incidental. In addition the option to include or exclude results from the statistics. | M |
| - Sample management |  |  |
| FU066 | The system should provide functionality for inventory and management of chemicals, reagents and other consumables. | M |
| FU067 | The system should provide functionality for inventory and management of standards and control samples. | M |
| FU068 | Standards and control samples should not be selectable for use in the system without warnings if their expiry date has passed. | M |
| FU069 | The system should be able to automatically warns if the expiry date has passed for Reagents and chemicals | M |
| FU070 | The system should be able to automatically warns specified users when inventory counts reach a definable threshold and either prompt for or process a reorder | L |
| FU071 | The system should include comprehensive scheduling, tracking, and flow management of samples | M |
| FU072 | The system must be able to determine whether the preservation period has been exceeded on the basis of the date of sampling and date in progress and make a comment on the report. | M |
| - Instrument management |  |  |
| FU073 | The system should contain configurable instrument inventory and management functionality | M |
| FU074 | The instrument inventory should support a hierarchical structure of instruments with different parts, configuration items or priority | L |
| FU075 | It should be possible for authorized users to configure the instruments with different parts or configuration items with their own calibration and maintenance dates. | M |
| FU076 | The system should provide functionality for maintaining an electronic logbook for the instrument. | M |
| FU077 | It should be possible for authorized users to configure and schedule other actions than maintenance and calibration. For example, periodic review | L |
| FU078 | It should be possible for authorized users to configure extra information fields in the instrument/computerized system inventories. | L |
| FU079 | It should be possible to display a forecast of scheduled actions for all instruments per time period (for example in the next month). | M |
| FU080 | Scheduled equipment actions that have not been performed should be visible in the system. | M |
| FU081 | It should be possible to configure a reference between equipment and the analysis it is used for. | M |
| FU082 | It should be possible for users to execute actions like calibration and qualification directly from the instrument management functionality | L |
| FU083 | The system should be able to track and report on the usage of attached laboratory instruments. | M |
| - Deviations |  |  |
| FU084 | It should be possible to configure LIMS deviation records of different types to facilitate different workflows. Types should include OOS/OOL and CLE (confirmed laboratory error). | L |
| - Calculations |  |  |
| FU085 | Authorized user must be able to configure calculations that use all data fields (“Inter-analysis calculation”) | H |
| FU086 | The system permits user-generated and modifiable (for authorized user) calculations to be applied to analysis if required | M |
| - Analyst Qualifications |  |  |
| FU087 | Authorized user must be able to grant authorization to analyst to perform particular analyses (e.g. based on gained qualifications) | H |
| FU088 | Analyst qualifications should be updatable according to configurable business rules. | M |
| FU089 | Records containing analyst authorisation history in the system should be available and maintained | M |
| - Invoicing |  |  |
| FU090 | The system should be able to automatically generate invoice when analysis is finalized. (analysis are completed and COA (Analyse rapport) is sent) | M |
| FU091 | The system should be able to automatically generate E-invoice when analysis is finalized. (Analysis are completed and COA (Analyse rapport) is sent) | M |
| FU092 | Authorized user must be able to manually generate invoice when analysis is finalized. (analysis are completed and COA (Analyse rapport) is sent) | M |
| FU093 | Authorized user must be able to configure invoicing rules for different projects. | M |

  - Data Migration Requirements (DM)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| DM001 | It should be possible to migrate existing static data to the new system | M |
| DM002 | It should be possible to migrate existing dynamic and static data through validated tools facilitating a mass data load. | M |

  - Audit Trail Requirements (AU)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| AU001 | The electronic audit trail must be generated automatically | H |
| AU002 | The audit trail must be human readable. | H |
| AU003 | The audit trail must record the identity of the operator, the action that creates, modifies or deletes electronic records, including any changed values (old and new), justification or reason and the date and time of the action. | M |
| AU004 | Audit trail entries must be protected against change or deletion by any individual. | H |
| AU005 | The system must have the ability to generate accurate and complete copies of audit trail records in both human readable (printable) and electronic form | H |
| AU006 | Time of the audit trail must be related to a predefined time standard used on the (application/database) server. | M |
| AU007 | The audit trail must be readily available for inspection (review). | M |
| AU008 | The system should record all login attempts whether successful of not. | M |
| AU009 | There should be functions available to make queries on the audit trail to be able to provide the exact filtered information needed. | H |
| AU010 | The audit trail should contain all changes to master data with user stamp and timestamp | H |

  - Human Interface Requirements (HU)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| HU001 | The system should be configurable to only show those menus, functions and data that are required for the user role(s) of the user that is logged in to the system. | H |
| HU002 | The system should be able to automatically retain entered information in an individual field after the press of the "enter/return" button. | L |
| HU003 | Entered information on a page is committed to the database after a press of a "save" button. | M |

  - Reporting Requirements (RE)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| RE001 | Authorized user must be able to configure templates for standard reports, Invoice, Certificate of analysis or labels. | H |
| RE002 | Authorized user must be able to configure reporting rules for different projects. | H |
| RE003 | It should be possible for authorized users to configure templates for analysis results reporting. | M |
| RE004 | It should be possible for authorized users to configure templates for ad-hoc reporting. | L |
| RE005 | It should be possible for authorized users to configure KPI’s and dashboard with (management) information within the application interface. | M |
| RE006 | The data accessible for building reports, KPI’s and dashboards should at minimum be the same as the data accessible for end-users. | L |
| RE007 | It should be possible for a report to contain attached documents for example the underlying raw data. | L |
| RE008 | It should be possible to obtain clear printed copies of electronically stored data.. | M |
| RE009 | It should be possible to automatically or manually print reports. | M |
| RE010 | It should be possible to export reports to file formats (secure) e.g. pdf, docx, xlsx. Txt. HTML | M |
| RE011 | It should be possible to automatically email reports to a distribution list defined in the project profile | M |
| RE012 | It must be possible for reports to be versioned. One version can be printed multiple times, but regenerating the report changes the version. | H |
| RE013 | The regenerated report must contain the following: - Reason of regeneration - Version number | H |
| RE014 | It should be possible for end-users to use templates with predefined queries and filters to search for and select data in the system. | M |
| RE015 | It should be possible to export the result from a search to a file. | M |
| RE016 | It should be possible to generate graphical (trend) representations (graphs or charts) from selected data. | L |
| RE017 | It should be possible to drill down in to underlying data by clicking on data point in graphs generated by the system. | L |
| RE018 | It should be possible to automatically or manually generate labels containing user-configurable information, during every part of the workflow. | M |
| RE019 | A trained LIMS administrator/key-user must be able to create templates, custom fields and user-role-based user-interfaces in the system. | H |
| RE020 | A trained LIMS administrator/key-user must be able to create reports in the system. | H |
| RE021 | The system must be able to generate concept reports (for not yet finalized projects, samples or analysis) | M |

  - Security, Electronic Record and Electronic Signature Requirements (ES)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| ES001 | The system must restrict access to authorised individuals. | H |
| ES002 | Records containing information about which users have access to the system must be available and maintained | H |
| ES003 | Users must access the system through a unique personal user account. | H |
| ES004 | When a user account has multiple roles, it should be possible to automatically assign all roles. | M |
| ES005 | Printouts should show all current and valid electronic signatures on paper copies of the signed record. | M |
| ES006 | Authorized user must be able to revoke or disable users authorizations. | H |
| ES007 | Record changes must not obscure previously recorded information. | H |
| ES008 | It should be possible to configure mandatory input of a reason for changes to data. | M |
| ES009 | It must be possible to secure PDF exports against editing after they have been generated. | L |
| ES010 | Data must be secured by both physical and electronic means against damage. | H |
| ES011 | Stored data must be checked for accessibility, readability and accuracy. | H |
| ES012 | Access to data must be ensured throughout the retention period. | H |

  - System Interface Requirements (IN)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| IN001 | It should be possible to import results from an external lab in an electronic format. | M |
| IN002 | The system should be able to interface with a third-party reporting application. e.g. Crystal Reports | M |
| IN003 | The system must be able to bilaterally interface with analytical instruments and related software. | H |
| IN004 | Computerised systems exchanging data electronically with other systems must include appropriate built-in checks for the correct and secure entry and processing of data, in order to minimize the risks. | M |

  - IT (Infrastructure) Requirements (IT)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| IT001 | The system must support multiple printers and be configured in such a way that users can choose the printer to print | H |
| IT002 | It should be possible to configure a maximum idle time after which the current user session is automatically closed. | M |
| IT003 | A backup and restore procedure must be in place. | H |
| IT004 | Security monitoring must be in place to log failed signature / identification authentication. | H |
| IT005 | Security logs must be retained in such a way that they are readily available for review. | H |
| IT006 | The system should have the option of Two factor authentication. | M |
| IT007 | The system must be able to integrate with Aqualysis site Active Directory (AD) | H |

Scope of Supply (SC)

| Req.No. | Description of the Requirement | Priority |
| --- | --- | --- |
| SC001 | The supplier must provide system installation and qualification services. All testing of critical items carried out by the supplier must be carried out following pre-approved protocols and test sheets witnessed by Aqualysis (where appropriate). | H |
| SC002 | Training and education programs must be made available to understand the full functionality of the system (including e.g., software, on-line help and tutorials). Vendor will provide training of Aqualysis personnel in: - Technical (IT) training - LIMS Administrator training - Master data management training - User training Training certificates must be provided to all Aqualysis participants | H |
| SC003 | The supplier must provide at least one user and administration manual along with the system | H |
| SC004 | Supplier maintenance contract should be in place offering telephone support during office hours (CET) | H |
| SC005 | The supplier should provide a method for periodic preventive maintenance scheduled with intervals and a manual. | M |
| SC006 | The warranty must be at least 12 months long after documented final acceptancy of fully functional system by Aqualysis (maintenance agreement). | H |

Related & Referenced Documents

The following documents were used as a basis for developing this User Requirements Specification.

Document Title

Processen en Functionaliteiten LIMS V1.0

Rapportage LabInfoScan Aqualysis V2.0

Projectplan Aanloopfase QM-AquaLims V1.0