| for | Erasmus M.C. |  |
| --- | --- | --- |
| by | Jens Lodahl Spie ICS |  |
| Date: | 01 May 2017 |  |
|  | PXP (Patient Experience Platform) Technical Design |  |

| Document Control |  |
| --- | --- |
| Documentname: | Technical Design PXP Erasmus 0.1 |
| foldername: | Technical Design PXP Erasmus_EN_0.1.doc |
| Version number: | 0.1 |
| Date: | 01-05-2017 |
| Status: | Draft |
| Auteur: | Jens Lodahl |

| Versie controle |  |  |
| --- | --- | --- |
| Version | Date | remark |
| 0.1 | 01-05-2017 | Draft version made by Jens Lodahl |
| 0.2 | 03-05-2017 | Addition John Sjouken |

| Contactpersonen |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Nr: | Name | Titel | Organisation | Emailadres: | Telefoonnumber: |
| 1. | Eric Schiltmans | Accountmanager | Spie | eric.schiltmans@spie.com | 0683201267 |
| 2. | Ronald van Holst | Projectmanager | Spie | ronald.vanholst@spie.com | 0621806315 |
| 3. | John Sjouken | Leading Eng. ziekenhuisMedia | Spie | John.Sjouken@spie.com | 0623941055 |
| 4. | Alexander van Herpen | Eng. CLB/ZiekenhuisMedia | Spie | alex.vanherpen@spie.com | 0651613228 |
| 5. | A.J.Heesterman | Projectleider | Erasmus MC | a.heesterman@erasmusmc.nl | 0633331017 |
| 6. | M. Kiel | Operationeel projectleider | Erasmus MC | m.kiel@erasmusmc.nl | 0619244707 |
| 7. |  |  |  |  |  |

| Locations |  |  |  |
| --- | --- | --- | --- |
| Location | Adres | city | Tel nr: |
| Erasmus MC | Burgm. Jacobplein 51 | Rotterdam |  |

Content:

1.	General Network scheme	4

2.	Servers	5

2.1	PXP-Cloudserver	5

2.2	PXP HL7 interface server	5

2.3	PXP Johnson interface server (baknet-server)	5

2.4	multicast-unicast server	5

3.	IPTV	7

3.1	TV nursing department	7

3.2	TV daycare	7

4.	CuliCart – Meal ordering	8

5.	HIX Webfrontend	9

6.	PXP HL7 interface server	10

7.	HL7 Interface to Hospital system (HIX)	11

8.	Ascom Unite	12

9.	Johnson Controls	14

# General Network scheme

Below an overview about the PXP system inside the Hospital Erasmus MC and the companies where we are interfacing to.

# Servers

## PXP-Cloudserver

The management-system of the PXP system will be installed as a local instance on the local hospital VMWare-environment.

Specifications:

- OS: Linux

- Memory: 16GB ram

- CPU: 8-core CPU

- HDD: 500Mb (primary disk)

This will be used as controlling mechanism for the layout of the tablets. It also is being used for the content of the tablets. This is beside a content management system, also the device management system.

## PXP HL7 interface server

The PXP interface server is an instance which connects to all PXP devices and to the parties we have to interface to. So for instance CuliCart, HIX, Ascom. It’s also equipped with a HL7 interface to communicate with HIX to get information about the patient. So for instance the patient name for a personal welcome message. But also the bed number and location. This is required to instance with other systems as well, so for instance CuliCart.

But furthermore this interface server is also handling the interface to Johnson Controls via a non HL7 connection. See more details below.

The PXP interface server is a virtual instance which is hosted and running on the Hospitals own servers.

All PXP devices like tablet and TV are connected to the PXP interface server. The connection between tablet and PXP interface server is done via socket connection.
The TV’s are connected via a HTTP connection and pulling information via a JSON/AJAX connection from the PXP interface server to the TV.

Specifications:

- OS:

- Memory:

- CPU:

- HDD:

## PXP Johnson interface server (baknet-server)

This device is being used as interface between the tablets and the domotica.

This device will be installed as a local instance on the local hospital VMWare-environment.

Specifications:

- OS:

- Memory:

- CPU:

- HDD:

## Multicast-unicast server

This device is being used to convert the supplied SPTS to a HLS or HTTP stream.

Specifications:

- OS:

- Memory:

- CPU:

- HDD:

# IPTV

For the IPTV (and radio) we have two groups:

- TV nursing department
The TV nursing department is provided on TV’s mounted on the walls of the patient rooms. However the patients cannot watch TV on their tablets.

- TV daycare
The TV daycare is provided for the tablets

To get this operational in the PXP-backend two groups will be configured to separate the tablets of the nursing department and the daycare.

In the detailed access of those 2 groups also the tv signals and description can be configured. So for every group access to tv and radio can be limited.

## TV nursing department

The IPTV is supplied by ????????? and provided to the TV’s as a multicast stream.

For this to be operational the network should be designed with IGMP. This to lower networktrafic.

The TV management is controlled at the backend on the PXP-Cloudserver

## TV daycare

The IPTV is supplied by ????????? and provided to the multicast-unicast server as a combined multicast stream. The multicast-unicast server will transcode the tv to an acceptable view on the tablets. However the access points should be able to handle the signal to all tablets. This should be considered by the network designers, so other access by the hospital staff is acceptable.

The TV management is controlled at the backend on the PXP-Cloudserver

# CuliCart – Meal ordering

The following part describes how the meal ordering system from CuliCart is integrated into the PXP platform.

CuliCart is offering an existing webfrontend which can be used by the patients to select their meals. This webfrontend is provided my culicart itself.

On the PXP platform an App will be generated which link to this CuliCart webfrontend. The webfrontend is can be reached via a URL:

URL CuliCart webfrontend : https://NTB

The URL has additional parameters:
- patientID or bed/location-number  -> needs to be defined in details
- a token to identify that this webfrontend is called from our system and is allowed by CuliCart. This token is defined by CuliCart

The CuliCart webfrontend is HTML based.

Once the patient is opening the according CuliCart app, the PXP platform is opening a webengine with the CuliCart URL. The webengine and therefore also the webfrontend from Culicart is opened in full screen for the best user experience and best usability.
The patient has no possibilities to see the URL or to change the URL or to change settings of the webengine and this is not a full Browser just a webengine.

# HIX Webfrontend

The following part describes how the HIX patient webportal is integrated into the PXP platform.

HIX is offering an existing webfrontend which can be used by the patients to see all patient information allowed by the Hospital. This webfrontend is provided my HIX itself.

On the PXP platform an App will be generated which link to this HIX webfrontend. The webportal is can be reached via a URL:

URL HIX webportal : https://NTB

The URL has no additional parameters as the webportal require an individual login by each patient. Means the patient has to login manually after the webportal appears.

The HIX webportal is HTML based.

Once the patient is opening the according HIX webportal app, the PXP platform is opening a webengine with the HIX webportal URL. The webengine and therefore also the webfrontend from HIX is opened in full screen for the best user experience and best usability.
The patient has no possibilities to see the URL or to change the URL or to change settings of the webengine and this is not a full Browser just a webengine.

# PXP HL7 interface server

The PXP interface server is an instance which connects to all PXP devices and to the parties we have to interface to. So for instance CuliCart, HIX, Ascom. It’s also equipped with a HL7 interface to communicate with HIX to get information about the patient. So for instance the patient name for a personal welcome message. But also the bed number and location. This is required to instance with other systems as well, so for instance CuliCart.

But furthermore this interface server is also handling the interface to Johnson Controls via a non HL7 connection. See more details below.

The PXP interface server is a virtual instance which is hosted and running on the Hospitals own servers.

All PXP devices like tablet and TV are connected to the PXP interface server. The connection between tablet and PXP interface server is done via socket connection.
The TV’s are connected via a HTTP connection and pulling information via a JSON/AJAX connection from the PXP interface server to the TV.

# HL7 Interface to Hospital system (HIX)

This interface is used for getting general patient information by using the HL7 v2 protocol.
For instance:
- Patient name
- PatientID
- Bed / location
- Discharge
- room/bed move
These information are generally used for instance as personal welcome of the patient, Showing the bed and room number and ward to the patient. 
Further these information, like for instance the patientID, are also used to forward them to for instance the meal ordering system from CuliCart. With the patientID we can identify a patient toward CuliCart to let CuliCart know for which patient the webfrontend needs to be adjusted.

The HL7 connection to HIX is done via an Interface Broker called Cloverleaf.
The communication is done via a TCP/IP socket connection between the PXP interface server and the Cloverleaf Broker.

The Cloverleaf Broker is hereby acting as a server and is waiting for the PXP interface server, acting as client, to start a connection.

The interface server is starting to establish a connection. The HL7 interface will now get the current status of all beds and associated patients and store this information. The connected devices (Tablet / TV) will then be configured accordingly.

# Ascom Unite

The Ascom Unite interface is used to forward patients requests for general service requests to the Ascom Unite system which inform the according staff of the Hospital to deliver the requested item. Such items can be glas of water, blanket, pillow or others.

The App on the tablet is used by the patient to start a request, for instance for a glas of water. This request is now sent to the local PXP interface server. The interface server is now sending this request to the Ascom unite server.

Means a direct connection between the tablet and the Ascom unite server is not in place. The request of all tablets will first go down to the PXP interface server and the PXP interface server is communicating to the Ascom unite server. 

On the PXP interface server no data is stored or buffered. All data are send live to the Ascom unite server.

The interface between our PXP interface server and the Ascom unite system is realised via REST (HTTP Post) and JSON.
The Ascom unite system acts as a server and the PXP interface server is the client which starts the requests.

Following a typical JSON call for a class of water

{

"Spie": {

"Event": {

"ID": "1",

"Type": "1"

},

"Display": {

"Text_NL": "glas water",

"Text_DE": "glas wasser",

"Text_EN": "glass of water"

},

"Location": {

"ID": "H10N10741",

"Name": "H10Noord room 1074 bed 1"

}

}

}

The message „Spie“ contains several information.
- „Display“: In this section the several names for every language of the selected item is added. Ascom is only using „Text_NL“ as the staff speaks dutch.

- „Location“: This is the location of the requested bed and also the bed where the requested item has to be delivered to. Ascom is only using „Location.ID“. The „ID“ is a combination of the building, ward, room and bed. The Field „Name“ show an according example.

Location.Id is the admitted hospital location, which you will receive through HL7-ADT messages. It is a combination of PV1-3-1, PV1-3-2 and PV!-3-3.

An sample message:

PV1||I|SK3A^3106^8|R|||RJON31|||KIN||||I||||K|0001098917|||||||||||||||||""|""|||||||20170213060000

In this case Location.Id needs to be SK3A31068, so PV1-3 without the delimiter.

If the PXP Interface server send the HTTP POST, it will receive an HTTP 200 when everything is ok. In case the JSON isn’t formatted ok, or Location.Id / TEXT_NL are missing/empty, the PXP interface server will receive an HTTP 400. When not using a POST request for the REST/JSON request, a HTTP 501 will be received by the PXP interface server.

In case of a HTTP 200/400 response, you also will receive an message body back. This can be:

{

"response": "OK",

"ID": "dbe9c9af-0431-4c57-87e8-27e89264810a"

}

Or

{

"response": "ERROR",

"ID": "d8f6f890-f2fd-4f21-ba69-a83a3b218303",

"Message": "Message successfully sent."

}

In case there’s something internally wrong on Ascom side, we will Queue the message, and retry. You will receive:

{

"response": "OK",

"ID": "da5068f7-3923-40c4-a04b-6d00ccd9e608",

"Message": "Error sending message to destination. Message is placed in Queue"

}

# Johnson Controls

The Johnson Controls interface is used to forward the room control actions started by the patient on the tablet, to the according unit/device in the room and to control it. Such devices are in Erasmus:
- blinds
- temperature
- 3 lamps

The connection between the PXP interface server and the Johnson Control server is done via TCP/IP HTTP connection via REST API. The Johnson Controls server acts as a server and the PXP interface server acts as a clients.

The room control App on the tablet is used by the patient to change a room control unit/device, for instance for a lamp. This request is now sent to the local PXP interface server. The PXP interface server is now sending this request to the Johnson controls server.

Means a direct connection between the tablet and the Johnson controls server is not in place. The request of all tablets will first go down to the PXP interface server and the PXP interface server is communicating to the Johnson controls. 

On the PXP interface server no data is stored or buffered. All data are send live to the Johnson controls.

There is also no mechanisms in place that the PXP interface server is waiting for a feed back when a certain event is reach or something, for instance a temperatur. We’ll not show the current temperature of the room on the patient tablet. Also we’ll not send back an information to the tablet that a certain lamp is no on or off. The patient has just 2 buttons per lamp. 1 button to turn the lamp on and 1 to turn the lamp off. We’ll not give a feedback whether a lamp was switched on or off. This would cause a higher load of the Johnson Controls server and the Bacnet communication bus and to prevent issues Johnson Controls said that this feature should not be used.

Johnson Control is using on their side the EDE (Extended Data Engine) with Web Services. We are sending REST API requested to this interface.

The Johnson Control interface allow several options for us to call.

1) List all Rooms

Command GET:
http://IP-JOHNSON-SERVER:9090/services/ede/virtualdevices

Result :

<virtualdevices>

<virtualdevice>

<name>Room1</name>

<address>1</address>

<enabled>true</enabled>

</virtualdevice>

<virtualdevice>

<name>Room2</name>

<address>2</address>

<enabled>true</enabled>

</virtualdevice>

<virtualdevice>

<name>Room3</name>

<address>3</address>

<enabled>true</enabled>

</virtualdevice>

</virtualdevices>

2) List all Items of Room 1

Command GET:

http:// IP-JOHNSON-SERVER:9090/services/ede/virtualdevices/Room1/items

Result :

<items>

<item>

<id>6</id>

<name>Lamp_1</name>

<description/>

<extra/>

<type>1</type>

<address>1</address>

<deviceaddress>1</deviceaddress>

<unit/>

<initialvalue/>

<status>Normal</status>

<value>1</value>

<acked>false</acked>

<reliable>true</reliable>

<source/>

<destinations>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC609S.nviCmdLum_1 function.Present Value

</ref>

<exp>6</exp>

</destination>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC609S.nviCmdLum_1 setting.Present Value

</ref>

<exp>[$Room1.Lamp_1.Value]*100</exp>

</destination>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC609S.nviCmdLum_1 rotation.Present Value

</ref>

<exp>-30</exp>

</destination>

</destinations>

</item>

<item>

<id>8</id>

<name>Lamp_2</name>

<description/>

<extra/>

<type>1</type>

<address>2</address>

<deviceaddress>1</deviceaddress>

<unit/>

<initialvalue/>

<status>Normal</status>

<value>0</value>

<acked>false</acked>

<reliable>true</reliable>

<source/>

<destinations>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC609S.nviCmdLum_2function.Present Value

</ref>

<exp>6</exp>

</destination>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC609S.nviCmdLum_2 setting.Present Value

</ref>

<exp>[$Room1.Lamp_1.Value]*100</exp>

</destination>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC609S.nviCmdLum_2 rotation.Present Value

</ref>

<exp>-30</exp>

</destination>

</destinations>

</item>

<item>

<id>1</id>

<name>Temperature</name>

<description/>

<extra/>

<type>5</type>

<address>1</address>

<deviceaddress>1</deviceaddress>

<unit/>

<initialvalue/>

<status>Normal</status>

<value>21.21</value>

<acked>false</acked>

<reliable>true</reliable>

<hialarm/>

<loalarm/>

<hiwarning/>

<lowarning/>

<source>

[bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC601N.nvoSpaceTemp.Present Value]

</source>

</item>

<item>

<id>2</id>

<name>Setpoint_Shift</name>

<description/>

<extra/>

<type>5</type>

<address>2</address>

<deviceaddress>1</deviceaddress>

<unit/>

<initialvalue>0</initialvalue>

<status>Normal</status>

<value>2.5</value>

<acked>false</acked>

<reliable>true</reliable>

<hialarm/>

<loalarm/>

<hiwarning/>

<lowarning/>

<source/>

<destinations>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC601N.nviShiftSetpoint.Present Value

</ref>

</destination>

</destinations>

</item>

<item>

<id>7</id>

<name>Sunblind_1</name>

<description/>

<extra/>

<type>12</type>

<address>1</address>

<deviceaddress>1</deviceaddress>

<unit/>

<initialvalue>1</initialvalue>

<status>Normal</status>

<value>1</value>

<acked>false</acked>

<reliable>true</reliable>

<source/>

<destinations>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC601N.nviCmdStore_1 function.Present Value

</ref>

<exp>6</exp>

</destination>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC601N.nviCmdStore_1 setting.Present Value

</ref>

<exp>0</exp>

<condition>

[$Room1.Sunblind_1.Value]==1 or [$Room1.Sunblind_1.Value]==3

</condition>

</destination>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC601N.nviCmdStore_1 setting.Present Value

</ref>

<exp>100</exp>

<condition>[$Room1.Sunblind_1.Value]==2</condition>

</destination>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC601N.nviCmdStore_1 rotation.Present Value

</ref>

<exp>-30</exp>

<condition>

[$Room1.Sunblind_1.Value]==1 or [$Room1.Sunblind_1.Value]==2

</condition>

</destination>

<destination>

<ref>

bn:DH-NC074CR-AS01/DH-NC074CR-AS01/LON Trunk1.NC.NC_C6.NCC601N.nviCmdStore_1 rotation.Present Value

</ref>

<exp>100</exp>

<condition>[$Room1.Sunblind_1.Value]==3</condition>

</destination>

</destinations>

</item>

</items>

3) Get Value of Lamp_1

Command GET :

http:// IP-JOHNSON-SERVER:9090/services/ede/virtualdevices/Room1/items/Lamp_1/value

Result :

<item>

<value>1</value>

</item>

4) Set Value of Lamp_1 to 1

Command PUT :

http:// IP-JOHNSON-SERVER:9090/services/ede/virtualdevices/Room1/items/Lamp_1/value/1

The general values which can be written for each item:

Lamp : 0 = Off, 1 = On

Setpoint Shift : -2 to 2 DegC

Sunblinds : 1 = Down , 2 = Up , 3 = Stop