Use case 10

Intercom

| Naam (Name) | 10 -- Intercom |
| --- | --- |
| Samenvatting / Description | If a relative of a patient or a visitor want to access a department where the door is closed an intercom must be used. |
| Actoren / Actors | - Visitor(s): Caregivers, Patients, relatives, other visitors and staff of Erasmus not having the permission to access this departments - Staff (person who opens the door) |
| Aannamen / Triggers | - The entrance door of the department has a card reader for giving staff access. At the door an intercom is in place for the other actors. - The intercom system is equipped with speech and has the possibility to open the door. |
| Customer user stories | RSA-A01 |
| System functions involved | - Entrance control system - Intercom - Unite - Caregiver device |
| System Beschrijving / Description | This usecase is describing the handling of the intercom a visitor want to have acces to a closed department. . At the entrance door of a department a intercom is placed. During certain hours of the day the doors are closed. If a visitor wants to access the department he/she must push the intercom button. A person who needs access to the department can make a call (via the intercom) to the staff on that department. They decide if the person is allowed to access the department. The initial message from the intercom is send to Unite. The alert will be handled according to the set assignment for that department at that time of the day. (see use case 1). The intercom can be handled from the Myco. In the message the option speech and cancel are presented. If the speech button is pressed a speech connection is automatically established. If the speech connection has been setup new options will appear in the display. “Open door” and “cancel call”. If the door may be opened, the door will be unlocked by pressing the button. If the door should remain closed the cancel button will be pressed and the speech connection will be terminated in this case. |
| Non-functional requirements (quality aspect) |  |
| Starting points Preconditions Points for attention System | - The intercom system should be able to send out a message to Unite when the intercom has been activated - The intercom should be able to be connected to the main PBX to be able to have voice communication between intercom and Myco. - The intercom shall be able to unlock the door after receiving the “door open” command Use of camera is not foreseen in this use case. |