Client Representatives Geotechnical Survey Nederwiek noord Wind Farm Zone

Annex 9b

Starting Points and Assumptions

Colophon

| Project name | Client Representatives Geotechnical Survey Nederwiek noord Wind Farm Zone |
| --- | --- |
| Project number | 202307200 |
| Version | Draft_04 |
| File name | VS_20230817_RVO_CR GT_Annex 9b SPA_D |
| Annex(es) | 1 |
| Author | M. Witteveen |
| Reviewed | S. Wezenberg |
| Approved | P. Lebbink |

Contents

1	Introduction	4

2	General introduction to the offshore wind energy in the Netherlands	5

2.1	Existing offshore wind farms	5

2.2	The roadmap 2030+ towards 21 GW offshore wind power	5

3	General introduction Nederwiek noord Wind Farm Zone	8

3.1	Layout & coordinates NW WFZ	8

3.2	Cables and pipelines within the NW WFZ	10

3.3	Units	11

3.4	Vertical and horizontal datum	11

4	Quality requirements	12

4.1	Spatial data	12

4.2	Taxonomy	12

4.3	GIS Metadata requirements	12

4.4	Mandatory data delivery BRO	12

4.5	Document control requirements	12

4.6	Standards, formats, templates	13

4.7	Other data	14

4.8	Deliverables (results / maps)	14

4.9	Storage of data (electronic / hardcopy)	14

5	Communication	15

5.1	Confidentiality	15

5.2	Accessibility public information	15

5.3	Authorities	15

5.4	Media	16

5.5	Email	16

5.6	Emergency Response	16

# Introduction

This document provides the starting points and assumptions for Nederwiek noord Wind Farm Zone.

The starting points and assumptions are to be used by contractors when executing investigations, studies or likewise, for the development of the Wind Farm Zones and commissioned by one of the following organisations:

- The Ministry of Economic Affairs and Climate Policy

- The Ministry of Infrastructure and Water Management

- Netherlands Enterprise Agency (RVO)

- Rijkswaterstaat

- TenneT TSO B.V.

This document is part of the QA/QC process for these studies.

Please note:

- This is a living document; before use check for the latest version available (to be confirmed by your client)

If you have any queries please contact your CLIENT.

# General introduction to the offshore wind energy in the Netherlands

## Existing offshore wind farms

The Netherlands’ existing offshore wind farms have a combined capacity of approximately 4,700 MW, of which 2,400 MW is fully operational, and another 2,300 MW will be operational at the end of 2023 (see Table 1).

Table 1: Wind Farms in the Dutch North Sea (operation or under construction)

| Windfarm | Sites | Distance of the coast | Year of commissioning | Turbines | Owner |
| --- | --- | --- | --- | --- | --- |
| Offshore Wind Farm Egmond aan Zee (OWEZ) |  | 10-18 km | 2008 | 36 Vestas 3 MW | Noordzeewind (joint venture NUON and Shell) |
| Prinses Amalia Wind Farm |  | 23 km (12 NM) | 2009 | 60 Vestas 2 MW | Eneco |
| Luchterduinen |  | 23 km (12 NM) | 2015 | 43 Vestas 3 MW | joint venture Eneco and Mitsubishi Corporation |
| Gemini |  | 85 km | 2017 | 150 Siemens 4 MW | Northland Power, Siemens, Van Oord and HVC |
| Borssele | I - II | 22 km | 2020 | 94 Siemens 8 MW | Ørsted |
|  | III - IV |  | 2020 | 77 Vestas 9,5 MW | Blauwwind |
|  | V |  | 2021 | 2 Vestas 9,5 MW | Two Towers |
| Hollandse Kust Zuid | I - IV | 18 km | Under Construction | 140 Siemens 11 MW | Vattenfall |
| Hollandse Kust Noord | V | 18,5 km | Under Construction | 69 11 MW | Crosswind |
| Hollandse Kust West | VI | 53 km | In Preparation |  | Ecowende |
|  | VII |  | In Preparation |  | Oranje Wind Power II |

## The roadmap 2030+ towards 21 GW offshore wind power

The Dutch Government has created the offshore wind roadmaps 2030+ to set out a schedule of tenders for wind farm permits (see Figure 1 and Table 2). In the beginning of 2022 the Dutch Government announced three new Wind Farm Zones called Lagelander, Nederwiek (Nederwiek zuid site I and Nederwiek noord sites II and III and Doordewind. The development of the southern part (HKWWFZ VIII) of the Hollandse Kust (west) Wind Farm Zone and the Ijmuiden Ver Wind Farm Site Gamma in the IJmuiden Ver Wind Farm Zone were also confirmed. The total capacity of offshore wind energy will then be 21 GW and around 90 TWh of power production. This equals 75% of the current Dutch electricity consumption.

The wind farms will be built in designated Wind Farm Zones. Wind farms outside these Wind Farm Zones are prohibited. Within the designated Wind Farm Zones, the government defines the specific sites where wind farms can be constructed using a so-called Wind Farm Site Decision (‘Kavelbesluit’). This Wind Farm Site Decision contains all conditions for building and operating a wind farm on that specific site.

Developers can apply in tenders for the permits to build and operate a wind farm according to a Wind Farm Site Decision. Winners of these permit tenders will be granted a permit and a grid connection to the mainland.

The Dutch Government provides comprehensive site data and Dutch transmission system operator TenneT is responsible for grid connection. RVO is responsible for the site data, which can be used for the preparation of bids for these tenders.

Table 2: Wind Farm Zones in the Dutch North Sea

| Wind Farm Zones in the Dutch North Sea |  |  |  |  |
| --- | --- | --- | --- | --- |
| Wind Farm Zone | Wind Farm Site | Tender | Capacity | Owner |
| Borssele | I and II | 2015 | 752 MW | Ørsted Borssele 1 B.V. |
|  | III and IV | 2016 | 731 MW | Blauwwind II C.V. (Shell, Van Oord, Eneco, Diamond Generating Europe) |
|  | V | 2016 | 19 MW | Two Towers |
| Hollandse Kust (zuid) | I and II | 2017 | 740 MW | Chinook (Vattenfall) |
| Hollandse Kust (zuid) | III and IV | 2018 | 760 MW | Vattenfall |
| Hollandse Kust (noord) | V | 2019 | 759MW | Crosswind |
| Hollandse Kust (west) | VI | 2022 | ≥700 MW | t.b.d. |
|  | VII | 2022 | ≥700 MW | Oranje Wind Power II |
| IJmuiden Ver | Alpha (I and II) | Q1 2024 | ≥ 2 GW | t.b.d. |
|  | Beta (III and IV) | Q1 2024 | ≥ 2 GW | t.b.d. |
|  | Gamma (V and VI) | Q2 2025 | ≥ 2 GW | t.b.d. |
| Nederwiek | I | Q2 2025 | ≥ 2 GW | t.b.d. |
|  | II | 2026 | ≥ 2 GW | t.b.d. |
|  | III | 2026 | ≥ 2 GW | t.b.d. |
| Hollandse Kust (west) | VIII | 2026/2027 | ≥700 MW | t.b.d. |
| Ten noorden van de Waddeneilanden | I | 2026/2027 | ≥700 MW | t.b.d. |
| Doordewind | I | 2027 | ≥ 2 GW | t.b.d. |
|  | II | 2027 | ≥ 2 GW | t.b.d. |

An overview for the Roadmap 2030+ is provided in Figure 1.

Figure 1: Dutch Offshore Wind Farm Zones overview for Roadmap 2030+

# General introduction Nederwiek noord Wind Farm Zone

## Layout & coordinates NW WFZ

The Nederwiek Wind Farm Zone (NW WFZ) is located approximately 100 kilometres off the west coast of the Netherlands. General characteristics of the NW WFZ are shown in Table 6. Table 7 and Figure 4 shows the coordinates of the Nederwiek Wind Farm Zone.

The NW WFZ is planned to contain a total capacity of 6 GW divided over 3 sites of 2 GW each. The wind farm permit tender is divided in two phases, schedules to be granted between 2025 and 2026. Nederwiek Wind Farm Site I (Nederwiek zuid) is planned to be the first phase and Nederwiek Wind Farm Sites II and III (Nederwiek noord) are planned for 2026.

Table 6: Characteristics of the Nederwiek Wind Farm Zone (noord and zuid)

| Characteristics |  |
| --- | --- |
| Water depth | Approximately 24,5-34,0 mLAT |
| Distance from shore | Approximately 100 km |
| Total Wind Farm Zone area | Approximately 947 km2 |

Table 7: Coordinates of the Nederwiek Wind Farm Zone

| Nederwiek Wind Farm Zone coordinates |  |  |
| --- | --- | --- |
| Point No. | Easting | Northing |
| WFZ_01 | 511262.1 | 5861912.4 |
| WFZ_02 | 504988.9 | 5898196.1 |
| WFZ_03 | 504551.9 | 5900695.3 |
| WFZ_04 | 506513.0 | 5900728.8 |
| WFZ_05 | 518517.0 | 5887991.9 |
| WFZ_06 | 519911.7 | 5886660.6 |
| WFZ_07 | 518524.1 | 5881971.5 |
| WFZ_08 | 517977.8 | 5880125.3 |
| WFZ_09 | 513135.7 | 5865490.3 |
| WFZ_10 | 522313.6 | 5894778.0 |
| WFZ_11 | 520808.6 | 5896425.1 |
| WFZ_12 | 507039.2 | 5911494.3 |
| WFZ_13 | 501892.9 | 5921837.6 |
| WFZ_14 | 503352.4 | 5925523.5 |
| WFZ_15 | 504405.7 | 5927462.4 |
| WFZ_16 | 508337.1 | 594699.5 |
| WFZ_17 | 531529.6 | 5927141.8 |
| WFZ_18 | 531897.7 | 5927021.8 |
| Coordinate system: ETRS_1989_UTM_Zone_31N |  |  |
| WKID: 25831 Authority: EPSG |  |  |

Figure 4: Maps of the Nederwiek Wind Farm Zone

## Cables and pipelines within the NW WFZ

Several operational and abandoned cables and pipelines cross the Wind Farm Zone, as shown in Table 8 and Figure 5.

Table 8: Characteristics of cables and pipelines in the Nederwiek Wind Farm Zone

| Cables and pipelines |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Name | Operator | Trace | Diameter [in] | Type | Status |
| PL0187_PR | Equinor ASA | Draupner (N) to Duinkerken (F) | 42 | Pipe | Active |
| PL0186_PR | Equinor ASA | Sleipner (N) to Zeebrugge (B) | 40 | Pipe | Active |
| PL0176_PR | BBL Company V.O.F. | Bagzand (NL) to Bacton (GB) | 36 | Pipe | Active |
| PL0004_PR | Wintershall Noordzee B.V. | K13-AP | 36 | Pipe | Active |
| PL0098_PR | Wintershall Noordzee B.V. | - | 24 | Pipe | Active |
| PL0018_PR | Wintershall Noordzee B.V. | - | 20 | Pipe | Abandoned |
| PL0019_PR | Wintershall Noordzee B.V. | Sidetap K13C#2 to K13-AP/K13-AW | 20 | Pipe | Abandoned |
| PL0017_PR | Wintershall Noordzee B.V. | K13-D to K13-C | 10 | Pipe | Abandoned |
| PL0017_HS | Wintershall Noordzee B.V. | K13-D to K13-C | 2 | Pipe | Abandoned |
| PL0028_PR | Wintershall Noordzee B.V. | K10-B to K13-C | 20 | Pipe | Abandoned |
| PL0072_PR | Wintershall Noordzee B.V. | K13-CP/K13-CW to Sidetap K13-C | 20 | Pipe | Active |
| PL0027_HS | Wintershall Noordzee B.V. | K10-C to K10-B | 2 | Pipe | Abandoned |
| PL0027_PR | Wintershall Noordzee B.V. | K10-C to K10-B | 10 | Pipe | Abandoned |
| PL0110_HS | Wintershall Noordzee B.V. | K10-V to K10-C | 2 | Pipe | Abandoned |
| PL0110_PR | Wintershall Noordzee B.V. | K10-V to K10-C | 10 | Pipe | Abandoned |
| PL0111_PR | Wintershall Noordzee B.V. | K10-V to J6-A/K13-A | 10 | Pipe | Active |
| PL0111_HS | Wintershall Noordzee B.V. | K10-V to J6-A/K13-A | 2 | Pipe | Active |
| PL0007_PR | NAM B.V. | - | 24 | Pipe | Active |
| PL0137_PR | NAM B.V. | K8-FA-1AP/K8-FA-1P to K14-FA-1C | 24 | Pipe | Active |
| PL0014_PR | NAM B.V. | K11-FA-1 to K8-FA-1 | 6.6 | Pipe | Abandoned |
| PL0076_PR | NAM B.V. | K8-FA-2 to K8-FA-1 | 10 | Pipe | Active |
| PL0015_PR | NAM B.V. | K8-FA-2 to K8-FA-1 | 11 | Pipe | Active |
| PL0023_PR | NAM B.V. | K7-FA-1P to K8-FA-1 | 18 | Pipe | Active |
| SEA-ME-WE 3 | T Systems International / Deutsch Telekom | n/a | n/a | Cable | Active |
| NeuConnect West | NeuConnect Britain Limited | n/a | n/a | Cable | Planned |
| UK – Germany 3 | - | n/a | n/a | Cable | Abandoned |
| Winterton – Borkum 1 | - | n/a | n/a | Cable | Abandoned |
| UK – DK 3 | - | n/a | n/a | Cable | Abandoned |
| UK – Germany 4 | - | n/a | n/a | Cable | Abandoned |
| UK – Germany 5 | British Telecom | n/a | n/a | Cable | Abandoned |
| UK – DK 1 | - | n/a | n/a | Cable | Abandoned |

## Units

SI units (ISO/IEC 8000) are applicable for any activities carried out at the Wind Farm Zone.

## Vertical and horizontal datum

The chart datum is Lowest Astronomical Tide (LAT) using the ‘NLLAT2018’ reference surface as described on the website of the Ministry of Defence.

All positioning data shall be collected in the European Terrestrial Reference System 1989 (ETRS89)

and projected using ETRS89 Transverse Mercator Coordinate Reference System (UTM Zone 31N).

The reference for this system is EPSG Projection code 25831 - ETRS89 / UTM zone 31N. The Epoch to consider will be provided by CLIENT.

# Quality requirements

This chapter describes quality requirements including data model and GIS data deliveries. During the process and delivery of the data further information or explanations can be requested at RVO.

As RVO will make this data publicly available, the metadata needs to be in line with the GDPR (AVG) requirements.

## Spatial data

All created data in the study will be delivered by the supplier in GIS format. The supplier will need to comply with the data requirements set out by RVO and provide an inventory list of the deliverables supplied. The inventory should include project name, name of contractor and responsible person, survey date, product medium (i.e. hard drive, web service, etc.), folder structure and description of data contents.

- All the data is required to be delivered in the geodetic system European Terrestrial Reference System 1989 (ETRS89) ETRS89 UTM ZONE 31 N - EPSG code: 25831 http://www.epsg-registry.org/.

- The data should be delivered in one or more of the following formats: Value / Parameter Encoded Raster, TIFF and ESRI Shapefile (.shp). Should another format be used, this can only be delivered after consulting with RVO.

- In addition to the above, an ESRI file Geodatabase deliverable is required in a standard structured format, preferably following IOGP standards. (https://www.iogp.org/bookstore/product/guideline-for-the-delivery-of-the-seabed-survey-data-model/)

- Both input data and interpreted data along with any relevant GIS package (*. ppkx / *.mpk) are required and should also include the associated reports and charts.

- All GIS data shall be provided with Metadata describing specific details and origin regarding the spatial data. The requirements regarding the metadata are included in Annex A.

## Taxonomy

All contractors must use the taxonomy as presented in the taxonomy list, which is available online at:

https://offshorewind.rvo.nl/files/view/dff880d4-9494-4e6d-b398-8fd1f097957e/162642502520210714_taxonomy%20list%20offshore%20wind%20energy_dutch-english.xlsx

## GIS Metadata requirements

The requirements for GIS metadata which all Contractors shall comply with is shown in Annex A of this document.

## Mandatory data delivery BRO

RVO as a government agency is obligated by law to provide subsurface data to the central ‘subsurface reqister’ in the Netherlands. The specifics on the mandatory delivery to the “Basisregistratie Ondergrond (BRO)” is provided in the Scope of Work document whenever applicable. Guidance on the BRO delivery for geotechnical works is presented in Appendix C of this document.

## Document control requirements

The naming of the documents shall be standardized in order to facilitate the document control during the whole project. The following naming code is required for all documents produced within the project:

AreaCode_Date_Organization_Name document_Author_Version_Status

- Area Code:

- NWN: Nederwiek noord

- Date: yyyymmdd, e.g. 20230101

- Organization: Name of the organization e.g.

  - ”RVO” for all internal and general documents produced by the Netherlands Enterprise Agency (RVO)

  - Consultant or Contractor name (shortened)

- Name document: Make sure the name clearly explains the content, include here the original name if from external party

- Author: Abbreviation of the author if an internal document. Author shall be removed for final documents

- Version: Revision number or date, respectively V01 till V0x_D or 20230101_F

- Status: (D) for draft documents, (F) for signed and approved documents

## Standards, formats, templates

All reports will be written in English and include both an English and Dutch management summary.

Contractors provide for each completed study a publishable summary for the Project and Site description. Maximum 2 A4 including some high-resolution pictures or graphs.

All reports must use the terminology as specified in the online taxonomy list https://offshorewind.rvo.nl/files/view/dff880d4-9494-4e6d-b398-8fd1f097957e/162642502520210714_taxonomy%20list%20offshore%20wind%20energy_dutch-english.xlsx

All reports must use the terminology as specified in Appendix A of the Starting points and assumptions. For your convenience this dictionary is also available as XLSX file.

Reports and deliverables will be delivered by contractors in pdf and original, regular, formats (docx; xlsx; ppt; etc).

Final reports will have a cover according to the corporate identity of the Netherlands Enterprise Agency, with the following text:

•	Front:

Client Representatives Geotechnical Survey Nederwiek noord Wind Farm Zone

Name document

•	Back:

A colophon with the following content:

The creative commons license 4.0 apply to this material. (including a Creative Commons logo

and hyperlink to the relevant Creative Commons website page: https://creativecommons.org/licenses/by/4.0/)

•	The (investigation) (report) was carried out by <<name contractor>>, commissioned by RVO, an agency of the Ministry of Economic Affairs. Whilst a great deal of care has been taken in compiling the contents of this document and the mentioned investigations, Netherlands Enterprise Agency (RVO) cannot be held liable for any damages resulting from any inaccuracies and/or outdated information.

•	Contact

Netherlands Enterprise Agency (RVO)

Croeselaan 15 | 3521 BJ | Utrecht | The Netherlands

P.O. box 8242 | 3503 RE | Utrecht | The Netherlands

www.rvo.nl/ http://english.rvo.nl

•	Netherlands Enterprise Agency (RVO) | Date of publication

## Other data

In the case of collected data: primary data must be delivered in regular formats.

## Deliverables (results / maps)

All maps (in GIS and PDF) will not show the investigation areas nor the specific Wind Farm Sites. The maps will only show the outer boundary of the Wind Farm Zone boundary of the Nederwiek noord Wind Farm Zone, or the investigation area for the Nederwiek noord Wind Farm Zone.

## Storage of data (electronic / hardcopy)

The Netherlands Enterprise Agency will store all reports and other relevant documents for at least five years both electronically and hardcopy.

# Communication

## Confidentiality

All information provided to contractors/suppliers by the Ministry of Economic Affairs and Climate Policy and/or the Netherlands Enterprise Agency and/or the Ministry of Infrastructure and Water Management and/or Rijkswaterstaat and/or TenneT must be regarded confidential.

All deliverables produced by contractors/suppliers remain confidential until a deliverable has been made public by the Ministry of Economic Affairs and Climate Policy and/or the Netherlands Enterprise Agency and/or the Ministry of Infrastructure and Water Management and/or Rijkswaterstaat and/or TenneT.

## Accessibility public information

- Site investigations: offshorewind.rvo.nl

- SDE: http://english.rvo.nl/subsidies-programmes/stimulation-sustainable-energy-production-sde

- Wind farm Site Decisions and EIA:	https://www.rvo.nl/subsidies-regelingen/bureau-energieprojecten/lopende-projecten

## Authorities

Wind Farm Site Decision (‘Kavelbesluit’):

Ministry of Economic Affairs and Climate Policy

Camiel van der Hout

E c.j.r.vanderhout@minezk.nl

T +31 (0)6-11 02 81 31

Rijkswaterstaat

Raoul Syrier

E raoul.syrier@rws.nl

T +31 (0)6 11072554

Site data:

Netherlands Enterprise Agency

Matté Brijder

E matte.brijder@rvo.nl

T + 31 (0)6 27878952

Grid connection (‘Transmissiesysteem op zee’):

TenneT TSO BV

Johan Dekkers

E johan.dekkers@tennet.eu
T +31 (0)6 29360328

## Media

All media contacts will be coordinated by the Ministry of Economic Affairs and Climate Policy. If a supplier is approached by press representatives than he/she should refer to the spokesman of the Ministry only.

Ministry of Economic Affairs and Climate Policy
Caspar Itz

c.itz@minez.nl

+ 31 610551061

## 5.5	Email

All emails related to the site investigations must be send in CC to the project mailbox: NW@rvo.nl

The subject of an email related to Client Representatives Geotechnical Survey Nederwiek noord Offshore Wind Farm Zone needs to start with the date and the abbreviation of Client Representatives Geotechnical Survey Nederwiek noord Wind Farm Zone. Then also it needs to be followed by an abbreviation of the type of site survey. E.g. : DDW_20230717_CR_GT for the Client Representatives Geotechnical Survey for the Nederwiek noord Offshore Wind Farm Zone Investigation Area.

## 5.6	Emergency Response

In case of an emergency suppliers are obliged to inform the Netherlands Enterprise Agency immediately.

#### GIS Metadata requirements

The goal of these metadata requirements is to make sure the future users of this data can use this data in the appropriate way and discover the data’s usage constraints from the metadata.

Format

All data collected by third parties, either in the field or as part of a desktop study, must be provided with metadata. Data is preferably collected in an ArcGIS file geodatabase (ArcGIS Pro version 2.9 or lower). In this case, metadata is stored within the data in the database. In case of a shapefile, GML or GRID metadata must be provided in pure XML-format as separate files.

Metadata must be compliant with the Dutch standard for metadata ISO 19115 version 2.0.0 (or any version available at the time of creating the data), in addition the column names of all data records More information on the standard can be found here:

https://www.geonovum.nl/geo-standaarden/metadata/nederlands-metadataprofiel-op-iso-19115-geografie

To validate your metadata a validator is available from the website of Geonovum:

https://validatie.geostandaarden.nl/etf-webapp/testruns/create-direct?testProjectId=3dd218a5-2ba3-3f1c-98fb-10e923839a3c

Manufacture options

Creating metadata should preferably done when the data is acquired or created and checked and finalised before delivering the data. If necessary, a valid example can be provided.

More information can also be found at:

https://github.com/Geonovum/Metadata-ISO19115/blob/master/Voorbeeld_Metadata_Dataset_2019_max.xml.

An XML editor which allows you to sort the metadata into an XML tree can also be helpful.

Preferred option:

Organisations that use Esri software can use the GeoSticker application to create and edit metadata: http://www.esri.nl/geosticker.

Other options:

The Metadatamaker offered by the RWS can also be used to make the metadata and can be found at http://rws.metainfomaker.nl/

The Nationaal Georegister (NGR) allows meta data to be made, to do this a guest account is required. This can be requested by mailing to EZ geodatabeheer (geodatabeheer.GISCC@rvo.nl )

Other options are possible as long as the data will be validated by the metadata validator for ISO 19115 version 2.0.0 (or newer version available at the time of creating the data) at the website of Geonovum

A General Overview Metadata Elements with some Explanation:

Contact Information

- Contact:

As this information will be made publicly available personal information should be avoided and generic contact details should be provided. If personal email addresses are provided, that person should have given consent to use their e-mail address.

At least one contact information of client and/or creator and/or point of contact is mandatory and it should include Name organisation, role organisation (owner, processor, point of contact, administrator, provider, creator), and a generic email address of contact department.

If website is available, name of contact and role of contact.

Due to the General Data Protection Regulation (GDPR) personal information in the data is no longer advisable, a general email instead of a personal one and a function title instead of a person’s name is preferable.

CI_Responsible Party:
Individual Name: …

Organisation Name: …

Telephone: …

Address: …

Online Resource (website): …

Date

- dateStamp

When was the metadata made?

Identification (identificationInfo)

- File title

Real name of the Title (abbreviations are acceptable)

- Alternate Title:

No abbreviations, complete title of the dataset covering the content of the dataset.

- date & dateType:

creation YYYY-MM-DD

publication YYYY-MM-DD (date of approval by RVO)

revision YYYY-MM-DD

- Abstract

Short description of the content of the dataset.

- Purpose of production:

Concise description of the original aim of the dataset

- Status:

According to a standard list:

completed; historicalArchive; obsolete; onGoing; planned; required; underDevelopment.

- ResourceMaintenance: Maintenance and update frequency:

N/A

- ResourceMaintenance: Date of Next Update:

N/A

- Graphic Overview

This can include a link to an example image online.

- Keywords:

Keywords describing the dataset taken from the GEMET Thesaurus https://www.eionet.europa.eu/gemet/en/themes/

- Resource Constraints

  - useLimitation:

Applications for which the data set is not to be used. I.e., Not to be used for navigation. Not to be used on a scale larger then 1:50.000

  - accessConstraints:

None, data will be published in the public domain.

  - useConstraints:

The creative commons license 4.0 apply to this material. See hyperlink:

https://creativecommons.org/licenses/by/4.0/

This investigation was carried out by …, commissioned by RVO, an agency of the Ministry of Economic Affairs and Climate Policy. Whilst a great deal of care has been taken in compiling the contents of this investigation, RVO cannot be held liable for any damages resulting from any inaccuracies and/or outdated information.

  - otherConstraints:

Other restriction(s) if any.

- Spatial Representation Type

https://standards.iso.org/iso/19139/resources/gmxCodelists.xml#MD_SpatialRepresentationTypeCode

eg vector, grid, textTable, tin, stereoModel

- Spatial Resolution: Equivalent Scale (for vector data)

Denominator: Scale on which the dataset can be used

- Spatial Resolution: MD_Resolution (grid data)

Distance: eg uom="meter">1.0 Note Only fill in the distance for grid data

- Language

Language that the data is in eg dut (Dutch), eng (English)

- Topic Category:

Define the key topic(s) of the dataset eg geoscientificInformation, oceans,elevation

Nederlands metadata profiel op ISO 19115 voor geografie versie 2.1.0 (geostandaarden.nl)

- Extent

Geographical bounding box in WGS84 with North and East in positive values eg 52.50. There is also a placeholder to fill in the SpatialTemporalExtent (beginPosition and endPosition) and the VerticalExtent (minimumValue and maximumValue)

Distribution Info

dataQualityInfo

- Scope

https://standards.iso.org/iso/19139/resources/gmxCodelists.xml#MD_ScopeCode

eg dataset

Metadata requirements

The goal of these metadata requirements is to make sure that future users can use this data in an appropriate way and discover the data’s usage constraints from the metadata.

ContentInfo

1.	FeatureCatalogueDescription

Attribute information: Description of all attributes, including - if applicable – all possible values and its measurement.

Distribution Info

2.	distributorContact

The organisation responsible for the distribution of the dataset

3.	contactInfo

Includes organisational email woz@rvo and link to Offshorewind RVO

dataQualityInfo

4.	Scope

https://standards.iso.org/iso/19139/resources/gmxCodelists.xml#MD_ScopeCode

eg dataset

5.	Lineage: Statement

General description (statement of when, how and where the data was created / collected.

6.	Lineage: ProcessStep

Processing of data (if applicable): If the data has been processed, then a description of each step is required i.e., transformation or generalisation.

7.	Lineage: Source

Source datasets (if applicable): A description of used source datasets (if any). Is the dataset complete? If not, a description of the incompleteness is necessary.

There is one major difference between the Dutch metadata standard ISO 19115 version 2.0.0 and the RVO metadata requirements: the description of the content of the dataset. A description of all attributes, including - if applicable – all possible values and its measurement is mandatory and not optional.