Technical specifications for a confocal microscope

Timo Hamers

Samantha Hughes

Lisa Baumann

Peter Cenijn

A-LIFE (EH&T)

Abstract

This describes the qualification and selection procedure for a broad-spectrum confocal microscope for fluorescence imaging of different cell and animal models.

Recipients

TenderNed

Document status

Revision	Date	Comment			Reviewed by		Approved by

2		08-08-2024

Revision History

Revision 	Date	Description

0			First Issue

# Documentation

## Abbreviations

Abbreviation	Description

CoC	Certificate of conformity

# Introduction

## Scope

The following specifications are for the market consultation of a broad-spectrum confocal microscope.

The purpose of this market consultation is to find potential suppliers of such an instrument, which fulfill our specific demands (listed below) and price point, in order to give direction on which procurement procedure the Vrije Universiteit Amsterdam should follow for this purchase. Therefore, we ask the potential suppliers to respond based on the information provided in this document regarding technical product specifications, functional specifications of the listed main applications, delivery, service provision and other conditions. If, after requesting the information, you decide not to respond further, we kindly request you to notify us. To be clear: this is NOT a quote request.

## Purpose of specification

The specification is written to formulate the requirements of the confocal microscope. We can use the following definition of requirement (RD 1):

Requirement: a statement that identifies a product or process operational, functional, or design characteristic or constraint, which is unambiguous, testable, or measurable, and necessary for product or process acceptability (by consumers or internal quality assurance guidelines).

# Specifications for the confocal microscope

This section details the minimum requirements regarding technical specifications and support of the confocal microscope. Any reply to this market consultation must meet all listed requirements. The applicant is requested to indicate its conformation to each individual requirement.

## General features

### The system must be a laser scanning confocal microscope, with a scanning stage

### The system must be equipped with 10x, 20x, 40x, 63x objectives, and a long working distance 25x objective

### The system must be provided with an actively damped optical table, and a computer table

### The system must be provided with workstation + software to control the microscope

### The system must contain a high precision z-stage

### For widefield observations the system must include DAPI, FITC and TRITC filter cubes and a light source.

### The confocal microscope system must capture a wide range of fluorescently stained proteins in the most flexible way

### The system must be capable of capturing 8-colour emissions across a spectrum from 391nm to 727nm with independent laser lines

### The system must have a white light laser to tune the wavelength of each excitation line and detection window to match the spectral profile of dyes in the chosen sample.

### The system must have lifetime-based technology to facilitates the access the complexity of functional imaging

### The system must have proven upgrade possibilities for extended excitation range up to the near-infrared spectrum

### The system must have proven upgrade possibilities for light sheet imaging

### The system must have proven upgrade possibilities for an incubation box for temperature and CO2 that can be used for live cell imaging.

### Installation, 2-day training and 1 year warranty must be included.

### The system must be equipped with all relevant hardware and software to operate the user interface and different detectors, and to enable the system to achieve the relevant specifications.

## Mandatory options: warranty and service

### All equipment should be guaranteed for trouble-free operation and against manufacturing defects for a minimum period of 1 year after the acceptance test at A-LIFE (EH&T). A new equivalent warranty period shall apply to all the products delivered by way of replacement. If applicable, the warranty period shall be extended by the period during which the product could not be used due to damage. The Contracting Authority may demand from the supplier that he replace the products at his expense within the set term, if they show defects which do not permit normal use in accordance with the terms of the contract, for a period of one year counting from the date of provisional acceptance at the place of delivery.

### The vendor should guarantee the availability of spare parts and service support for at least 10 years.

### Technical help desk support should be available during normal local office working hours.

### The company must have a local (EU) service department.

### The language of all support (visits and technical help desk) should be English or Dutch.

### The Vendor should offer software updates for at least 10 years.

## System price

### The Total Price of the system should not exceed € 300.000,00 including VAT. This will include delivery, installation, warrantee, and training, as well as a pre-installation site check.

## Process demonstration

### The vendor selected at the evaluation of the tenders could be asked, on request of The Contracting Authority to demonstrate the capability of the offered system to meet the above requirements.

# Logistics

## Shipping and delivery

### The system must be delivered no later than 8 months after the award of the contract by A-LIFE (EH&T).

### The holder ensures, under his full responsibility, the transport and installation of all equipment on the site.

### Responsibilities for customs, shipping, transport, transport insurance and taxes follow DAP (Delivery Duties Unpaid), Incoterms 2020.

## Packaging

### The system must be packaged for shipment in a way to ensure safe delivery.

## Documentation

### The following documentation should be provided as electronic pdf files, in English language:

### installation Manual

### operation Manual

### maintenance Manual

### The following details should be provided in the documentation:

### system footprint

### system dimensions and weight

### infrastructure requirements

The VU would like to give you the opportunity to ask any questions you may have about this market consultation. You can do this by e-mail until 15-08-2024, 11:00 a.m. at the latest. You should send your application and questions by e-mail to aanbestedingen.fpc@vu.nl for the attention of the Purchasing Management department. When submitting your response, please include " broad-spectrum confocal microscope” in the subject line. Answers to the questions will be published on Tenderned by means of an information memorandum by 22-08-2024 at the latest. The results of the market consultation can be used as input for any tender to be followed. After the market consultation has ended, written feedback of the market consultation is provided to all respondents in the form of a summary of the answers, taking into account any commercial interests of the parties. This feedback does not affect the obligation of the market consultation. Prices and/or rates are excluded from publication. Candidates who have participated in this market consultation and who have answered the questions will receive the summary by e-mail, no later than 30-09-2024. This summary will also be published as an appendix to any tender to be followed.

In addition to the terms and conditions stated elsewhere in this document, this market consultation is subject to the following terms and conditions:

• Parties cannot derive any (mutual) obligations or rights towards the VU from this market consultation;

• Participation in this market consultation does not guarantee a role as supplier;

• Any costs for participating in this market consultation will not be reimbursed by the VU;

• The official language during this market consultation is in principle English;

• the VU is in no way bound by the results of the market consultation or obliged to realize and/or tender for the subject to which the market consultation relates;

• Claims about the use of information, confidentiality or requests for compensation in connection therewith will not be honored;

• The VU reserves the right to suspend this market consultation temporarily or permanently