Bijlage 2:	Specificaties CNC-machines

Specificaties machines perceel 1: drie compacte 3-assige freesmachines

| Dimentions - shipping |  | Metric |
| --- | --- | --- |
| Domestic/export Pallet | max | 240 cm x 220 cm x 240 cm |
| Weight | max | 5000 kg/m2 |

| Axis |  | Metric |
| --- | --- | --- |
| X Axis | min | 400 mm |
| X Axis Rapids | min | 30 m/min |
| X Axos trust | min | 8500 N |
| Position accuracy | min | 0.01mm |
| Y Axis | min | 350 mm |
| Y Axis Rapids | min | 30 m/min |
| Y Axos trust | min | 8500 N |
| Position accuracy | min | 0.01mm |
| Z Axis | min | 380 mm |
| Z Axis Rapids | min | 30 m/min |
| Z Axis trust | min | 8500 N |
| Position accuracy | min | 0.01mm |
| Z Axis Spindel nose to Platter | max | 480 mm |
| Z Axis Spindel nose to Platter | min | 100 mm |

| Table |  | Metric |
| --- | --- | --- |
| lengt | min | 900 mm |
| width | min | 300 mm |
| Number of T-slots | min | 3 |
| Weight on table | max | 150 kg |

| Spindle |  | Metric |
| --- | --- | --- |
| Rating | min | 10 kW |
| Speed | min | 10.000 rpm |
| Torque | min | 26 Nm / 5000 rpm |
| Drive System |  | Directdrive or belt drive |
| Taper |  | BT40 |
| Baring lubrication |  | Grease |
| Cooling |  | Air Cooled |

| Tool Changer |  | Metric |
| --- | --- | --- |
| Capacity | min | 30 + 1 |
| Chip-to-chip | min | 5 sec |
| Tooldiameter (full) | min | 60 mm |
| Tool Length (frone gaga line) | min | 170 mm |
| Type tool changer |  | Side Mount |

| Electrical Specification + light |  | Metric |
| --- | --- | --- |
| Full load Voltage (3 Phase) | max | 440 VAC |
| Full load Amps (3 Phase) | max | 40 A |
| machinelight |  | Yes Led. |

| Coolant and Air Requirments |  | Metric |
| --- | --- | --- |
| Coolant Capacity | min | 150 L |
| Placement |  | external |
| Flow | min | 20 L/min |
| Filter |  | Yes external |
| Air required | max | 120l / 9 bar |
| Variable flow coolant |  | no |
| Coolant through the spindle |  | no |
| Programmable coolant nozzle |  | no |

| Options |  | Metric |
| --- | --- | --- |
| Program memory | min | 1GB |
| Control systeem |  | Fanuc, Siemens or derivatives |
| Synchronized tapping |  | yes |
| Chip conveyor |  | yes |

Specificaties machines perceel 2: twee compacte 5-assige simultaan freesmachines

| Dimentions - shipping |  | Metric |
| --- | --- | --- |
| Domestic/export Pallet | max | 280 cm x 280 cm x 280 cm |
| Machine Dimentions | max | 280 cm x 280 cm x 280 cm |
| Weight | max | 5000 kg/m2 |

| Axis |  | Metric |
| --- | --- | --- |
| X Axis | min | 350 mm |
| X Axis Rapids | min | 60 m/min |
| X Axos trust | min | 10.000 N |
| Position accuracy | min | 0.01mm |
| Y Axis | min | 350 mm |
| Y Axis Rapids | min | 60 m/min |
| Y Axos trust | min | 10.000 N |
| Position accuracy | min | 0.01mm |
| Z Axis | min | 300 mm |
| Z Axis Rapids | min | 60 m/min |
| Z Axis trust | min | 15.000 N |
| Z Axis Spindel nose to Platter | max | 400 mm |
| Z Axis Spindel nose to Platter | min | 100 mm |
| Position accuracy | min | 0.01mm |
| B Axis travel | min | 120° to -45° |
| B Axis Speed | min | 80°/sec |
| B Axis Torque | min | 400Nm |
| B Axis Brake Torque | min | 500Nm |
| B Axis Accuracy | min | 25 arc-sec |
| B Axis Repeatability | min | 10 arc-sec |
| B Axis accuracy | min | 0.01° |
| C Axis travel | min | 360° unlimited |
| C Axis Speed | min | 100°/sec |
| C Axis Torque | min | 280Nm |
| C Axis Brake Torque | min | 250Nm |
| C Axis Accuracy | min | 15 arc-sec |
| C Axis Repeatability | min | 10 arc-sec |
| C Acis accuracy | min | 0.01° |

| Trunnion |  | Metric |
| --- | --- | --- |
| Platter Diameter | min | 200 mm |
| Weight on platter | min | 40 Kg |

| Spindle |  | Metric |
| --- | --- | --- |
| Rating | min | 10 kW |
| Speed | min | 10.000 rpm |
| Torque | min | 60 Nm / 1700 rpm |
| Drive System |  | Inline / directdrive |
| Taper |  | BT40 |
| Baring lubrication |  | Air / oil injectoin |
| Cooling |  | Air Cooled |

| Tool Changer |  | Metric |
| --- | --- | --- |
| Capacity | min | 18 + 1 |
| Chip-to-chip | min | 5 sec |
| Tooldiameter (full) | min | 60 mm |
| Tool Length (frone gaga line) | min | 170 mm |
| Type tool changer |  | Side Mount |

| Electrical Specification + light |  | Metric |
| --- | --- | --- |
| Full load Voltage (3 Phase) | max | 440 VAC |
| Full load Amps (3 Phase) | max | 40 A |
| machinelight |  | Yes Led. |

| Coolant and Air Requirments |  | Metric |
| --- | --- | --- |
| Coolant Capacity | min | 180 L |
| Placement |  | external |
| flow | min | 40L/min |
| Filter |  | Yes external |
| Air required | max | 120l/min / 9 bar |
| Variable flow coolant (M-code) |  | yes |
| Coolant through the spindle (M-code) |  | yes |
| Programmable coolant nozzle (M-code) |  | yes |

| Options |  | Metric |
| --- | --- | --- |
| Program memory | min | 1GB |
| Control systeem |  | Fanuc, Siemens or derivatives |
| Synchronized tapping |  | yes |
| Chip conveyor |  | yes |