Annex 2

Requirements document

as part of the tendering procedure

in respect of

| Version date | 01-01-2025 |
| --- | --- |

Flight Inspection Services

Table of Contents

Table of Contents	2

1. Tasks	3

1.1 Regular tasks	3

1.2 Additional tasks	4

2. General Technical Requirements	5

3. Flight Inspection aircraft and equipment	6

3.1 Aircraft	6

3.2 Flight Inspection system	7

4. Preparation and Operational specifications	10

5. Navaid Calibration requirements	12

5.1 Requirements for ILS and terminal DME	12

5.2 Requirements for (D)VOR and enroute DME	12

5.3 Requirements for PBN	13

5.4 Requirements for DF	13

Appendix A	15

Appendix B	16

Appendix C	17

Appendix D	18

Appendix E	19

Appendix G	22

# Tasks

## Regular tasks

| Requirement nr. | FI2024 – 1.1.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The tenderer shall perform regular calibration of the following systems by flight inspections: ILS (CAT I, II and III), SVOR, DVOR, DME, NDB/L and DF installations in the Netherlands. |  |  |
| Additional information | An overview of all civil ground based navigation equipment installed in the Netherlands can be found in table 1. |  |  |
| Compliance | Use Pull Down menu |  |  |

Table 1 Overview of installed civil ground based navigation equipment

| Airport | ILS/DME | DVOR/DME | SVOR/DME | DME | DF | NDB/L |
| --- | --- | --- | --- | --- | --- | --- |
| EHAM | KAG 06 |  |  |  | Schiphol |  |
| (Amsterdam | ZWA 18C | SPL |  |  |  |  |
| Schiphol) | VPB 18R |  |  |  |  |  |
|  | SCH 22 |  |  |  |  |  |
|  | BVB 27 |  |  |  |  |  |
|  | MSA 36C |  |  |  |  |  |
|  | ABA 36R |  |  |  |  |  |
| EHRD | RSV 24 | RTM |  |  | Rotterdam |  |
| (Rotterdam – The Hague) | ROS 06 |  |  |  |  |  |
| EHBK | BKN 21 |  | MAS |  | Beek |  |
| (Maastricht | BKZ 03 (S) |  |  |  |  | NW |
| Aachen) |  |  |  |  |  |  |
| EHGG | GRO 23 (S) | EEL |  |  | Eelde |  |
| (Groningen |  |  |  |  |  |  |
| Eelde) |  |  |  |  |  |  |
| EHLE | ILSN 05 |  |  |  | Lelystad |  |
| (Lelystad) |  |  |  |  |  |  |
| En-Route |  |  |  | FRT | Texel |  |
|  |  |  |  | FRO | Nijkerk |  |
|  |  |  |  | PAM | Veldhoven |  |
|  |  |  |  | SPY | Maasland |  |
|  |  |  |  | RKN | Assendelft |  |
|  |  |  |  | HDR |  |  |
|  |  |  |  | AMS |  |  |

(S) = no standby transmitter

| Requirement nr. | FI2024 - 1.1.2 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The flight inspection procedures and tolerances shall fully comply with all relevant ICAO international standards and recommended practices. |  |  |
| Additional information | Further details on flight inspection for navigation systems are given in appendix A to F. |  |  |
| Compliance | Use Pull Down menu |  |  |

## Additional tasks

| Requirement nr. | FI2024 - 1.2.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The tenderer shall perform on request incidental calibration by flight inspections after incidents, and/or trouble shooting and/or for (radio) interference investigation (incidental calibration flights) within 48 hours after the initial request. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 1.2.2 | Type: Knock out |  |
| --- | --- | --- | --- |
| Description | The tenderer shall provide Flight Validation capabilities for the validation of instrument flight procedures. |  |  |
| Additional information | In ICAO Doc 9906, Vol 5 requirements in this regard are described. Instrument flight procedures are not validated on a regular basis, but only on request. This includes RNAV and RNP departures and approaches. |  |  |
| Compliance | Use Pull Down menu |  |  |

# General Technical Requirements

| Requirement nr. | FI2024 - 2.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | Tenderer shall state that all procedures, requirements, definitions and measurements of parameters for the flight inspection of the ATC-NL navaid systems are in compliance with ICAO Annex 10 and the ICAO Manual on Testing of Radio Navigation Aids, Doc 8071 (Volume I, Volume II and Volume III), most recent versions. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 2.2 | Type: Essential | Max points: 7 |
| --- | --- | --- | --- |
| Description | ATC-NL wants the smallest measurement uncertainty per parameter as possible. The measurement uncertainty to 95% confidence level should be calculated for each of the parameters to be measured. The method of calculation, any assumptions made and errors involved in the measurements of the aircraft position, should be explained. |  |  |
| Additional information | - |  |  |
| Compliance | The answer will be assessed on the following - but not exclusively: - Uncertainty per parameter. The focus will be mainly on the ILS parameters. - Thoroughness of the answers. Maximum number of sheets 10 A4. |  |  |
| Explanation |  |  |  |

# Flight Inspection aircraft and equipment

## Aircraft

| Requirement nr. | FI2024 - 3.1.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The Tenderer shall perform all work according to the regulations published in the Netherlands Aeronautical Information Publication (AIP) and Aeronautical Information Circulars (AIC). |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 3.1.2 | Type: Essential | Max points: 7 |
| --- | --- | --- | --- |
| Description | ATC-NL wants as little disruption to the airport operation as possible. The tenderer should state the duration required for a Routine ILS flight inspection on one transmitter accordingly: - Appendix B, Localizer parameters and procedures and - Appendix C, Glide path & DME parameters and procedures |  |  |
| Additional information | As flight checks disrupt the operations, especially at Amsterdam Airport Schiphol, the duration (air time) of a routine ILS flight check is of significant importance. With air time is meant: - The time from take-off to landing in an ideal situation (no ATC or traffic restrictions), comprising: take-off, flight check and landing in the same direction at the same runway. - Recording of LOC/GP course measurement shall start at 8NM from threshold. |  |  |
| Compliance | The answer will be assessed on the following: - Tenderer with the shortest duration will get maximum points. - Tenderer should explain in detail how the calculation is done. Tenderer may use examples of ILS flight inspection on Schiphol or comparable airports as evidence for the given duration. |  |  |
| Explanation |  |  |  |

| Requirement nr. | FI2024 - 3.1.3 | Type: Essential | Max points: 7 |
| --- | --- | --- | --- |
| Description | ATC-NL wants ILS/DME flight inspections to be performed by an aircraft which can be used in as many weather conditions as possible. The tenderer should state the limits which apply to the flight inspection aircraft. |  |  |
| Additional information | This statement should include the limitations within which the flight inspection aircraft is able to perform accurate flight inspections. These limitations are expressed in: - crosswind - tailwind - visibility - cloud base - precipitation - wake turbulence sensitivity - whether the aircraft(s) are certified for flight checks during icy conditions. |  |  |
| Compliance | Tenderer will be assessed on: - best overall result (lowest limitations) |  |  |
| Explanation |  |  |  |

| Requirement nr. | FI2024 - 3.1.4 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The endurance of the aircraft shall make it possible to carry out actual flight inspection missions of at least three (3) hours flight duration. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

## Flight Inspection system

| Requirement nr. | FI2024 - 3.2.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The flight inspection equipment, including associated antennas shall be completely independent from the aircraft’s operational avionics equipment. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 3.2.2 | Type: Essential | Max points: 5 |
| --- | --- | --- | --- |
| Description | All aircraft which are used for this service, including operated by a backup party (if applicable), should use a Flight Inspection System from the same type and manufacturer to ensure the recorded data sets and flight inspection reports are fully compatible with those from the tenderer’s FIS. |  |  |
| Additional information | The tenderer should explain this in the Plan of Action. |  |  |
| Explanation |  |  |  |

| Requirement nr. | FI2024 - 3.2.3 | Type: Knock Out |  |  |
| --- | --- | --- | --- | --- |
| Description | Tenderer shall prove to ATC-NL that all systems and associated equipment used for flight inspection operations are subject to a calibration schedule and that the calibration is performed by an ISO/IEC or national equivalent, certified calibration institute. |  |  |
| Additional information | All equipment involved in flight inspection and subject to calibration shall have traceability to national or international standards. |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 3.2.4 | Type: Essential | Max points: 5 |
| --- | --- | --- | --- |
| Description | Tenderer will provide ATC-NL with a motivated calibration interval of each part of the measurement system and a calibration protocol for the measurement system appropriate for ATC-NL. |  |  |
| Additional information | This involves showing how the equipment’s interval standard is applied to each of the parameters which it can measure or generate. The internal standard should have traceability to national or international standards. |  |  |
| Compliance | The answers will be assessed on: - calibration interval motivation, - calibration protocol of the measurement system, - further explanation and traceability to national or international standards Maximum number of sheets 3 A4. |  |  |
| Explanation |  |  |  |

| Requirement nr. | FI2024 - 3.2.5 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | If a ground based reference station for positioning (like Differential GNSS) is needed, tenderer shall provide a permanent ground based reference station at Amsterdam Airport Schiphol. |  |  |
| Additional information | The Tenderer shall perform flight checks at Amsterdam Airport Schiphol without assistance of the ATC-NL technical support staff at the airside. If a ground based reference station for positioning is needed, a permanent ground based reference station is required, as setting up a mobile ground based reference station at the airside can be very time consuming. This requirement is intended to gain efficiency in the preparation time of a flight check. See table “Operational hours and support” in Appendix G for airport specific assistance and facilities. When tenderer will use a ground based reference station (like DGNSS), tenderer shall obtain necessary data link frequencies from the Radio Communication Agency in The Netherlands. Concerning the frequency usage of these data link frequencies ATC-NL can assist the tenderer for acquiring the licenses from the Radio Communication Agency. |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 3.2.6 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The flight inspection system shall include equipment that can record and replay all measured parameters of the navigation aid being inspected. All recordings shall be correlated to the aircraft’s position at the time of measurement. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 3.2.7 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The flight inspection system shall be capable of exporting all results in Microsoft/Windows compatible data-files for evaluation and reporting. |  |  |
| Additional information | Compatible file types are e.g. Excel, txt. |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 3.2.8 | Type: knock Out |  |
| --- | --- | --- | --- |
| Description | Tenderer shall provide all what is required to facilitate replay of any recorded inspection data on a PC/laptop at the ATC-NL premises (e.g. software, user licences). |  |  |
| Additional information | ATC-NL intends to use flight inspection data for comparison with simulation results of navigation aids |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 3.2.9 | Type: Essential | Max points: 2 |
| --- | --- | --- | --- |
| Description | ATC-NL wants to be able to have continuous recording of high resolution 3D position information during the whole flight. The tenderer should state if the flight inspection system has a facility for continuous recording of high resolution 3D position information during the whole flight, including taxi, take-off and landing phase. |  |  |
| Additional information | - |  |  |
| Explanation |  |  |  |

# Preparation and Operational specifications

| Requirement nr. | FI2024 - 4.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The tenderer shall report measured deviations immediately after the flight inspection to ATC-NL when during a flight inspection deviations of the signal in space are being measured. |  |  |
| Additional information | All definitions and specifications (tolerances) of parameters for the flight inspection of the navaid facilities for ATC-NL are described in ICAO Annex 10. Not only "Out of tolerances" of measured parameters as described in aforementioned documents shall be reported, also (significant) differences from previous flight checks or differences from what was to be expected shall be mentioned. |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 4.2 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The tenderer shall hand over a flight inspection certificate with the status of the inspected systems, within 24 hours after each flight inspection. This certificate contains information whether the inspected system meets the requirements. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 4.3 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The tenderer shall provide a flight inspection report of each and individual system within 10 working days after each flight inspection. The flight inspection report shall contain the measured values and recordings of the inspected parameters including written remarks by the flight inspector. Not only remarks for "Out of tolerances" of measured parameters but also (significant) differences from previous flight checks or differences from what was to be expected shall be reported. |  |  |
| Additional information | The flight inspection reports shall be sent as electronic documents - Reports with graphics as pdf standard - Results of the measured values in Excel or XML on request - Raw data by FIS specific data files on request |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 4.4 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The tenderer shall comply with the flight timeslots given by ATC-NL concerning the inspection flights as mentioned in Appendix G. As commercial air traffic is subject to change and as works at airports are carried out, ATC-NL preserves the right to require other timeslots. |  |  |
| Additional information | Timeslots, working hours, calibration hours of navigation aids are summarized in the table “Operational hours and support” in Appendix G (this table contains airport specific assistance and facilities). |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 4.5 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | The tenderer must be able to perform flight inspection outside the flight timeslots given by ATC-NL on request concerning the inspection flights as mentioned in Appendix G. |  |  |
| Additional information | Timeslots, working hours, calibration hours of navigation aids are summarized in the table “Operational hours and support” in Appendix G (this table contains airport specific assistance and facilities). |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 4.6 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | Immediately after the conclusion of the Agreement, a Dossier of Agreements and Procedures (DAP) between ATC-NL and the tenderer will be prepared by the tenderer. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

# Navaid Calibration requirements

## Requirements for ILS and terminal DME

| Requirement nr. | FI2024 - 5.1.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | For ILS localizer inspections the flight inspection system of the tenderer shall measure and record the parameters as described in appendix B with a periodicity as described in appendix A. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 5.1.2 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | For ILS glide path inspections the flight inspection system of the tenderer shall measure and record the parameters as described in appendix C with a periodicity as described in appendix A. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 5.1.3 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | For terminal DME inspections the flight inspection system of the tenderer shall measure and record the parameters as described in appendix C with a periodicity as described in appendix A. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

## Requirements for (D)VOR and enroute DME

| Requirement nr. | FI2024 - 5.2.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | For (D)VOR inspections the flight inspection system of the tenderer shall measure and record the parameters as described in appendix D with a periodicity as described in appendix A, and by flying the (D)VOR procedures given in appendix E. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

| Requirement nr. | FI2024 - 5.2.2 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | For enroute DME inspections the flight inspection system of the tenderer shall measure and record the parameters as described in appendix D with a periodicity as described in Appendix A, and by flying the enroute DME procedures given in appendix E. |  |  |
| Additional information | - |  |  |
| Compliance | Use Pull Down menu |  |  |

## Requirements for PBN

| Requirement nr. | FI2024 - 5.3.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | Tenderer shall be able to perform flight inspection for PBN procedures. |  |  |
| Additional information | The tenderer should state the possibilities and methods for inspecting PBN procedures in the Plan of Action. |  |  |
| Compliance | Use Pull Down menu |  |  |

## Requirements for DF

| Requirement nr. | FI2024 - 5.4.1 | Type: Knock Out |  |
| --- | --- | --- | --- |
| Description | For DF the flight inspection system of the tenderer shall measure and record the parameters as described in appendix F with a periodicity as described in Appendix A. |  |  |
| Compliance | Use Pull Down menu |  |  |

Signature

Name of person signing

Name of company

Date

# Appendix A

Flight check periodicity overview

| System | Type inspection | Interval (months) |  | Max interval |  |
| --- | --- | --- | --- | --- | --- |
| ILS / DME Cat I | Routine | 6 |  | + 45 days |  |
| ILS / DME Cat II & III | Routine | 6 |  | + 30 days |  |
| ILS / DME Cat I / II & III | Coverage | 24 |  | + 90 days |  |
| S-VOR/DME | Orbits and radials | 12 |  | + 90 days |  |
| D-VOR/DME | Orbits and radials | 12 |  | + 90 days |  |
| DME en route | Orbit | 12 |  | + 90 days |  |
| DF |  | On request |  |  |  |
| NDB |  | On request |  |  |  |
| VHF/UHF COM |  | On request |  |  |  |
| Radar (PSR en MSSR) |  | On request |  |  |  |
| Wide Area Multilateration |  | On request |  |  |  |

# Appendix B

Localizer parameters and procedures

The flight inspection system shall as a minimum be able to measure and record the following system parameters during a localizer calibration.

| Parameter | Routine Calibration on one transmitter | Coverage Calibration on one transmitter | Commissioning Calibration on two transmitters |
| --- | --- | --- | --- |
| Course alignment1 | × |  | × |
| Course structure1 | × |  | × |
| Identification1 | × |  | × |
| Course modulation depth1 | × |  | × |
| Course alarm 90Hz |  |  | × |
| Course alarm 150Hz |  |  | × |
| Course width nominal including symmetry2 |  |  | × |
| Course width wide including symmetry |  |  | × |
| Course width narrow including symmetry |  |  | × |
| Off course clearance3 | × |  | × |
| High angle clearance4 |  | × | × |
| Localizer Coverage5 |  | × | × |

Ad.1	These parameters for Localizer and Glide Path, have to be measured in one Approach.

Ad.2	In case measurement of Width nominal is not a part of the flight inspection, standard width of the corresponding system will be used for the calculations.

|  |  | Sector | Altitude | Distance to LOC |
| --- | --- | --- | --- | --- |
| Ad.3 | Arc | ±35° about runway centerline | 1500 ft | 6 NM |
| Ad.4 | Arc | ±35° about runway centerline | 4500 ft | 6 NM |
| Ad.5 | Arc | ±35° about runway centerline | 2000 ft | 17 NM |
| Ad.5 | Arc | ±10° about runway centerline | 2000 ft | 25 NM |

# Appendix C

Glide path & Terminal DME parameters and procedures

The flight inspection system shall as a minimum be able to measure and record the following system parameters during a glide path calibration.

| Parameter | Routine Calibration on one transmitter | Coverage Calibration on one transmitter | Commissioning Calibration on two transmitters |
| --- | --- | --- | --- |
| Glide Path |  |  |  |
| Course alignment1 | × |  | × |
| Glide Path structure1 | × |  | × |
| ILS Reference datum1 | × |  | × |
| Course modulation depth1 | × |  | × |
| Course alarm 90Hz |  |  | × |
| Course alarm 150Hz |  |  | × |
| Course width nominal +75µA2 |  |  | × |
| Course width nominal -75µA2 |  |  | × |
| Course width wide +75 µA |  |  | × |
| Course width wide -75 µA |  |  | × |
| Course width narrow +75 µA |  |  | × |
| Course width narrow -75 µA |  |  | × |
| Clearance3 | × |  | × |
| Obstruction clearance4 |  |  | × |
| Glide Path Coverage5 |  | × | × |
| Terminal DME |  |  |  |
| Mean DME distance error1 | × |  | × |
| DME coverage1 |  | × | × |

Ad.1	These parameters for Localizer, Glide Path and Terminal DME, have to be flight checked in one Approach.

Ad.2	In case measurement of Width nominal is not a part of the flight check, standard width of the corresponding installation will be used for the calculations.

Ad.3	Level inbound flight on 1250ft above terrain from 13 NM to threshold on LOC centreline.

Ad.4	180mA fly-up run

|  |  | Sector | Altitude | Distance to threshold |
| --- | --- | --- | --- | --- |
| Ad.5 | Arc | ±8° about runway centerline | 1400 ft | 10 NM |
| Ad.5 | Arc | ±8° about runway centerline | 3200 ft | 10 NM |
| Ad.5 | Arc | ±8° about runway centerline | 5600 ft | 10 NM |

# Appendix D

VOR and DME enroute parameters

The flight inspection system shall as a minimum be able to measure and record the following system parameters during a periodic VOR and DME enroute calibration.

Parameter

VOR Orbits

Mean (D)VOR bearing error

Max Bearing error with corresponding bearing

Min. Bearing error with corresponding bearing

VOR bearing error structure

Modulation depths

Signal strength

Ident

VOR radials

VOR alignment error

VOR bearing error structure

Radial bends

Radial roughness

Modulation depths

DME en route

mean distance error

Signal strength

# Appendix E

VOR / DME procedures

DME AMS

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | AMS | 3000 ft | 20 NM |  | 126 NM around |

VOR/DME EEL

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | EEL | 3000 ft | 19 NM |  | 120 NM around |
| R 048 | EHGG 23 | 2000 ft | 7...4 NM | Approach 23 * | length 11 NM |
| R 229 | EHGG 05 | 2000 ft | 17...4 NM | Approach 05 * | length 13 NM |

DME Lelystad (FRO)

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | FRO | 2000 ft | 20 NM |  | 126 NM around |

DME Haastrecht (FRT)

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | FRT | 2000 ft | 20 NM |  | 126 NM around |

DME HDR

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | HDR | 2000 ft | 20 NM |  | 126 NM around |

VOR/DME MAS

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | MAS | FL 050 | 30 NM |  | 188 NM around |
| R243 | FIR BDRY | FL 050 | 12 NM | M617 |  |
| R 253 | SORGI | FL 050 | 9 NM | UM617 |  |

DME PAM

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | PAM | 2000 ft | 26 NM |  | 163 NM around |

DME RKN

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | RKN | 3000 ft | 19 NM |  | 119 NM around |

VOR/DME RTM

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | RTM | 3000 ft | 10 NM |  | 63 NM around |
| R 056 | EHRD 24 | 2000 ft | 8...-2 NM | Approach 24 * | length 10 NM |
| R 236 | EHRD 06 | 2000 ft | 12...2 NM | Approach 06 * | length 10 NM |

VOR/DME SPL

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | SPL | 2000 ft | 26 NM |  | 163 NM around |

DME SPY

| Profile | Waypoint | Altitude | DME Dist | Airway | Remarks |
| --- | --- | --- | --- | --- | --- |
| Orbit | SPY | 2000 ft | 25 NM |  | 157 NM around |

*  To be measured during an ILS approach flight check.

# Appendix F

Direction Finder procedures

For all Direction Finders:

| Profile | Altitude | Distance | Remarks |
| --- | --- | --- | --- |
| Orbit around DF | 3000 ft | 20 NM | 126 NM around Test channel: 119.705 Bearing measurement every 2° on the orbit |

# Appendix G

Table “Operational hours and support”

As commercial air traffic is subject to change and as works at airports are carried out, ATC-NL preserves the right to require other timeslots.

|  |  | Available support during flight inspection |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Location | Time for flight inspection 1) | General Air Traffic Control | 1st line technical support 2) | 2nd line technical support 3) 6) | Technical support at airside 4) | General support at airside 5) | Remark |
| EHAM ILS/DME | 7 days per week 21:30 - 23:00 LT | Yes 7 days per week 24/7 | Yes 7 days per week 24/7 | Yes 5 working days per week 08:30 - 24:00LT | No | General Aviation Jetcenter until 23:00 LT Possibility after 23:00 LT on request | Including DF Schiphol |
| EHRD ILS/DME | 7 days per week 08:00 - 16:30 LT | Yes 7 days per week | Yes 7 days per week | Yes 5 working days per week 08:00 - 16:00LT | Yes 5 working days per week 08:00 - 16:00 LT | KLM Jetcenter 07:00 - 23:00 LT | Including DF Rotterdam |
| EHGG ILS/DME | 7 days per week 08:00 - 16:30 LT | Yes 7 days per week | Yes 5 working days per week | Yes 5 working days per week 08:00 - 16:00 LT | Yes 5 working days per week 08:00 - 16:00 LT | 06:30 - 23:00 LT | Including DF Eelde |
| EHBK ILS/DME | 5 working days per week 08:00 - 15:00 LT | Yes 7 days per week 06:00-23:00 LT | Yes 5 working days per week | Yes 5 working days per week 08:00 - 16:00 LT | Yes 5 working days per week 08:00 - 16:00 LT | General Aviation 06:00 - 23:00 LT ATC-NL technical staff present 5 working days per week 08:00 - 16:00 LT | Including DF Beek |
| EHLE ILS/DME | 7 days per week 07:30 - 09:00 LT | Yes 7 days per week 07:00-22:00 LT | Yes 7 days per week 06:00 - 23:00 LT | no | no | General Aviation 06:00 - 23:00 LT | Including DF Lelystad |
| VOR/DME en-route | MAS/ RTM/ EEL 10:00 - 15:00 LT SPL 17:00 - 23:00 LT | Yes | Yes 24/7 | Yes 5 working days per week 08:30 - 16:00 LT | n/a | n/a |  |
| DME en-route | AMS/ PAM/ SPY 15:00 - 23:00 LT FRO/FRT/RKN 15:00 - 23:00 LT HDR 10:00 - 17:00 LT | Yes | Yes 24/7 | Yes 5 working days per week 08:30 - 24:00 LT | n/a | n/a |  |
| DF en-route | Nijkerk, Assendelft, Texel, Veldhoven, Maasland | Yes | Yes 24/7 | Yes 5 working days per week 08:30-24:00 LT | n/a | n/a |  |

- The operational  local time allowed for flight inspection in 2025.

- First line remote technical support for ATC-NL. Basic technical execution, such as switching equipment (done by Integrated Service and Control Dept. (ISC) at ATC-NL Amsterdam).

- Second line technical support.  Advanced technical execution.

- Technical support at airside, such as placing DGNSS reference station.

- General airport support, such as picking up personnel from the platform and customs facilities.

- The designated frequency in The Netherlands for communication between the flight inspector and technical ground staff  is 122.755 MHz.