Concept version Measurement Parameters contact wire thickness

Annex 2.0 Parameter Specification

Concept

Table of contents

1	Introduction	2

1.1	General	3

1.2	Structure of the Annex	3

2	References and Definitions	3

3	General requirements	4

3.1	Localization of measurment data	5

3.2	Requirements for the Measurement vehicle	5

3.3	Requirements for the Measurement system	5

3.4	General Requirements of the Measurements	5

3.5	Technical Requirements for the Measurements	5

4	General parameters	5

4.1	Introduction	6

4.2	General parameters to be delivered	6

4.3	Technical Parameters to be delivered	7

4.4	Measurement distance	8

4.5	Sampling distance	8

5	Demonstration	8

# Introduction

## General

This Annex describes the general technical specifications relating to the Measurement Vehicle and the technical requirements. These specifications apply to all measurements unless stated otherwise.

## Structure of the Annex

The structure of this Annex is as follows:

Chapter 1 provides an introduction to this Annex.

Chapter 2 lists the references and definitions referred to in this Annex.

Chapter 3 lists the general requirements. These apply to all Measurement Vehicles and measurements unless stated otherwise.

Chapter 4 describes the specifications for each Parameter or image that must be generated..

# References and Definitions

The definitions listed below and their explanations apply to all annexes and associated appendices to the annexes, and to this document.

Definitions start with a capital letter.

| Dataset | A collection of measurement data and other data (also referred to as Parameters) related to the measurement data. In the Annexes, these are also referred to as ‘files’. |
| --- | --- |
| False negative | A measurement outcome that is incorrectly negative, meaning the result indicates that the situation is acceptable, while in reality it is not. |
| False positive | An incorrect alert or outcome where a measurement indicates that something is present (such as a thin spot), while this is not actually the case. Essentially a ‘false alarm’. |
| Field test | A field test of a measurement system is a practical test carried out in a realistic environment (outside the laboratory) to verify whether the measurement system functions properly under real conditions. Such tests are crucial to determine whether the system is reliable, accurate, and robust enough for practical use. |
| Measurement distance | The distance within the measurement system (internal) over which measurements are taken. This is a distance along the longitudinal direction of the track. |
| Measurement run | The execution of measurements while driving with a measurement train from the planned start point where measurements begin to the end point where measurements stop. |
| Measurement run number | A unique number identifying a Measurement Run. A number may not be reused for a subsequent Measurement Run. |
| Measurement uncertainty | Measurement uncertainty relates to the measurement error of an individual measurement. It means that the measurement error falls within a certain interval with a given probability. Both accuracy and precision are part of measurement uncertainty and are specified separately where necessary. |
| Measurement vehicle | A vehicle, or a series of coupled vehicles, equipped with measurement systems, that moves over railway tracks and complies with the requirements of the network statement. |
| Parameter | A parameter is a measurable (or derivable) property describing the condition of a system. Parameters are used to define measurement specifications and compare them with desired values to assess whether a measurement is correct and reliable. |
| Reliability level | The degree to which a measurement result corresponds to the actual situation. A higher reliability level indicates a greater likelihood that the results apply to the entire population. |
| Sampling distance | The distance between the measurement points as delivered to ProRail. |
| Thin spot | Plekken in de rijdraad met een dikte van < 7.5 mm of < 8.5 mm onder een lasklem. |

# General requirements

## Localization of measurment data

### GNSS-coordinates may deviate by a maximum of 1.0 m.

### The track name must have a reliability of  ≥ 99.9%.

## Requirements for the Measurement vehicle

### No requirements are imposed on the Measurement vehicle other than the usual admission requirements.

### The measurement speed shall be between 20 and 40 km/h.

## Requirements for the Measurement system

    - The accuracy of the measurement result shall not be affected by carbon deposits.

    - The accuracy of the measurement result shall not be affected by a clamp or other artefact on or near the contact wire.

## General Requirements of the Measurements

    - The data retention period is at least 5 years.

    - Measurement data must be available to ProRail within 14 days after the measurement.

    - The contact wire must be inspected for Thin spots, defined as locations with a thickness of < 7.5 mm or < 8.5 mm underneath a catenary clamp.

    - An image must be delivered for each Thin spot. The image provides further substantiation of the Thin spot.

## Technical Requirements for the Measurements

### The maximum Measurement uncertainty of the contact wire measurement is ±0.1 mm.

### The Measurement system must be able to measure up to 6 contact wires (3 contact wire pairs) simultaneously. The measurement signals of a specific contact wire within the measured section shall not change measurement channel or column name in the Dataset.

### The measurement method must be capable of detecting local contact wire wear with the following Reliability Levels:

### Maximum 1% False positives

### Maximum 5% False negatives

### The maximum 5% of measurement data that falls outside the Reliability level is randomly distributed over the infrastructure. In other words, an artefact on or near the contact wire shall not result in such an increase in Measurement Uncertainty at that location that the specified requirements are no longer met.

# General parameters

## Introduction

This chapter describes the Parameters that must be delivered in all Datasets. Section 4.2 provides an overview of the general Parameters to be delivered. Section 4.3 provides an overview of the technical Parameters to be delivered.

## General parameters to be delivered

The following general Parameters must be delivered in accordance with the specified requirements.

    - Measurement location

| Column names in Dataset | GPS_N GPS_E |
| --- | --- |
| Description | The ‘raw’ measured GNSS location. This signal shall not be corrected based on theoretical track alignment or previous data, but interpolation is permitted. North Coordinate: WGS84 DD format East Coordinate: WGS84 DD format Where GNSS signals are unavailable due to buildings or tunnels, NaN may be delivered. |
| Applicability | No exceptions |
| Measurment range | North: ≥ 50.7 en ≤ 53.5 Oost: ≥ 3.3 en ≤ 7.3 |
| Resolution | ≤ 0.000001 degrees |
| Measurement uncertainty | Absolute Measurement uncertainty: 1.0 meter with a reliability interval of 99%. |

    - Measurement day

| Column names in Dataset | Datum |
| --- | --- |
| Description | Date of the Measurement run, delivered in format [YYYYMMDD]. |
| Applicability | Always |
| Measurment range | N/A |
| Resolution | N/A |
| Measurement uncertainty | N/A |

    - Measurement run number

| Column names in Dataset | Meetritnr |
| --- | --- |
| Description | Identification of the Measurement run chosen by the Contractor, consisting of a maximum of 12 characters. The identification consists exclusively of letters and/or numbers and serves as a link between different files. The number is unique per supplier’s Measurement run. |
| Applicability | Always |
| Measurment range | N/A |
| Resolution | N/A |
| Measurement uncertainty | N/A |

    - Omschrijving indicent

| Column names in Dataset | Incident_omschrijving |
| --- | --- |
| Description | Description of the incident provided by the contractor, using a predefined format agreed in advance. |
| Applicability | Always |
| Measurment range | N/A |
| Resolution | N/A |
| Measurement uncertainty | N/A |

## Technical Parameters to be delivered

The following technical Parameters must be delivered in accordance with the specified requirements.

### Number of measured contact wires

| Column names in Dataset | Aantal rijdraden |
| --- | --- |
| Description | Number of measured contact wires. Up to 6 contact wires (3 pairs) can be measured. |
| Applicability | Always |
| Measurment range | 2 to 6 |
| Resolution | 1 |
| Measurement uncertainty | None |

### Thin spot

| Column names in Dataset | Dunne plek |
| --- | --- |
| Description | Locations in the contact wire with a thickness in accordance with requirement 3.5.3. |
| Applicability | Always |
| Measurment range | 0 – 8.5 mm |
| Resolution | 0.1 mm |
| Measurement uncertainty | TBD |

### Contact wire thickness wire 1

| Column names in Dataset | Rijdraaddikte draad 1 |
| --- | --- |
| Description | Thickness of the contact wire |
| Applicability | Always |
| Measurment range | 0 – 13.5 mm |
| Resolution | 0.1 mm |
| Measurement uncertainty | 0.1 mm |

### Contact wire thickness wire 2

| Column names in Dataset | Rijdraaddikte draad 2 |
| --- | --- |
| Description | Thickness of the contact wire |
| Applicability | Always |
| Measurment range | 0 – 13.5 mm |
| Resolution | 0.1 mm |
| Measurement uncertainty | 0.1 mm |

### Contact wire thickness wire 3

| Column names in Dataset | Rijdraaddikte draad 3 |
| --- | --- |
| Description | Thickness of the contact wire |
| Applicability | Always |
| Measurment range | 0 – 13.5 mm |
| Resolution | 0.1 mm |
| Measurement uncertainty | 0.1 mm |

### Contact wire thickness wire 4

| Column names in Dataset | Rijdraaddikte draad 4 |
| --- | --- |
| Description | Thickness of the contact wire |
| Applicability | Always |
| Measurment range | 0 – 13.5 mm |
| Resolution | 0.1 mm |
| Measurement uncertainty | 0.1 mm |

### Contact wire thickness wire 5

| Column names in Dataset | Rijdraaddikte draad 5 |
| --- | --- |
| Description | Thickness of the contact wire |
| Applicability | Always |
| Measurment range | 0 – 13.5 mm |
| Resolution | 0.1 mm |
| Measurement uncertainty | 0.1 mm |

### Contact wire thickness wire 6

| Column names in Dataset | Rijdraaddikte draad 6 |
| --- | --- |
| Description | Thickness of the contact wire |
| Applicability | Always |
| Measurment range | 0 – 13.5 mm |
| Resolution | 0.1 mm |
| Measurement uncertainty | 0.1 mm |

## Measurement distance

    - The Measurement distance shall be a maximum of 5 mm.

## Sampling distance

    - The Sampling distance shall be a maximum of 5 mm (minimum of 200 measurement points per meter).

# Demonstration

The Contractor shall prepare a demonstration file showing compliance with the specified requirements. The demonstration file shall include at least:

- The requirements applicable to the Parameter(s).

- A description of the measurement system, including:

  - Description of the systems and measurement method.

  - Description of the calibration method.

- The method of demonstrating compliance through measurements, including:

  - Description of the Field Tests.

  - Frequency of the Field Tests.

  - Acceptance/rejection criteria for the Field tests.

- The Contractor’s substantiation that the Parameter complies with the specified requirements.

Part of the demonstration file is an evaluation of a Field test in which at least 15 km of contact wire is measured. The contact wire to be measured is determined in mutual agreement between the contractor and ProRail. During this Field test, the contractor demonstrates through manual measurements that the measurements performed with the Measurement vehicle are valid and reliable. After execution of the Field tests, the demonstration file is supplemented with:

- Results of the most recently executed Field tests.

- Evaluation of the most recent measurements and the requirements imposed on the Field tests.