Bijlage Marktconsultatie High-speed stereoscopic 3-camera systeem

Wij vragen u de bijgevoegde lijst te beantwoorden.

Requirements for each camera

| Nr | Eisen | Akkoord bevestigen met “ja” | Eventuele Toelichting |
| --- | --- | --- | --- |
| 1 | Frame rate: Recording should be possible at least 12,500 frames/second at an image resolution of at least 1,000,000 pixels. |  |  |
| 2 | Frame rate: Recording should be possible at least 19,000 frames/second at an image resolution of at least 580,000 pixels. |  |  |
| 3 | Frame rate: Recording should be possible at least 37,500 frames/second at an image resolution of at least 260,000 pixels. |  |  |
| 4 | Image sensor type: The image sensor should be monochrome. |  |  |
| 5 | Bit depth: The image sensor should have a bit depth of at least 12 bit. |  |  |
| 6 | Shutter type: The camera shutter type should be a global electronic shutter. |  |  |
| 7 | Shutter exposure time: exposure time should be able to range from at least 500 ns to 1/frame rate. |  |  |
| 8 | The camera should have a mechanical shutter (required for sensor intensity calibration) |  |  |
| 9 | An option for the image output format should be ‘.tif’ . |  |  |
| 10 | ISO 12232 SAT sensitivity: D (daylight) ISO value of at least 25,000 is required |  |  |
| 11 | IR sensitivity: the camera should be infra-red sensitive, such that QE > 10% for light at a wavelength of 900 nm. |  |  |
| 12 | Each camera should have a circular buffer mode (continuous recording) |  |  |
| 13 | Internal image memory for each camera should be at least 16 GB |  |  |
| 14 | Lens mounts: both a F-mount (compatible with G-type lenses) and a C-mount are required |  |  |
| 15 | Camera synchronization: phase-locked synchronization between cameras is required |  |  |
| 16 | Camera synchronization: the camera system should be able to be synchronized phase-locked using an external TTL signal (5V). |  |  |
| 17 | Camera control: remote control from a computer should be possible. |  |  |
| 18 | Data download: no external frame grabber should be required. |  |  |
| 19 | Data download: data download speed should be at gigabit LAN speed or faster. |  |  |
| 20 | Camera mounting: both a 1/4 UNC mount and a 3/8 UNC mount are required |  |  |
| 21 | Video output: a HD-SDI output is required. |  |  |
| 22 | Operating temperature should be able to range between 0 to 40 degrees Celsius. |  |  |
| 23 | The cameras should be able to run for at least 8 hours continuously. |  |  |
| 24 | The camera system should have an "automatic recording mode": after receiving a trigger, each camera should automatically download the recorded data to a PC, after which each camera re-arms itself automatically. |  |  |
| 25 | Het bevestigen van de gestelde eisen met ja betekent dat het bewezen kan worden. Op verzoek kan Wageningen UR dit, door het (laten) uitvoeren van testen, verifiëren. |  |  |

Wij vragen u onderstaande vragen te beantwoorden.

Vragen High-speed stereoscopic 3-camera system

| Nr | Vraag | Antwoord |
| --- | --- | --- |
| 1 | Kunt u aangeven op welke punten uw apparatuur zich onderscheidt van de apparatuur van uw concurrenten? |  |
| 2 | Kunt u aangeven op welke punten de bijgeleverde besturingssoftware zich onderscheidt van de besturingssoftware van uw concurrenten? |  |
| 3 | Hoe goed is de lichtgevoeligheid van de sensor? |  |
| 4 | Lichtgevoeligheid van de sensor in het infrarood spectrum is van primair belang. Hoe goed scoort uw product hierin? |  |
| 5 | Hoe goed is de scherptediepte van uw camera in vergelijking met dat van de producten uw concurrenten? |  |
| 6 | Hoe flexibel is de camera in het instellen van de ‘region of interest’ op de sensor? |  |
| 7 | Is het mogelijk om de camera aan verschillende zijden te monteren? |  |
| 8 | Aan welke eisen moet de ruimte voldoen waar het camera systeem wordt geïnstalleerd? Welke omgevingstemperatuur en luchtvochtigheid is vereist? |  |
| 9 | Wat is de gemiddelde levensduur van de camera? |  |
| 10 | Wat is de prijs van de set van drie camera’s, inclusief oplevering? |  |
| 11 | Bent u geïnteresseerd in wetenschappelijke samenwerking? Zo ja op welke manier? |  |
| 12 | Bent u geïnteresseerd in het ontvangen van demo/promotiemateriaal naar aanleiding van ons onderzoek? |  |
| 13 | Geef de gegarandeerde levertijd van het camera systeem |  |

Algemene bedrijfsgegevens

We vragen u hieronder de bedrijfsgegevens in te vullen

.

Naam onderneming		:

Vestigingsadres			:

Postcode en plaats		:

Land				:

Postadres			:

Postcode en plaats		:

Land				:

Rechtsvorm van de onderneming	:

Contactpersoon				:

Functie contactpersoon			: 
Telefoon contactpersoon			:

E-mail contactpersoon			: